3GPP TSG-SA WG2#170	S2-2506918

Goteborg, Sweden, 25-29 August 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	Support for IMS DC in early media session

Document for:	Discussion/Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / Rel-20

Abstract of the contribution: Solution to address the support for IMS data channel application in early media session.

# 1.	Discussion

Proposes changes in the solutions to support IMS data channel application in early media session.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-56:

# >>>>BEGINNING OF CHANGES<<<<

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |
| X | X |  |  |

# >>>>NEXT CHANGE (new text)<<<<

## 6.X	Solution #X: Support for IMS data channel application in early media session

### 6.X.1	High level principles

This solution has the following high level principles, in order to support IMS data channel application in early media session:

- 	Data Channel Application in Alerting Phase (DCAP) service can be configured by the user and stored in the HSS as part of the subscription data.

-	IMS AS triggers the application data channel in alerting phase based on UE subscription data.

-	IMS AS performs as a B2BUA to update the SDP for data channel in early session and media flow for the main session.

-	The application data channel may coexistent with other early session services, e.g., CAT, CRS.

### 6.X.2	Description

This is a solution for Key issue #1 " DC establishment in alerting phase for early media".

The user of UE#2 is able to subscribe to the Data Channel Application in Alerting Phase (DCAP) service, including activate (or de-activate) the service, and update the settings, e.g., to change by configuration the active data channel application. The DCAP subscriber is able to refine the data channel application selection behaviour with configured rules, e.g. time, calling party's location, called party's location, the identity of the calling and called party. The DCAP service is able to select the appropriate data channel application according to the rules.

DCAP is a terminating network service, but can also have an originating network functional component. That is, DCAP  can be selected on behalf of the called subscriber for presentation to the calling party, but the calling (IMS) subscriber can also subscribe to and activate the DCAP service.

### 6.X.3 Procedures

The overall procedures of data channel establishment in early media phase are illustrated in figure 6.X.3-1.

Figure 6.X.3-1: DC establishment in early media phase

1.	UE#1 sends the SIP INVITE request with an initial SDP to the IMS AS in the terminating network. The initial SDP contains offers for audio/video flow, bootstrap data channel. UE#1 indicates support for preconditions, reliable provisional responses, and early-session.

2.	The IMS AS sends Nimsas_SessionEventControl_Notify to the selected DCSF to notify the session establishment.

3.	If the remote bootstrap data channel (BDC from UE#1 to the DCSF in the terminating network) is not requested by UE#1, the DCSF instructs the IMS AS to add the bootstrap data channel.

4.	The DCSF invokes the Nimsas_MediaControl_MediaInstruction instructing the IMS AS how to set up bootstrap data channel with MF both for originating and terminating side.

5.	The IMS AS instruct the MF to allocate required bootstrap data channel media resources.

6.	The IMS AS responds to the MediaInstruction request received in step 4.

7.	The DCSF responds to the Notify Request received in step 2.

8.	The IMS AS sends the INVITE to UE#2.

9.	UE#2 determines the subset of the media flows and responds with an Offer Response message back to the IMS AS.

10. The IMS AS forwards the Offer Response message (e.g., 183 Session Progress) to UE#1. The IMS AS may add a remote bootstrap data channel with UE#1 if not request in the INVITE message.

11-12.	 UE#1 reserves resource and confirms receipt of the Offer Response and sends the PRACK, which acknowledges the 183 (Session Progress) provisional response, towards UE#2.

13-14.	UE#2 reserves resource and sends a SIP 200 (OK) response for the SIP PRACK request to UE#1.

15.	UE alerts the user and sends Ringing indication to the IMS AS.

16.	Base on UE#2’s subscription data, the IMS AS decides to trigger the data channel application to UE#1.

17. The IMS AS notifies the DCSF with the request of data channel application (e.g., application ID) in alerting phase.

18-19. The DCSF determines the media parameters for the application data channel and responses to the IMS AS.

20. The IMS AS instructs the MF to reserve the media resource and MDC2 end points to the DC AS.

21.	The IMS AS response the DCSF the MediaInstruction with MDC2 end points information.

22.	The DCSF indicate the DC AS reserving the media resource for the application data channel and MDC2 end points to the MF.

23-24. The DCSF response the notification in step 17 and instructs the MF to update the media resource.

25.	The IMS AS triggers the IMS session update to UE#1 containing SDP for the application data channel.

26.	If the data channel application is not locally available, UE#1 downloads the application form the DCSF via the established bootstrap data channel.

27.	 UE#1 confirms the update for the application data channel.

28.	UE#1 starts data channel application with the DC AS in early session.

29. UE#2 accepts the session and sends 200 OK to the INVITE.

30-31.	The IMS AS updates UE#1 with SDP offer to initiate the media flow of the main session and terminate the application data channel in the early session.

32.	The IMS AS instructs the MF to update the media resource and terminate the application data channel.

33-34. The IMS AS forwards 200 OK to UE#1. UE#1 sends ACK to UE#2.

35. The media flow of the main session between UE#1 and UE#2 starts.

### 6.X.4	Impacts to Services, Entities and Interfaces

UE

-	Indicates supporting of early media session in SIP INVITE message, as specified in existing specification, e.g., TS 24.229.

-	Supports application data channel in the early session.

HSS

- Supports management of  user subscription for Data Channel Application in Alerting Phase (DCAP) service.

- Supports the user configuration of data channel application(s) in alerting phase.

IMS AS:

-	Triggers the application data channel in alerting phase.

-	Terminates the application data channel when the call accepted by the terminating UE.

-	Performs as B2BUA to update the media flow in the SDP exchanged with UE.

# >>>>END OF CHANGES<<<<