SA WG2 Meeting #170	S2-2506926

25 – 29 Aug, 2025, Goteborg, SE

Source:	China Telecom

Title:	[WT#1.3, Non-3GPP Access and multi-access] Support for non-3GPP access in 6G

Document for:	Agreement

Agenda Item:	20.6.1.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the consideration on the study of non-3GPP access which is under WT#1.3 in SP-250806.

1. Justifications

SP-250806 indicates WT#1.3. Study whether and how to support and/or enhance different non-3GPP access (e.g. Wi-Fi, wireline) in 6G and support multi-access data connections between 3GPP access and non-3GPP access.

Regarding the WT mentioned above, this paper considers the following aspects:

Architectural Motivation

The 5G Core Network (5GC), architected by 3GPP, fundamentally embraces the principle of access independence. This core design decouples the network functions providing services from the underlying access technologies used to connect User Equipment (UE). While native 3GPP access (5G NR, LTE) is paramount, standardized support for Non-3GPP Access Networks is a critical enabler for true network convergence, seamless service delivery, and leveraging diverse infrastructure. The non-3GPP access can act as the supplement of the 3GPP access network in some scenarios. By unifying control and service delivery across cellular, Wi-Fi, and fixed broadband, operators can deliver pervasive, high-performance, feature-rich 5G experiences, maximize infrastructure utilization, and pave the way for true fixed-mobile convergence. Therefore, in the study of 6G, non-3GPP can be a key feature that needs to be studied, including the support of non-3GPP access and the support of multi-access data connections between 3GPP access and non-3GPP access.

Gap Analysis

In the standardization of 5G, it differentiated several different kinds of non-3GPP access scenarios, including the trusted non-3GPP access, untrusted non-3GPP access and the wireline access. These scenarios provide operators with the flexibility to securely integrate virtually any access technology into their unified 5G service framework, enabling seamless experiences, convergence, and new business models.

ATSSS feature, supporting that the UE accesses the 5G network via 3GPP access and non-3GPP access simultaneously, was also supported in 5G. ATSSS is a cornerstone 5G Core (5GC) feature enabling operators to dynamically manage user traffic across both active 3GPP (e.g., 5G NR) and Non-3GPP (e.g., Wi-Fi, fixed broadband) access paths – all within a single Protocol Data Unit (PDU) Session. Unlike basic offload or access switching, ATSSS operates with per-flow granularity, allowing distinct steering policies for different applications based on performance needs, operator rules, and real-time network conditions.

However, the commercial deployment of non-3GPP access network in 5G faces the limitation on the support of the hardware of terminals and the network. The lesson from the previous work should be taken into consideration when the discussion on the non-3GPP access is ongoing. The ePDG, which was deployed successfully in operator’s network because of the commercial application of voWiFi, can be seen as a potential study path in 6G’s work. And enhancements on previous work or a brand new mechanism can also be alternatives. What is important is that solutions that are easier to deploy deserve greater consideration. In addition, based on the support of the non-3GPP access network in 6G, the traffic steering, switching and splitting between 3GPP access and non-3GPP access should also be supported.

Technical Impact

Considering a mechanism for non-3GPP access is needed in 6G, there may be the following technical impact:

-	The transport protocol and call flow of the non-3GPP access network in 6G

-	The handover between the 3GPP access and the non-3GPP access, e.g. the 3GPP access in 5G and the non-3GPP access in 6G, the 3GPP access in 6G and the non-3GPP access in 5G.

-	Policies to support the traffic steering, switching and splitting.

**** First Change ****

Annex A.X. WT#1.3 Scope

To study whether and how to support and/or enhance different non-3GPP access (e.g. Wi-Fi, wireline) in 6G and support multi-access data connections between 3GPP access and non-3GPP access.

-	Study whether and how to support and/or enhance untrusted non-3GPP access in 6G.

-	Study whether and how to support and/or enhance trusted non-3GPP access in 6G.

-	Study whether and how to support and/or enhance wireline access in 6G.

-	Study whether and how to support and/or enhance multi-access data connections between 3GPP access and non-3GPP access.

**** Second Change ****

5.X. Key Issue #X: Support of non-3GPP access and multi-access

This key issue will address the following issues:

- For supporting the non-3GPP access to the 6G Core network:

-	Identify what transport protocol and call flow need to be defined to support non-3GPP access, e.g. untrusted non-3GPP access, trusted non-3GPP access and wireline access.

-	Identify how to support the registration and the establishment of PDU Session via the non-3GPP access.

-	Identify how to support handover of a PDU Session between non-3GPP access and 3GPP access in non-roaming and roaming case.

- For supporting multi-access data connections between 3GPP access and non-3GPP access:

-	Identify how the 6G Core network and the UE can support access traffic steering between 3GPP and non-3GPP accesses.

-	Identify how the 6G Core network and the UE can support access traffic switching between 3GPP and non-3GPP accesses.

-	Identify how the 6G Core network and the UE can support access traffic splitting between 3GPP and non-3GPP accesses.

**** End of Changes ****