SA WG2 Meeting #170	S2-2506928

25 – 29 Aug, 2025, Goteborg, SE

Source:	China Telecom

Title:	[WT#2, Migration and Interworking] Support for migration and interworking

Document for:	Agreement

Agenda Item:	20.6.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the consideration on the study of migration and interworking which is under WT#2 in SP-250806.

1. Justifications

Regarding the WT#2 in SP-250806, this paper considers the following aspects

Architectural Motivation: When it comes to the study of the new generation’s network, the discussion on the architecture, the migration of the network and the seamless interworking mechanisms with the legacy network is inevitable. The lesson from 5G and the value of the innovation of the network should be fully considered.

Gap Analysis: In the first release of 5G, there were several architecture options specified, including Stand Alone (SA) and Non-Stand Alone (NSA) deployment options. NSA served as a crucial pragmatic stepping stone, enabling operators to capitalize on early 5G marketing opportunities, deliver significantly faster speeds via eMBB, leverage existing investments, and gain operational experience with NR radios while the 5GC ecosystem matured. Its limitations, however, are fundamental and prevent the realization of 5G's most transformative capabilities. SA is the definitive end-state architecture. While requiring greater upfront investment and facing initial coverage and maturity hurdles, SA offers superior long-term performance, efficiency, and service agility. Operators who prioritize future-proofing, differentiation through enterprise/industrial services, and harnessing the full power of 5G innovation will inevitably transition to SA as the cornerstone of their network strategy. However, the transition of the deployment from NSA architecture to SA architecture involved managing complexity, and also resulted in significant waste of human and material resources. Therefore, it’s proposed to use SA architecture as the basis in day one.

In the evolution of the new generation network, it’s necessary to consider the interworking with the legacy network, i.e. the 5G network. In 5G, a comprehensive suite of standardized interfaces, protocols, and procedures are meticulously defined for interworking to bridge the gap between 4G and 5G, and between different 5G deployment flavours. Dual Connectivity (EN-DC) is the bedrock of NSA, tightly coupling 4G and 5G radios under the 4G core's control. For SA deployments, the N26 interface is pivotal for optimized mobility and service continuity with the existing 4G EPC, while EPS Fallback/VoNR Fallback ensures reliable voice services during the transition. These mechanisms collectively enable operators to deploy 5G incrementally, leverage existing infrastructure, protect user experience during mobility, and ultimately migrate towards the full potential of Standalone 5G.

On the assumption that the SA architecture is determined as the basis in day one, solutions standardized in 5G can be used for reference, e.g. with/without N26-like solutions.

The N26 interface is a key enabler for smooth and efficient interworking mobility between 5G SA and 4G EPC networks. By allowing direct context transfer between the AMF and MME, it minimizes handover delays, preserves IP session continuity, and provides a superior user experience during network transitions. While its deployment is optional and involves considerations around complexity and security, N26 remains the most standardized solution for achieving truly seamless service continuity across 5G and 4G domains, particularly crucial in the early phases of 5G SA rollout where coverage might be patchy. Its presence significantly enhances the practical usability of 5G SA for subscribers on the move. Under this approach, a new interface used for interworking needs to be defined between 5GC network and 6GC network.

For the solution without N26 interface, it leverages HSS/UDM-mediated context transfer, unified user-plane nodes (SMF+PGW-C), and UE registration modes to sustain IP continuity. While feasible, it trades slight handover latency for reduced core-network complexity.

6GC

6G RAN

5GC

5G RAN

In addition, dual stack, which support the UE accesses the 6G network via two different 3GPP access, i.e. gNB and 6G RAN, can also be an optional choice for interworking. In this kind of solution, there is no interface between gNB and 6G RAN, and the UE needs to support the capability of dual registration in both 5GC and 6GC.Under this mechanism, based on the service requirements and the status of the two access networks, service data flows can be routed through the optimal or lowest-latency network access path via traffic offloading and migration. To achieve data transmission, switch or split between 5G and 6G, user service continuity and quality can be ensured.

6GC

6G RAN

5GC

5G RAN

Technical Impact: This work task considers the migration and the interworking of the network. Based on the analysis above, the solutions rely on the design of the overall architecture. The definition of the new interface may be needed, and interactions between network functions need to be considered.

**** First Change ****

Annex A.X. WT#2 Scope

SP-250806 indicates WT#2: Study migration and interworking, including

-	How to support migration to 6GS.

-	How to support interworking with 5GS.

-	Whether and how to support interworking with EPS

-	Whether and how to support interworking with EPS

NOTE 1:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 2:  The detailed migration study scope will be coordinated and aligned with RAN.

Regarding the scope of this WT, the following aspect needs to be considered:

-	Identify what interfaces and procedures need to be defined to support the migration from 5GC to 6GC.

-	Identify what level of service continuity, possibly specific per service, shall be maintained for UEs changing between 6GC and legacy 3GPP core network, e.g. the 5GC, EPC, 4G/5G NTN/satellite access that use EPS/5GS.

-	Identify what interfaces and procedures need to be defined to support the interworking with the legacy network to ensure the service continuity on non-roaming and roaming case.

**** Second Change ****

5.X. Key Issue #X: Migration and Interworking

This key issue will address the following issues:

- For the migration to 6GS

-	Identify the potential migration path from 5GS to 6GS.

-	Identify what interfaces and procedures need to be defined to support the migration from 5GC to 6GC.

- For supporting interworking with 5GS

-	Identify what level of service continuity, possibly specific per service, shall be maintained for UEs changing between 6GC and 5GC.

-	Identify what interfaces and procedures need to be defined to support the interworking with the 5G network to ensure the service continuity on non-roaming and roaming case.

- For supporting interworking with EPS

-	Identify what level of service continuity, possibly specific per service, shall be maintained for UEs changing between 6GS and EPS.

-	Identify what interfaces and procedures need to be defined to support the interworking with the 4G network to ensure the service continuity if supported.

- For supporting interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS

-	Identify what level of service continuity, possibly specific per service, shall be maintained for UEs changing between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

-	Identify what interfaces and procedures need to be defined to support the interworking with 4G/5G NTN/satellite access that use EPS/5GS to ensure the service continuity.

**** End of Changes ****