SA WG2 Meeting #170	S2-2506930

25 – 29 Aug, 2025, Goteborg , SE

Source:	Ericsson

Title:	[WT#1.2, UP] User plane architecture

Document for:	Agreement

Agenda Item:	20.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes Work Tasks (WTs) related to User Plane Architecture.

# 1. Justifications

Architectural Motivation

The 5G User plane architecture and features have evolved during several 5G releases to provide connectivity for a wide variety of 5G use cases. The user plane, integrated with the PCC framework, supports UE connectivity-based services according to the subscription and applicable policies. The 5G standard capabilities include the support of IPv4, IPv6, IPv4v6, Ethernet and Unstructured PDU sessions, flexible N6 termination placement e.g. at edge, support of variety of deployments (like specific SMF service areas) and support of roaming scenarios (with home routed traffic, traffic break-out…). It supports multiple accesses (3GPP and non-3GPP), multi-access PDU Sessions, ultra reliable PDU session with redundant transmission, integration with TSN, interworking with EPC and a long list of characteristics and capabilities. The 5G user plane is decoupled from the transport network and supports any undelaying transport technology.

The proposal is to assume the 5G User Plane architecture as base for 6G. This way, from day-1 6G can make use of the user plane capabilities specified for 5G. In addition, the interworking, mobility, migration and NF sharing between 5GS and 6GS will be much simpler, and successful aspects of 5G UP, like N3/N9 reference points widely deployed as multi-vendor and secure roaming interfaces can be kept.

With a good base in place, the efforts can go to the impacts of adding the 6G radio access type and to any additional value-add capabilities needed by 6G UE connectivity-based services. There are some aspects that may require special attention, and the 6G study should also help to identify any enhancements needed to address them:

- Traffic and sessions will continue their growth trend over all available accesses, and for user plane, performance, and cost and energy efficient scaling will remain a priority.

- The evolution of the user plane for the 6G timeframe needs to consider the internet traffic evolution trends, like 1) increasing encryption (content, header and control information, such as transport protocol headers, domain names etc. 2) hiding the traffic destination by increasing usage of proxies or VPNs, and cloud front ends (e.g. LBs). 3) high momentum of transport protocols. As a result, how applications use the network requires more and more application-network collaboration.

In 5GC user plane architecture, SMF selects the PDU session UPF(s), which among other, perform UE IP address anchoring and enforcement of the session policies (e.g. gating, QoS control, packet marking, packet buffering, traffic steering, usage reporting, LI, event detection, QoS monitoring…), SMF manages the N3/N9 GTP-U tunnels  that conform the session data path with consideration for session continuity and SMF provisions rules in the data path entities according to the PDU Session needs, in particular, the UPF(s) over N4.

Some other topics have been mentioned:

- The N4 for packet processing control has remained non-SBI in the 5GC architecture. However, this alone does not justify to re-define this interface as SBI:

  - The exposure of UPF events has already been defined as SBI and several NFs make use of this service for different procedures. SMF would be the only consumer of a UP packet processing service.

  - SBI instead of PFCP would drive cost and complexity both in UPF and SMF since two separate protocols for the same purpose would need to co-exist for a long time.

  - PFCP on N4 has been enhanced when needed to satisfy the needs of the 5G use cases. It can also be enhanced for new 6G cases.

Based on the above, being non-SBI is not enough motivation for doing N4 differently for 6G and no sub-WT is proposed for this. In principle 6G can continue to use PFCP for packet processing control and SBI for exposure.

- In 5GC user plane, Flow and Application identification strongly relies on the 5-tuple, which considering internet trends, is becoming legacy and insufficient to address the different requirements of the application multiple sub-flows. Explicit application-network collaboration will have to be the core principle for other than best-effort treatment.

**** First Change ****

# Annex A.X. WT#1.2 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT #1.2 The 6G study assumes the 5G User Plane Architecture and studies enhancements on the user plane architecture in relation to the following topics:

- What enhancements are needed to support the new 6G access on the user plane and new UE-connectivity based 6G use cases, including interworking with 5G (and potentially 4G), different mobility events with consideration of session continuity, non-roaming and roaming scenarios and variety of deployments (e.g. specific network service areas, like SMF service areas in 5G)

- Whether enhancements can be identified in the control of the data paths and the user plane packet processing (e.g. dynamic activation/deactivation of capabilities) which further improve UP performance and UP scaling and energy efficiency.

- Whether and how the user plane can be enhanced to provide better application-network collaboration (e.g. with in-band mechanisms) for application traffic.

**** Second Change (all new) ****

# 5.1.2.x Key Issues #1.2.x User Plane Architecture in 6G System

The KI assumes the 5G User Plane as base for the study.

The following KIs are proposed:

#1. What enhancements are needed to support the new 6G access on the user plane and new UE-connectivity based 6G use cases, including interworking with 5G (and potentially 4G), different mobility events with consideration of session continuity, non-roaming and roaming scenarios and variety of deployments (e.g. specific network service areas, like SMF service areas in 5G).

#2. Whether enhancements can be identified in the control of the data paths and the user plane packet processing (e.g. dynamic activation/deactivation of capabilities) which further improve UP performance and UP scaling and energy efficiency.

#3. Whether and how the user plane can be enhanced to provide better application-network collaboration (e.g. with in-band mechanisms) for application traffic.

**** End of Changes ****