3GPP TSG-SA2 Meeting #170	S2-2506931

# Gothenburg, Sweden, 2025-08-25 – 2025-08-29

Source:	Ericsson

Title:	[WT#1.2, SBA framework] SBA for 6G

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution:  Proposed way forward for SBA framework enhancements

# 1	Discussion

Work Task 1.2 states:

“Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, …”

Whether:

In Ericsson’s view, there is no drive to go away from SBA in 6G. One could ask what an alternative would be? Having NFs offering (producing) a set (one or more) services that NF consumers can use has not put any hinders when spefifying 5G, on the contrary, it has been a powerful way to specify the system. As stated also in the SID:

“The 5G network architecture marked a significant leap forward compared with previous generations, with its adoption of a Service-Based Architecture (SBA) enabling a cloud-native deployment, and 5G promoted business opportunities to providing services to verticals. These innovations enhanced flexibility and scalability, enabling more dynamic and adaptable network orchestration.”

Thus, any alternative to SBA needs to be justified and proved better.

How:

We don’t see any other alternative than keeping service speficiations, NF profiles, NRF, etc.

Enhance:

The major enhancement to SBA would on the protocol level, e.g. adding QUIC. Other aspects such as overload control etc, could also be of importance, however, these improvements are not of SA2 concern, and should be handled by CT groups.

# 2	Proposal

For 6G study in SA2, the SBA framework as specified for 5G should be re-used. There is no need to study any enhancements within SA2. SBA related enahancements are proposed to be done in by CT.

# 3	pCR

****************** start change ****************

## A.1.2 	Work Task 1.2

Study whether and how to support and/or enhance the following aspects in 6G: network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

NOTE x: Work/Study related to enhancements to SBA framework will be done by stage 3. For stage 2 in 6GS, SBA framework specified in TS 23.501 [ref], clause 7 is used, and thus no stage 2 work/study is required.

****************** end change ****************