SA WG2 Meeting #170	S2-2506932

25 – 29 Aug, 2025, Goteborg, SE

Source:	Ericsson

Title:	[WT#1.2] Network slicing

Document for:	Discussion/Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This template is intended for companies contributing to the SA2 6G SID to propose Work Tasks (WTs) and/or Key Issues (KIs), including the rationale, expected scope, and any potential structuring of the task.

# 1. Justifications

End-to-end slicing in legacy 5GC is a powerful feature but have had a hard time to take off, many times due to its complexity and UE (NAS) dependency, especially in conjunction with URSP. We need to investigate the future of slicing in an evolved 5GC and how it shall relate to/interact with the legacy slicing solution.

From operators’ point of view, additional flexibility, automation and efficiency are possibly needed in the 6G network slice design and architecture, to deal with the increased dynamics required by verticals and for the new use cases. For example, in the 5G system, a slice creation/termination requires various network function (NF) configurations and optional rules for route selections depending on the UE platforms, resulting in delay and customized efforts to bring up a slice. These issues for deployment and potential solutions for the flexibility refer to enhancements in RAN, Core and UE.

Fundamental questions like e.g.,

How much of the legacy 5GC slicing solution makes sense to keep?

How many of the legacy slicing features makes sense to discard?

How much of the remaining logic need to be (benefits by being) evolved?

Are there any simplifications in Core and in UE that can be done to increase the traction?

Architectural Motivation:

This key issue addresses several separate aspects to make this happen in 6G, for example

- How much, if any, changes can be achieved to support slices in 6G without changing the architecture?

- What will be the architectural impact to the Core network if the system shall support slices only served by a single RAT (e.g., Served for 6G only), how shall the allowed slices and corresponding PDU Sessions be treated in e.g., IRAT mobility to/from NR?

- Can/Shall we assume that the legacy logic to align/coordinate slices cross domains (i.e., Supported S-NSSAI per TA) will remain unchanged (i.e., the TA concept will remain and that the different RATs will have unique TAIs i.e., no TAI valid for both 5G and 6G)?

- Can/shall we assume that the S-NSSAI format will remain identical with legacy format but instead add new standardized values?

Gap Analysis:

- Which 3GPP slice solution baseline/features used for 5G shall be used and evolved to support 6G?

- Which simplifications can be done to increase the traction in Core and in UE?

- Is the URSP mechanism sufficient to manage the slice policy?

Technical Impact: How would this WT/KI influence:

- We need to secure that interworking with the 5G slice solution typically at IRAT mobility and/or at roaming continues to work.

**** First Change all new****

# Annex A.X. WT#1.1 Support of the Network Slicing

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WTs. This clause is part of the TR Annex.

The study of network slicing for 6G systems covers the following aspects

-	Investigate which 3GPP slice solution baseline/features used for 5G shall be used and evolved to support 6G.

-	Investigate what simplifications (e.g. simplification of the non-homogeneous slice support in the system) can be done to increase the traction in Core and in UE.

-	Investigate how to decrease dependency towards UE e.g., NAS upgrades, URSP to support an evolution, new slice related features.

-	Investigate impact on 5G functionality e.g., IRAT mobility and roaming, that any suggested simplification and/or evolution may create.

…

**** Second Change all new ****

# 5.X. Key Issue #X: Key Issue Title

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

This KI investigates support of network slicing for 6G system and covers the following aspects:

-	Investigate which 3GPP slice solution baseline/features used for 5G shall be used and evolved to support 6G.

-	Investigate what simplifications (e.g. simplification of the non-homogeneous slice support in the system) can be done to increase the traction in Core and in UE.

-	Investigate how to decrease dependency towards UE e.g., NAS upgrades, URSP to support an evolution, new slice related features.

-	Investigate impact on 5G functionality e.g., IRAT mobility and roaming, that any suggested simplification and/or evolution may create.

**** End of Changes ****

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency Editor’s Note: This column should highlight if WT#x is self-contained, or is depended on completion of other WTs |
| --- | --- | --- | --- |