SA WG2 Meeting #170	S2-2506933

25 – 29 Aug, 2025, Goteborg, SE

Source:	Ericsson

Title:	[WT#1.2, Exposure] Exposure Framework architecture in 6G

Document for:	Discussion/Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes aspects of the exposure framework to support and/or enhance in 6G architecture, including the rationale, expected scope, and any potential structuring of the work task.

# 1. Justifications

This paper proposes to maintain the core and principles of the existing 5G Network Exposure framework in 6G, but at the same time simplify and enhance the Network Exposure framework.

Below are some aspects of the exposure framework which require simplification, improvements and usability in order to meet the requirements for 6G system and co-exist with 4G/5G but focusing on an evolution step from 5G solely.

- Common Network Exposure framework that supports co-existence of 4G, 5G and 6G systems and selection of the 4G, 5G and/or 6G system domain (e.g. legacy nodes/system still kept for 4G IoT Devices), that is not fully and clearly specified in the 3GPP today and shall allow the AF/AS to invoke a Unified Network API towards CSP networks and hide underlying network topology. Possible improvements may be: (i) assist Network Exposure Layer (e.g. SCEF/NEF) for core network domain selection (EPS, 5GS, 6GS); (ii) how to locate the system that currently handles the subscription/request from AF/AS, e.g. avoid try-and-error throughout the different domains to eventually provide an unsuccessful result.

- Network Exposure procedures do not clearly cover roaming situations. We should aim to describe the Network Exposure use cases considering roaming, whether applicable. Similarly, for new 6G procedures it is key to include the roaming considerations. 
In addition, the procedures may not be applicable in roaming situations, or be subject to restrictions, in these cases, the traffic between home and visited networks should be avoided. This is not fully covered today in existing specifications.

- Simplify and avoid API fragmentation for network exposure capabilities moving forwards in 6G. 
Currently there are plenty of  NEF services defined in stage 2, even more APIs created in stage 3 like multiple NEF APIs corresponding to one generic External Parameter Provisioning service in 5GS, which will bring more challenges and increase implementation, test and maintenance cost for CSPs, vendors and third parties to adopt the network exposure service/APIs.

**** First Change ****

# Annex A.X. WT#1.2 [Exposure] Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

[WT#1.2, Exposure] The 6G study maintains the core and principles of existing 5G Network Exposure framework, and studies enhancements in relation to the following specific aspects:

- Study a common Network Exposure framework that supports co-existence and determine the selection of 4G, 5G and 6G system domain.

- Study whether and how to restrict Network Exposure procedures in roaming, aiming to avoid traffic from the home network to visited networks when not allowed or unnecessary.

- Study the simplification of network exposure NBI APIs capabilities in 6G, e.g. define clear principles for Network Exposure API Design and possibility merge non-self-contained APIs that provide dependent functionality or even remove/deprecate network exposure services that may no longer be required.

**** Second Change (all new) ****

# 5.1.2.x Key Issues #1.2.x Network Exposure Framework in 6G

This KI assumes the 5G Network Exposure Framework as the base for the study.

The following KIs are proposed:

- Study a common Network Exposure framework that supports co-existence and determine the selection of 4G, 5G and 6G system domain.

- Study whether and how to restrict Network Exposure procedures in roaming, aiming to avoid traffic from the home network to visited networks when not allowed or unnecessary.

- Study the simplification of network exposure NBI APIs capabilities in 6G, e.g. define clear principles for Network Exposure API Design and possibility merge non-self-contained APIs that provide dependent functionality or even remove/deprecate network exposure services that may no longer be required.

**** End of Changes ****