SA WG2 Meeting #170	S2-2506934

25 – 29 Aug, 2025, Goteborg , SE

Source:	Ericsson

Title:	[WT#1.2, Policy] Policy Framework architecture in 6G

Document for:	Discussion/Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes aspects of the policy framework to support and/or enhance in 6G architecture, including the rationale, expected scope, and any potential structuring of the work task.

# 1. Justifications

5G Policy and charging control framework, as specified in TS 23.503, is an evolution of the Policy and charging control architecture specified in TS 23.203 for EPS. In addition, the principles of 5G Policy and charging control framework specified in Release 15 have been maintained in later releases, whilst allowing the addition of new features and capabilities.

Hence, this contribution proposes to maintain the core and principles of the existing 5G Policy and charging control framework in 6G.

However, there are some aspects of the policy framework that need to be simplified or enhanced in order to meet the requirements for 6G system

- Remove the support of Rx diameter interface, and complete the migration to SBA N5/N30 interface of IMS-based services, MCPTT, Emergency services, Multimedia Priority Services, Mission Critical service and exposure

- Study how to evolve the External Parameter Provisioning Framework aspects of policy control in the context of 6G and fulfil in an efficient manner the use cases for exposure of policy control capabilities that make use of Application Data (e.g. AF guidance on URSP, AF requested QoS for future sessions, AF influence on AM policies, etc). The following shortcomings have been identified in the current framework defined in 5G:

  - Application Data contains multiple sub-sets with different data keys that follow different patterns according to the use case to be realized

  - NEF stores AF provided data directly in UDR, without ensuring the information is valid, applicable and authorized by the PCF

  - Different AFs may have stored conflicting information that PCF needs to resolve during traffic procedures

  - The PCF needs to manage multiple data sets for policy decisions, as for a given subscriber, input for policy decisions is scattered in different UDR resources.

**** First Change ****

# Annex A.X. WT#1.2 [Policy] Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

[WT#1.2, Policy] The 6G study maintains the core and principles of the existing 5G Policy and charging control framework, and studies enhancements in relation to the following specific aspects of the policy control framework:

- Study whether it is possible to deprecate the support of Rx Diameter interface, and complete the migration to SBA N5/N30 interface of IMS-based services, MCPTT, Emergency services, Multimedia Priority Services, Mission Critical service and exposure.

- Study whether and how to evolve the External Parameter Provisioning Framework aspects of policy control in the context of 6G and fulfil, in an efficient manner, the use cases for exposure of policy control capabilities that make use of Application Data (e.g. AF guidance on URSP, AF requested QoS for future sessions, AF influence on AM policies).

**** Second Change (all new) ****

# 5.1.2.x Key Issues #1.2.x Policy Control Framework in 6G

This KI assumes the 5G Policy and Charging Control Framework as the base for the study.

The following KIs are proposed:

- Study whether it is possible to deprecate the support of Rx diameter interface, and complete the migration to SBA N5/N30 interface of IMS-based services, MCPTT, Emergency services, Multimedia Priority Services, Mission Critical service and exposure.

- Study whether and how to evolve the External Parameter Provisioning Framework aspects of policy control in the context of 6G and fulfil, in an efficient manner, the use cases for exposure of policy control capabilities that make use of Application Data (e.g. AF guidance on URSP, AF requested QoS for future sessions, AF influence on AM policies).

**** End of Changes ****