SA WG2 Meeting #170	S2-2506936

25 – 29 August, 2025, Goteborg, SE

Source:	Ericsson

Title:	[WT#1.3] WT and KI on non-3GPP access in 6G

Document for:	Discussion/agreement

Agenda Item:	20.6.1.3

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This contribution proposes WT and KI for non-3GPP access, focusing on the following aspects:

- One WT focusing on non-3GPP access and another WT on multi-access

- Target one common solution for non-3GPP, for all device types and potentially all non-3GPP accesses

# 1. Justifications

#### Architectural Motivation and Gap Analysis:

Using non-3GPP access has become an important complement to 3GPP access for many operators.

In rel-15 to rel-19 multiple solutions to support non-3GPP have been defined, as shown in Table 1.

Table 1. Solutions for non-3GPP access

| # | Feature | First added in release |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Rel-15 | Rel-16 | Rel-17 | Rel-18 | Rel-19 |
| 1 | Untrusted non-3GPP access (N3IWF) | X |  |  |  |  |
| 2 | Trusted non-3GPP access (TNAP + TNGF) |  | X |  |  |  |
| 3 | Wireline access with 5G RG (5G-RG + W-AGF) |  | X |  |  |  |
| 4 | Wireline access with legacy RG (FN-RG + W-AGF) |  | X |  |  |  |
| 5 | Untrusted non-3GPP access IWK with EPS (ePDG) | X |  |  |  |  |
| 6 | Support for devices that do not support 5GC NAS over WLAN access (N5CW devices + TWAP + TWIF) |  | X |  |  |  |
| 7 | Support for Non-5G capable devices behind 5G-CRG and FN-CRG (N5GC devices + W-AGF) |  | X |  |  |  |
| 8 | Support for Authenticable Non-3GPP (AUN3) devices behind 5G-RG |  |  |  | X |  |

Out of these, only #5, the ePDG-based solution, has been deployed on a wide scale.

Observation 1: Despite a collection of non-3GPP access solutions in 5G, none of the 5G-native non-3GPP solutions have (yet) reached the market on a wide scale.

The 5G-specific solutions (i.e. not based on ePDG) cater for different non-3GPP accesses (untrusted, trusted and wireline) and device capabilities (device that support NAS via non-3GPP and devices that do not support NAS via non-3GPP). None of these solutions have been deployed on a wider scale. Furthemore, the solutions to support non-NAS capable devices (N5CW, N5GC, AUN3, FN-RG) rely on a NW entity supporting NAS on behalf of the device, which is a questionable work-around, where a UE-side of a protocol needs to be implemented in the network to compensate for lack of UE support.

Therefore, in order to support non-3GPP access for 6G, the learnings from 5G should be considered. 6G should target a single solution applicable for all types of devices and should balance implementation complexity (UE and network side) against solution capabilities, where focus should be on capabilities to fulfil mainstream use cases for non-3GPP access (mainly VoWiFi today, but also other use cases may be relevant for operators in a 6G time frame).

Observation 2: 6G solutions for non-3GPP should aim at a single solution with limited complexity to support different non-3GPP accesses and device capabilities, focusing on the mainstream use cases.

#### Technical Impact:

The current WT description in the 6G SID is:

WT#1.3: Study whether and how to support and/or enhance different non-3GPP access (e.g. Wi-Fi, wireline) in 6G and support multi-access data connections between 3GPP access and non-3GPP access.

Support multi-access data connections between 3GPP access and non-3GPP access can be assumed to build on the general support for non-3GPP access. It is therefore proposed to split this WT into two sub-WTs, one for general non-3GPP support and one for MA data connection support.

**** First Change ****

## Annex A.1.3 WT#1.3 Scope

WT#1.3 Support for non-3GPP access in 6G

- 	WT#1.3.1: Study whether and how to support connectivity over different non-3GPP accesses (e.g. Wi-Fi, wireline) in 6G, including support for session continuity between non-3GPP access and 3GPP access.

Study whether access specific solutions are needed for wireline and WiFi, or a common solution for both can be defined.

Solutions should minimize complexity in the device and in the network and ensure that they can be supported by devices of different capabilities. Introduction of different “non-3GPP device types” should be avoided. Devices shall not be required to support NAS via non-3GPP access and the network shall not be required to support NAS on behalf of the device. This WT targets a single solution for non-3GPP access support, common for different device capabilities.

- 	WT#1.3.2: Study whether and how to support multi-access data connections between 3GPP access and non-3GPP access.

**** Second Change ****

## 5.X Key Issue #X: Support for non-3GPP access in 6G

This key issue will study solutions that enable a UE to access via non-3GPP access (e.g. WiFi, wireline).

In 5GS, multiple solutions for non-3GPP access were defined to cater for variants in terms of device capabilities, different non-3GPP accesses etc. Most of those solutions have not been deployed on a wide scale.

This KI targets a single solution for non-3GPP access support, common for different device capabilities. I.e. solutions to the KI should minimize complexity to ensure that it can be supported by devices of different capabilities.

Solutions for this key issue will study:

- 	KI#X.1:

- 	UE connectivity establishment via non-3GPP access, including authentication and session establishment

- 	Session continuity between non-3GPP access and 3GPP access.

- 	KI#X.2: Multi-access sessions, i.e. sessions simultaneously active via 3GPP and non-3GPP access. This includes to study whether and how the 5G solution for Multi-Access PDU Session can be adapted in 6G.

NOTE: KI#X.2 is dependent on KI#X.1.

**** End of Changes ****