SA WG2 Meeting #170	S2-2506937

25 – 29 Aug, 2025, Goteborg , SE

Source:	Ericsson

Title:	[WT#3] WT and KIs on how to support and enable use of AI in 6G

Document for:	Discussion/Agreement

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This document provides justification and clarification of the scope of WT#3 and proposes Key issue descriptions according to the Wt#3 scope.

# 1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

The reference architecture in 5GC to support the analytics services defines NWDAF as the NF responsible to provide  those services to consumer NFs, to perform exposure of analytics services outside the 5GC domain and to support Life Cycle Management operations including training, monitoring, inference and to a certain extend deployment of ML Models needed to provide the analytics services. This architecture implies that the possibilities to use AI techniques, mainly ML models in 5GC, relies and depends on the use of NWDAF. The need to support AI in NFs has been discussed in 5GC then supported by LMF already. The LMF case can be seen as first step towards AI native NFs using inherent AI instead of relying on an external NF, such as NWDAF, performing the calculations.

AI native can lower dependencies between 6G entities (6G entities is used throughout the document referring to NFs for 6G), lower signalling, and as such, the capability to support AI techniques in each 6G entity is an opportunity to enhance the 6G architecture to support 6G procedures e.g. mobility management or session management using AI techniques.

It is vital not only to restrict AI/ML to predictions and statistics as offered by NWDAF in 5GC. It is essential to look at closed loops covering the requirements and follow-up per management responsibility. These closed loops may e.g. make use of Reinforcement and monitoring of the environment.

When placing AI techniques supported by a ML Model into 6G entities, the LCM operations for ML Models including training, monitoring, deployment of trained ML Models are close to the implementation specifics of each 6G entity vendor. Model LCM can be performed inside each entity. If needed to be defined, Model LCM is suggested to be in coordination with SA5, which specify generic end to end capabilities that are used for Core Network and RAN domain. This prevents duplication of specification work on more than one working group and the standardization of multiple architectural options to cover the different LCM operations. Placing AI capabilities in each 6G entity requires to study how the 6G entities integrate to the e2e LCM framework defined in SA5 and what core network domain specific aspects are needed in Core 6G entities.

Most of the LCM operations as well as Analytics/inference related to a ML Model require data collection that is based on subscription to events provided by 6G entities and information retrieved from OAM mainly from RAN. In 6G the data collection architecture should align with the conclusions of WT#5 (Study data framework) as such each 6G entities running an ML Model should use the data collection architecture defined in that WT and do not study any additional method.

One type of training that differs from central training and makes sense to perform between 6G entities in Core is federated learning. Especially VFL since training and inference of each local model is preferably done by same entity. The federated learning differs in the sense that data is not sent centrally but rather the intermediate results, therefore this should be studied in SA2.

The limitations in the existing 5G architecture include:

- Limited support for ML Models in the 5G NFs.

- No standardized support for other AI techniques, such reinforcement learning or federated learning cross all NFs, to support 5G system procedures.

- Lack of integration with the end to end LCM operations for ML Models.  Model LCM is done in SA5. SA2 may in coming releases put requirements on work done in SA5 for model LCM in training phase.

Possible impacts on the overall system architecture is to incorporate a new functional model for AI and for ML Models LCM. 6G entities can run ML models without interaction with NWDAF.

**** First Change ****

Annex A:
Work Tasks

Editor's note:	This Annex will capture updates to WT scope based on the 6G SID in SP-250806. The plan is to update the Study Item Description with updated Work Task descriptions from this Annex.

This Annex contains clarifications of Work Task scope, as preparation for Key Issue drafting.

## A.X 	Work Task 3

Editor's note:	Description of Work Task 3. Sub-clauses can be used if needed.

WT#3: Study how to support and enable use of AI in 6G (e.g. AI agent, framework). The framework includes support for native AI in 6G entities for the purpose to enhance the procedures and functionality defined in WT#1 (Define the overall 6G architecture). Whether or how the use of AI agents e.g. can be part of closed-loop AI techniques (such as re-enforcement Learning) is to be studied.

NOTE 1: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

NOTE 2:	This work task may require coordination with SA5 for the purpose to prevent conflicts between decisions made for the different 6G entities with AI capabilities.

**** Second Change ****

# 5.X. Key Issue #x:  Support for AI in 6G Core entities (framework)

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

The objective of this Key issue is to study which 6G features and functionalities, being defined in WT#1 (Define the overall 6G architecture), can be enhanced by using native AI in 6G entities. This KI will study whether it is an issue and how to avoid that conflicts may arise e.g. when different 6G entities optimize towards their goals. Standardized data used for inference is to be studied. If the inference is internal to 6G entities and not exposed externally then the output is not to be specified.

NOTE 1: The data collection is performed according to the conclusions of WT#5 (Study data framework) as such is low priority in this sub-WT.

NOTE 2:	This key issue may require coordination with SA5 for coordination of AI in 6G entities to prevent conflicts.

NOTE 3:	The Model LCM is low priority in this key issue and may depend on SA5.

# 5.X. Key Issue #y:  Vertical Federated Learning between 6G Core entities (framework)

When deploying and training a ML Model into a 6G entities, the existing Federated Learning techniques that are defined in 5GC for model training are also applicable for 6G entities. This key issue studies if and how Vertical Federated learning can be used between 6G entities to perform model training, monitoring and inference using the locally available data for this purpose. This key issue does not need input from WT#5 (Study data framework) as the training and inference is performed with the locally available data at the 6G entities but depends on the outcome of Key issue #x on AI in 6G entities.

NOTE: The data collection is performed according to the conclusions of WT#5 (Study data framework) as such is low priority in this sub-WT.

# 5.X. Key Issue #z:  Applicability of learning using Closed-loop AI techniques in 6G Core (framework, AI agents)

The objective of this key issue is to study whether and how Closed-loop AI techniques can be applied for 6G features and functionalities, being defined in WT#1 (Define the overall 6G architecture). This key issue will cover reinforcement learning (RL) as one of these closed-loop techniques, where an agent is placed inside an 6G entity and learn to make decisions and maximize a reward within its responsibility. This agent is believed to include AI and work as an AI agent. It is related to the native AI in 6GS studied in Key issue #x.

NOTE 1:	Closed-loop AI techniques in this context refer to a 6G Core entity that continuously monitors, evaluates, and adjusts its actions based on data and feedback received internally or externally to the 6G entity. This creates a feedback loop that enables the 6G entities to improve its performance over time and adapt dynamically to changing conditions. Some closed loops may be entirely inside one 6G entity and will in this case impose very little or no impact in standardization.

NOTE 2:	This KI may place some OAM requirement for the implementation of the closed loop therefore coordination with SA5 may be needed.

**** End of Changes ****