SA WG2 Meeting #170	S2-2506941

Goteborg, SE, 25 – 29 August 2025	(revision of S2-xxxxxxx)

Source:	Ericsson

Title:	[WT#8] Support for Massive IoT

Document for:	Approval

Agenda Item:	20.6.8

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides content proposal on WT#8 and the related KI for support for massive IoT in 6GS.

# 1. Justifications

One of the IMT-2030 usage scenarios is Massive Communication, which is described as follows in [1]:

Massive Communication This usage scenario extends massive Machine Type Communication (mMTC) of IMT-2020 and involves connection of massive number of devices or sensors for a wide range of use cases and applications. Typical use cases include expanded and new applications in smart cities, transportation, logistics, health, energy, environmental monitoring, agriculture, and many other areas such as those requiring a variety of IoT devices without battery or with long-life batteries. This usage scenario would require support of high connection density, and depending on use cases, different data rates, low power consumption, mobility, extended coverage, and high security and reliability.

In this contribution we provide input on the WT content for Massive IoT Communication.

Architectural Motivation:

A wide range of Internet-of-Things (IoT) use cases can be successfully served by the low-end 4G/5G based IoT technologies defined in 3GPP (NB-IoT, LTE-M/Cat-M, RedCap, eRedCap). These technologies are expected to live on for a long time, and 6G must be able to coexist with these technologies.

Eventually, 6G should be able to replace the legacy 4G/5G technologies for all significant use cases including IoT use cases. To facilitate a smooth migration to 6G, it is important that IoT use cases are considered already in the first 6G release. Otherwise, the alternative would be to retrofit a non-backwards-compatible IoT solution into 6G at a later stage.

Considering all the different services defined by other Work Tasks, it’s critical to have an understanding how massive IoT fits in 6G system and the potential impact to the architecture.

Gap Analysis:

Some fundamental Questions, see example list below, needs to be addressed

-	Investigate the implications of the lowest complexity device supported in 6G system with regards to functionalities supported by 6G system that are in SA2 purview.

-	Investigate the re-using and potential improvement/simplification possibilities of the 5G functions (e.g. RRC Inactive state) in 6G system, considering the influence in 6G system architecture work in Work Task #1.

-	Investigate potential simplifications to the UE power saving schemes.

-	Investigate improvements to DL data transfer to a group of devices.

-	How to support interworking with 5G system, considering the influence of 6G system architecture work in Work Task #2.

Technical Impact:

-	The Work Task has impact on the 6G system architecture.

-	The Work Task has impact on the interworking with 5G system.

# References

[1] 	M.2160-0 (11/2023), “Recommendation ITU-R M.2160-0: Framework and overall objectives of the future development of IMT for 2030 and beyond”, ITU-R, November 2023.

**** First Change (all new text) ****

# Annex A.X. WT#8 Support for Massive IoT

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

The study for the support of Massive IoT includes the following aspects:

-	Study potential implications of the lowest complexity device supported in 6G system with regards to functionalities supported by 6G system that are in SA2 purview.

NOTE: This depends on RAN WGs output.

-	Study whether, what and how to re-use 5G functionalities for support of Massive IoT in 6G system, e.g., re-use of RRC Inactive state functionality to minimize signaling overhead, re-use of Small Data Transfer (SDT) to enable efficient UP transfer.

-	Study whether and how to simplify/harmonize the UE power saving schemes.

-	Investigate improvements for scheduled DL data delivery to a group of devices.

-	How to support interworking with 5G system, considering the influence of 6G system architecture work in Work Task #2.

**** Second Change (all new text) ****

# 5.X. Key Issue #X: Support for Massive IoT

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

This KI investigates the support of Massive IoT includes the following aspects:

-	Study potential implications of the lowest complexity device supported in 6G system with regards to functionalities supported by 6G system that are in SA2 purview.

NOTE: This depends on RAN WGs output.

-	Study whether, what and how to re-use 5G functionalities for support of Massive IoT in 6G system, e.g., re-use of RRC Inactive state functionality to minimize signaling overhead, re-use of Small Data Transfer (SDT) to enable efficient UP transfer.

-	Study whether and how to simplify/harmonize the UE power saving schemes.

-	Investigate improvements for scheduled DL data delivery to a group of devices.

-	How to support interworking with 5G system, considering the influence of 6G system architecture work in Work Task #2.

**** End of Changes ****