3GPP TSG-SA WG2 Meeting #170	S2-2506960

Goteborg , SE, Aug 25 – 29, 2025

Source:	Mediatek Inc.

Title:	KI#2: New Sol: IMS call optimization for NB-IoT (GEO)

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes a minimal impact solution for IMS session (e.g. voice call) setup time optimization.

Introduction

This contribution proposes a solution for IMS session (e.g. voice call) setup time optimization by disabling precondition mechanism and omitting the setup of a dedicated bearer.

Discussion

Due to GEO satellites having a very limited bandwidth (1-3kbps) and a high delay, IMS session setup optimizations are needed. Typical IMS signaling may include multiple SIP requests and responses (messages). E.g.: INVITE → 100 Trying → 183 Progressing → PRACK → 200 OK → UPDATE → 200 OK → 180 Ringing → 200OK → ACK. NAS signaling is needed for the dedicated bearer setup. With limited resources this leads to a long call setup time.

To reduce the number of the SIP messages and their sizes and to reduce the amount of NAS and AS signaling in NB-IoT (GEO), following changes are proposed:

- Disable IMS precondition mechanism;

- Omit the setup of the dedicated bearer for the voice.

Proposal

A solution is proposed for KI#2 for incorporation in the FS_5GSAT_Ph4_ARC TR 23.700-19.

* * * * Start of Change * * * *

6.0	Mapping of solutions to key issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| X |  | X |  |  |

* * * Next Change (all new texts) * * * *

6.X	Solution #X: IMS call optimization for NB-IoT (GEO)

6.X.1	High level principles

This solution addresses Key Issue #2: "IMS Enhancements for GEO NB-IoT NTN Access".

This is a solution to reduce time for IMS session (e.g. voice call) setup over NB-IoT (GEO) access proposing the signalling and message size optimizations. The solution could be further extended to other IP-CANs also.

The signalling and message size optimizations proposed in this solution can be implemented according to existing 3GPP specifications (e.g., TS 24.229) with minimal changes to protocols.

The objective is to disable precondition mechanism and omit the setup of a dedicated bearer i.e., the voice data is routed over the default bearer used also for SIP signalling.

Other optimizations can be utilized on top of this solution.

6.X.2	Description

The solution proposes a solution for IMS session (e.g. voice call) setup time optimization by disabling precondition mechanism and omitting the setup of a dedicated bearer. Disabling of the precondition mechanism leads to:

- Reduced amount of SIP messages in the session setup

- Smaller SIP messages

- Reduced amount of NAS and AS messages (no dedicated bearer setup)

On top of this solution, multiple other optimizations can be utilized (e.g. compact headers, more optimal header usage, signalling compression etc.) not limited by this proposal.

6.X.3	Procedures

6.X.3.1		MO (call) session setup

The solution focuses on mainly on voice call session setup, but similar session setup flow is applicable also for other media types. The solution aims to cause as little impact for the UEs and NW implementation as possible.

Figure 6.X.3.1-1 illustrates the SIP signaling sequence with precondition mechanism and with a dedicated bearer. Figure 6.X.3.1-2 illustrates the SIP signaling sequence without precondition mechanism and without a dedicated bearer.

In Figure 6 X.3.1-2 the sizes of the SIP messages are smaller as e.g. Supported header would be shorter (no “precondition” or “100rel”) and QoS related parameters can be omitted from the SDP bodies. More reduction in message sizes can be achieved with further optimizations (e.g. compact headers, more optimal header usage, signalling compression etc.)

Figure 6.X.3.1-1: IMS call session setup with preconditions and dedicated bearer

Figure 6.X.3.1-2: IMS call session setup without preconditions and dedicated bearer

### 6.X.4	Impacts on services, entities and interfaces

UE:

- No dedicated bearer activation for voice.

- Reducing message sizes and signaling with no significant impact on the existing UE IMS implementation.

P-CSCF:

- No dedicated bearer activation for voice.

- Reducing message sizes and signaling with no significant impact on the existing P-CSCF implementation.

* * * * End of Change * * * *