SA WG2 Meeting #170	S2-2506974

25 – 29 Aug, 2025, Goteborg , SE

Source:	TCL

Title:	[WT#5] Key Issue on Unified Data Framework Design for 6G

Document for:	Approval

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes to add a new key issue on data framework.

Introduction

In the next generation mobile network systems, Artificial Intelligence (AI), Integrated Sensing and Communication (ISAC), and Digital Twin (DT) will be introduced to further enhance communication capabilities and user experience. However, these emerging technologies also impose new design requirements on networks. For instance, the deployment and operation of AI, ISAC, and DT involve extensive data collection, transmission, processing, storage, and retrieval processes. These processes not only require traditional features like low latency, high speed, and high reliability but also demand network support for data transmission between arbitrary endpoints or on-path processing/on-path reception during transmission. Although enhancements have been made to control plane and user plane transmission in 5G discussions, existing 5G enhancement solutions still fall short of meeting the core requirements for such data transmission due to limitations in control plane transmission rates and fixed user plane transmission paths. Consequently, new technologies like AI, ISAC, and DT cannot perform optimally. To fully support these technologies in the next generation mobile communication networks, the system must possess native data service capabilities (e.g., Data Plane). Specifically, the system needs to address the following key issues.

Proposal

It is proposed to add the following Key Issues to the TR 23.801-01 “Study on Architecture for 6G System”.

**** First Change ****

5	Key Issues

Editor's Note: This clause identifies key issues to be addressed.

5.1	Key Issue #1: Data Life Cycle Management

To comprehensively support data service requests from any node, end-to-end data service management, and quality-of-service oriented data transmission across arbitrary paths in next-generation mobile communication networks, the following issues need to be addressed:

- Based on 5G system architecture, new data plane functionalities must be incorporated into the next-generation network architecture to enable comprehensive data service capabilities.

- How data plane functionalities manage data collection nodes, data transmission, data processing, data storage, data exposure and security, while ensuring quality of service at every stage.

- How to provide unified data service interfaces for different service requestors and implement full data life cycle management from service requests to data delivery.

- Given that the service targets of data plane functionalities have expanded beyond traditional user nodes for data transmission to include data consumers, and considering that data transmission may bypass conventional billing elements like UPF, redesigning billing mechanisms and policy control has become a crucial aspect of data plane management.

5.2	Key Issue #2: Differentiated Management and Efficient Reuse of Data

Given that the requirements for data services between the core network and RAN may differ (e.g., end-to-end latency, security and privacy) or overlap (e.g., requiring the same type of data in identical geographical areas), a critical challenge in enhancing data service capabilities lies in how the network can ensure efficient data reuse while providing differentiated services and security safeguards. Specifically, the following issues must be addressed:

- How to support differentiated data service requirements for the core network and RAN, such as end-to-end latency, data security and user privacy, and area-specific data characteristics. How to redesign RAN protocol stack processes to satisfy specific data service requirements on the RAN side.

- How to guarantee efficient data reuse and reduce redundant control overhead, encompassing both real-time reuse (e.g., concurrent usage of identical data by multiple tasks/users) and non-real-time reuse (e.g., asynchronous utilization of the same data).

- How to ensure cross-layer or cross-regional data service quality and data security.

**** End of Changes ****