3GPP TSG-SA WG2 Meeting #170	S2-2506983

Goteborg, Sweden, August 25 – 29, 2025

Source:	MediaTek Inc.

Title:	[WT#1.2, Policy] Policy delivery in 6G

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: The contribution addresses scoping of WT#1.2 on policy delivery in 6G covering Motivation, Gap analyses and expected Technical Impact and suggests some potential scope for relevant Key Issue(s).

1. Justifications

Architectural Motivation:

The UE policy provisioning is always provided via initial registration and mobility registration update when UE moves from EPS to 5GS. When UE switches on, the UE needs to wait until all mobility management steps completed before the UE can establish an internet PDU Session, which lead to an obvious delay for service access.

Furthermore, the provisioned policy rules are not used or preferred by the UE but the UE still needs to spend time on control signaling procedure, i.e., the UE needs to receive the policy rules provided by PCF since PCF considers the UE lack of the corresponding policy rules for accessing the services.

Gap Analysis: The policy provisioning does not consider whether the UE needs the policy and enforce the UE to spend time on signaling procedure to always accept the policy rules provided or updated from the network side. The network does not know about the service or application condition triggered in the UE side

Technical Impact:

-	Decouple the policy provisioning from initial registration to speed up the internet PDU Session establishment

-	The policy provisioning rules sync between the UE and network by considering the UE preference

**** First Change ****

Annex A.X. WT#X.2 Scope

Extracted below from SP-250806:

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

NOTE  1:	The duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split.

1.1.	Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

1.2.	Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

1.3.	Study whether and how to support and/or enhance different non-3GPP access (e.g. Wi-Fi, wireline) in 6G and support multi-access data connections between 3GPP access and non-3GPP access.

1.4.	Study whether and how to support and/or enhance the essential/regulatory services (i.e. voice, Messaging, location services, Emergency services, MPS, Mission Critical services, PWS) in 6G.

Note the below update is just intended as information in the TR and no change is expected in the WT wording in the SID.

Study aspects on whether and how to enhance the UE policy request and provisioning procedure based on the UE preference

**** Second Change ****

5.X. Key Issue #Y: Policy provisioning triggering

Study aspects on whether and how to enhance the UE policy request and provisioning procedure based on the UE preference

**** End of Changes ****