SA WG2 Meeting #170	S2-2506985

25 – 29 Aug, 2025, Goteborg , SE

Source:	vivo

Title:	[WT#6, 6G computing] 6G computing for UE

Document for:	Agreement

Agenda Item:	20.6.6

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution discusses the architecture motivation for 6G computing and proposes to update the scope of WT#6 and the KI for 6G computing.

1. Justifications

Explain the technical relevance of the proposed Work Task (WT) and (if possible) Key Issue (KI) in the context of SA2’s scope (i.e., system architecture).

1.1 Architectural Motivation

Architectural Motivation: What architectural challenges or opportunities does this WT/KI address?

According to the SA1 study for 6G and TR22.870v0.3.1, there are several use case and requirement requiring 6G network to provide 6G computing service to UE as summarised in the following table:

Table 1.1-1: SA1 computing related requirements per TR22.870v0.3.1

| category | Potential requirements |
| --- | --- |
| Computing service provision | - support a mechanism for providing Computing Services to user (via UE). - expose different services, e.g. computing offloading service in Service Hosting Environment, to the authorized third-party AI agent. |
| Computing service requirements/QoS | - receive the requested computing requirements of computing services. - fulfil the user-requested performance requirement (e.g. latency) for the computing service. |
| Computing Resource selection | - support selection of computing resources in Service Hosting Environment for a computing task. - selection of computing resources in the Service Hosting Environment upon request from AI-based applications. |
| Security | - ensure security and privacy when providing Computing Services. |
| Charging | - collect charging information for the computing resource(s) for providing computing service. |

Observation 1: 6G architecture needs to support the SA1 requirements to support 6G computing service for UE, i.e. supporting offloading computing task from UE to the network.

Figure 1.1-1: 6G computing service offloading UE computing task to the computing resource controlled by 6G network

There is abundant computing resource in 6G network. There is motivation for 6G operator to providing 6G computing service utilizing the computing resource in the 6G network or controlled by 6G network for monetization. The 6G operator could provide computing resource to support the 6G computing service in the similar way as cloud provider, providing to another option for user and application service provider. the 6G computing service could face the similar challenge as could computing, such as: security and scalability, data privacy, architecture customization and etc. and the 6G computing service can Leverage the experience of cloud computing.

There is plenty network data in 6G network. 6G operator could provide unique computing service with better user experience utilizing the network data which is not available in could computing.

There are ubiquitous connections supported by 6G network. 6G computing service could take advantage of both ubiquitous connections and computing resource inside 6G network to better meet the user computing service requirements

Observation 2: there are strong motivation for 6G operator to support 6G computing service for monetization utilizing the computing resource, connection resource and other resources controlled by 6G network, motivated to offload UE computing task to 6G network, not to cloud.

1.2 Gap Analysis

Gap Analysis: What are the limitations or gaps in existing architecture (from Rel-15 to Rel-20) that this proposal aims to resolve?

This section is to provide gap analysis between 5G edge computing and 6G computing.

As shown in the following Figure 1.2-1, for 5G edge computing, the computing resource is provided by the EAS which is outside of 5G network, the computing resource is not controlled by 5G network, leading to:

-	separate control or separate schedule of computing resource and communication resource;

- 	the information exposure between 5G network and EAS is limited and non-real time;

-	the policy control and QoS mechanism of 5G network only apply to communication aspects;

-	separate schedule of computing resource and communication resource, leading to locally optimal solution for both user experience and resource utilization efficiency, not form holistic view.

Considering in 5G edge computing, the EASDF uses DNS query mechanism to select EAS based on FQDN and other pre-configured information, which is static and not able to enhance to provide real time computing status of EAS, e.g. whether the computing processing delay will increase due to not enough computing resource.

While for 6G computing, both computing control function and the computing resource could be controlled by 6G network, leading to：

- 	joint control and schedule of computing resource and communication resource by 6G network；

-	fully and real time information exchange could be supported within the 6G network, including both computing related information and communication related information.

-	the policy control and QoS mechanism could jointly apply to both communication aspect and computing aspect, leading to globally optimal solution for both user experience and resource utilization efficiency.

Figure 1.2-1: comparison between 6G computing and 5G edge computing

Observation 3: for 6G computing, the computing resource is located in or controlled by the 6G network, under the guidance of a unified policy, the 6G network can achieve joint scheduling of computing resources and communication resources, achieving globally optimal solution for both user experience and resource utilization efficiency.

1.3 Technical Impact

Technical Impact: How would this WT/KI influence:

1.3.1 Overall system architecture

Figure 1.3-1 shows the system architecture to support 6G computing. There are two logical network entities introduced to the 6G network: computing node and computing control NF.

The computing node is the abstracted representation of computing resource in 6G network or computing resource controlled by 6G network. It represents not only the (centralized or distributed) hardware resource (e.g. CPU, GPU, NPU, memory, storage, and other infrastructure), but also the corresponding software resource or platform to manage, drive, and abstract the underlay hardware using technology such as NFV and etc. It is out of 3GPP scope on how the computing node manages and drives the hardware resources. However, it is in 3GPP scope and needs SA2 to study how the computing node provide abstracted/standard information of computing resource for 6G network to control and select.

Base on the computing resource information provided by the computing node, the computing control function need to understands the requirement from UE computing task, translates into the requirement for computing resource, select the computing resource and allocate selected computing resource to UE, in order to fulfil the service performance requirement for UE computing service together with communication control function under policy control.

Figure 1.3-1: system architecture to support 6G computing

1.3.2 Interaction between network functions/services?

Computing control NF:

-	needs to coordinate with policy control function to determine the QoS mechanism for computing aspects, based the service request for a computing task;

-	need to coordinate with computing to select and allocate proper computing resource for UE’s computing task.

-	need to coordinate with communication control function to establish and control the computing bearer to perform the computing task utilizing the allocated computing resource.

Computing node:

-	need to provide (abstracted or standardized) computing resource information to computing control function;

-	need to select and allocate proper computing resource under control of computing control function.

**** First Change ****

Annex A.X. WT#6 Scope

WT#6: Study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination between UE, core network and application server, exposure of computing service in the core network, etc.), including:

- how to support 6G computing service for UE, i.e. how to support offloading UE computing task to 6G network;

- how to support 6G computing service for application server, i.e. how to support AF to invoke the computing resource provided by network.

NOTE 7: Application layer mechanism and exposure framework may require coordination with SA6.

**** Second Change ****

5.X. Key Issue #X: System architecture and functionality design to support 6G computing service for UE

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

This Key Issue aims at:

- How to support UE to request for offloading a computing task to 6G network;

- How to support computing resource control and registration;

- How to support computing resource selection based on the requirement of UE computing task;

- How to support offloading UE Computing task to selected computing resource;

- How to support joint policy control and joint QoS mechanism (covering both computing aspect and communication aspect) to meet user service performance requirement;

- How to support service continuity for 6G computing service, i.e. relocation of computing resource for 6G computing service.

**** End of Changes ****