3GPP SA WG2 Meeting #170	S2-2506988

25 – 29 Aug, 2025, Goteborg , SE

Source:	Qualcomm Incorporated

Title:	[WT#1.1] Update to WT#1.1 for clarification on flexible addition of new services in 3GPP

Document for:	Approval

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution elaborates on the need for flexible addition of new NAS services in 6G.

# 1. Justifications

Historically, in a new G, the first release and especially the first deployments provide essential connectivity and session functionality, e.g UE to NW NAS protocol, network functions, inter network function procedures, etc.

However, complexity of addition of new services has increased substantially due to:

- Full stack solutions, with their own configuration solution, transport solution, routing solution.

- Solutions that have service specific impacts in intermediate nodes, typically the MME/AMF/SMF, S1/N2 interface, RAN, and other impacts north of AMF/SMF or MME.

This causes that after first deployment, a new service typically has required upgrades across large parts of the deployed network, and come with their own impacts across all layers.

For 6G, 3GPP should develop a forward-looking framework that allows, as much as possible, the introduction of new services into the 6GS without or at least with minimal impacts in the network functions and interfaces deployed for connectivity. This should be the case for both services offered over the control and over the user plane.

This should allow for a more flexible, less costing, evolution of the network as new use cases and solutions are developed for 3GPP.

Specifically for control plane services, which are covered in WT 1.1a and WT 1.1b, it is proposed to introduce a new Key issue in TR 23.801-01.

Proposal 1: It is proposed to include a Key issue in 23.801-01, to cover WT#1.1a and WT#1.1b, to address the need for flexible evolution and addition of new services with minimal impact to first release connectivity functionality, and identify a minimum set of NAS functionality that is not impacted by other NAS functionalities and/or features.

Specifically for operator services offered over the user plane  (e.g. data collection for UE side model training, location services over user plane, and potentially any new 6G service if decided to be offered over user plane), 3GPP should avoid defining completely different solutions at all layers, and instead identify potential underlying functionality that is common to any service. For instance, all services require to be discoverable and/or configurable in the UE, the UE needs to be authorized for receiving the specific service, a session needs to be established to provide a user plane transport path between the UE and the service, and 3GPP security needs to be established.

Proposal 2: It is proposed to amend WT#1.1 and include corresponding Key issue in TS 23.801-01, to study a common framework for services offered over the user plane, including:

- Service Discovery & configuration

- Service authorization

- Session configuration and establishment

- Security (in collaboration with SA3)

# 2. Proposal

It is proposed to include the following changes to TR 23.801-01:

**** First Change ****

## A.X 	Work Task 1

Editor's note:	Description of Work Task X. Sub-clauses can be used if needed.

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

NOTE  1:	The duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split.

1.1.	Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services offered over the control plane.

1.2.	Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

1.3.	Study whether and how to support and/or enhance different non-3GPP access (e.g. Wi-Fi, wireline) in 6G and support multi-access data connections between 3GPP access and non-3GPP access.

1.4.	Study whether and how to support and/or enhance the essential/regulatory services (i.e. voice, Messaging, location services, Emergency services, MPS, Mission Critical services, PWS) in 6G.

1.5			For operator services offered over the user plane, identify functionality that may be common to the establishment of such services (e.g. service discovery and configuration, service authorization, session configuration and establishment, security) and study a common solution framework.

NOTE 2: Security aspects require coordination with SA3.

….

**** Second Change ****

## 5.X	Key Issue #X: Key Issue on flexible evolution and introduction of new NAS services without impacting previous NAS functionality

In order to enable a fast, cost effective and flexible evolution of the 6GS functionality and services, the 6G system’s essential control functionality and protocols should be defined in a manner that - once 6G system is deployed - the introduction of new functionality and services does not require large impacts across the system. This would allow introducing new services or evolving certain functionality in the system without affecting other parts of the system, reducing inter-functionality dependencies.

This key issue should address:

-	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

-	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

-	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services offered over the control plane.

**** Third Change ****

## 5.Y	Key Issue #Y: Key Issue on common framework for operator services offered over the user plane

For operator services offered over the user plane  (e.g. data collection for UE side model training, location services over user plane, and potentially any new 6G service if decided to be offered over user plane), 3GPP should avoid defining completely different solutions at all layers, and instead identify potential underlying functionality that is common to any service. For instance, all services require to be discoverable and/or configurable in the UE, the UE needs to be authorized for receiving the specific service, a session needs to be established to provide a user plane transport path between the UE and the service, and 3GPP security needs to be established.

This key issue should identify underlying functionality that may be common to different operator services offered over the user plane, and study a common framework for those functionalities, including:

-	Service Discovery & configuration

-	Service authorization

-	Session configuration and establishment

-	Security

NOTE: Security aspects require coordination with SA3.

**** End of Changes ****