3GPP TSG-SA WG2 Meeting #170	S2-2507006

# 25-29 August, 2025, Goteborg , Sweden	(Revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0033 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on AIOTF selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In real AIoT deployment, AIOTF should be able to be deployed flexibly, i.e. AIOTF can be deployed to serve specific customers, e.g. AIOTF can be deployed locally in customer’s field. Therefore it shall be also supported to select AIOTF based on AF identifier provided by AF. - It is also mentioned in the specification that NEF may use last known AIOTF instances stored in ADM. But it is not clear how NEF handles the situation that multiple AIoT devices have multiple last known AIOTF instances stored. It is proposed to clarify that NEF handles it by implementation for simplicity. It is clarified that if multiple AIOTFs are selected, the NEF need to send multiple AIoT service requests to each AIOTF with corresponding Target Area Information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that AF may use AF iD to select AIOTF instance. Clarify if multiple last known AIOTFs are selected, the NEF need to send multiple AIoT service requests to each AIOTF with corresponding Target Area Information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.3.1	AIOTF Discovery and Selection

The AIOTF discovery and selection functionality is to determine an AIOTF(s) to handle an AIoT service operation request.

The NEF determines AIOTF instances(s) by providing the NRF Target Area information and the NRF returning AIOTF instance(s) that match the Target Area information, or by using local configuration.

The NEF may use AF ID to select AIOTF instance(s).

A service operation request received by the NEF from an AF may include External Target Area information and the NEF uses it to determine the Target Area information that is provided to the NRF, if used. The External Target Area information is a pre-configured External Area Identifier or geographic area (e.g., a civic address or shapes). The Target Area information is a list of AIoT Areas.

NOTE:	The mapping between AIoT Areas and External Area Identifiers provided by an AF is configured in the NEF.

When the information about the target AIoT Device(s) in a service request  indicates individual AIoT Device(s), the AIOTF instance(s) may be selected by taking into account the last known AIOTF instance(s) (e.g. AIOTF ID/address) for those AIoT Device(s) obtained from the ADM. If there are multiple AIOTF instances stored for different AIoT devices, whether and how the NEF selects one or multiple AIOTF(s) for the request is up to NEF implementation.

When the NEF forwards the AIoT service request to the selected AIOTF, if multiple AIOTFs are selected, the NEF sends multiple AIoT service requests to selected AIOTFs along with corresponding Target Area information.

# * * * End of changes * * * *