SA WG2 Meeting #170	S2-2507016

25 – 29 Aug, 2025, Goteborg , SE

Source:	Qualcomm Incorporated

Title:	[WT#1.2, Network Slicing] Key issue on enhancements to Network Slicing for 6G

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution elaborates on the specifics of Network Slicing scope within WT#1.2.

# 1. Justifications

Network slicing is one of the features in WT#1.2 subject to study whether and how to support and/or enhance in 6G.

The current network slicing design has two drawbacks:

Drawback of 5G Network Slicing #1: Unnecessary signaling overhead for scenarios where only a single data session is used

There are some scenarios, e.g., LPWA, that typically do not require multiple data sessions. However, the current network signaling design is not very efficient for these scenarios (i.e., when only a single data session is used) due to unnecessary signaling overhead.

Proposal #1: It is proposed to study enhancements to Network Slicing in order to optimize signalling overhead for network slicing for different scenarios, uses cases and/or verticals.

Drawback of 5G Network Slicing #2: Lack of awareness of Network Slice capabilities at the UE

In 5G Network Slicing, the UE is unaware of the KPIs and capabilities that are provided by a PLMN for a given S-NSSAI.

This is an issue for scenarios where the binding of applications to slices is not done based on AppID but using other means.

For instance, there have been efforts to enable operators to bind slices to traffic categories (NSSP in URSP), and the HLOS to then bind applications to those traffic categories. GSMA has attempted to define semantics (KPIs and capabilities) for those traffic categories, see for example GSMA NG.141 [1], NG.116 [2] and NG.135 [3] for more details. However, a standardized approach turned out to be too inflexible as it prevents operators from deciding which KPIs to support per traffic category based on deployment, area, etc.

Proposal #2: It is proposed to study solutions for PLMNs to provide the UE with the KPIs allocated to each network slice, and this information to be exposed to the UE’s upper layers to facilitate binding of applications to network slices.

# 2. Proposal

It is proposed to include the following changes to TR 23.801-01:

**** First Change ****

## 5.X	Key Issue #X: Key Issue on Network Slicing support in 6G System

As a 5G prominent feature, Network Slicing should be supported in 6G system.

This key issue objectives are:

-	Study whether and how to support Network Slicing features in 6G, including configuration, authorization, policies, etc.

-	Study enhancements to Network Slicing including:

-	Optimization of signalling overhead due to Network Slicing for different scenarios, use cases and verticals, e.g. when only a single data session is used

-	Enhancements to facilitate the binding of applications to Network Slices when appID is not available to the 3GPP system, e.g. via enabling awareness of network slice capabilities at the UE.

**** Second Change ****

## A.X 	Work Task 1.2

Editor's note:	Description of Work Task X. Sub-clauses can be used if needed.

1.2.	Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

a. For Network Slicing, study whether and how to support the following enhancements:

a.1: Optimization of signalling overhead due to Network Slicing for different scenarios, use cases and verticals, e.g. when only a single data session is used,

a.2: Enhancements to facilitate the binding of applications to network slices when appID is not available to the 3GPP system, e.g. via enabling awareness of network slice capabilities at the UE.

**** End of Changes ****

# 3. References

[1] GSMA NG.141: Guidelines for URSP

[2] GSMA NG.116: Generic Network Slice Template and NESTs

[3] GSMA NG.135: E2E Network Slicing Requirements