SA WG2 Meeting #170	S2-2507018

25 – 29 Aug, 2025, Goteborg , SE

Source:	Cisco Systems, Deutsche Telekom,

Title:	[WT#1.2, user plane enhancements] User Plane Enhancement for 6G

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes Sub-Work Tasks (WTs) and/or Key Issues (KIs), including the rationale, expected scope for user plane enhancements for 6G.

# 1. Justifications

Operators are facing economic and ecological pressure to significantly increase efficiency. They need approaches which significantly reduce overhead, decrease latency, and enable the network to rapidly adapt to diverse and dynamic service requirements. Applications being served as well as underlying transport networks have significantly evolved and are now fully IP-centric. This drives the requirements for a more intelligent, adaptive user plane, enabling sustainable and cost-effective service delivery.

In this contribution, we aim to provide justification for studying architectural evolution of user plane function (UPF) for addressing the needs for future 6G network. User plane enhancement is a foundational enabler for 6G networks to support the new performance guarantees, ultra-low latency and hyper reliable targets as specified by IMT 2030 (x1). In addition to this flexibility, observability, AI-related capability and automation are some of the important needs by future 6G applications and business models. Without enhancements of the user plane , the  3GPP system would not be able to deliver on the full promise of 6G.

## Key Reasons for User Plane enhancement

Some of the key reasons for User plane enhancements are as follows:

- Support for enhanced capabilities

IMT-2030 (subclause 4 of Ref 1) defines the framework and lists enhanced capabilities requirements for 6G network. Sub-millisecond latency, 1Tbps data rate, mobility support at 1000km/h and connection density of 106 to 108 devices/km2 are some of the key performance requirements listed for the 6G system. To support these extreme requirements, the current user plane architecture and design need to be enhanced.

- Support for energy-efficient user plane processing

Existing user plane mobility anchors require per-session state replication across UPFs with handovers generating up to 30–40% control plane/user plane signalling overhead and ~25% higher CPU utilization in mobility-intensive scenarios. In addition to this, the UPF performs a range of computation intensive packet processing operations which results in significant resource consumption. Protocol processing alone contributes 20–35% of UPF’s power consumption. The UPF is one of the most energy-demanding network function, primarily due to the high processing load associated with user plane protocol operations and the necessity to sustain massive throughput for diverse applications. Considering these aspects, 6G UP shall support energy-proportional forwarding and state-lite mobility procedures reducing handover signalling load by >25%.

- Real-time performance monitoring and quality assurance

UPF needs enhancement with deep packet inspection (DPI) and interfaces that monitor network quality in real time, allowing operators to analyse traffic trends, monitor per-application quality, and deliver SLA-guaranteed services for advanced 6G use cases like IoT, smart cities, and industrial automation. At the same time, the User Plane needs to support such enhanced monitoring capabilities flexibly and selectively, for highest efficiency.

- Seamless integration with external analytics and control systems

Enhanced UPF features include interfaces to expose real-time network data to external analysers and systems. This supports advanced analytics, automation, and AI-driven network optimization for 6G use cases, resulting in greater flexibility and value-added services.

- Service-Based Architecture (SBA) Integration

UPF enhancements are required for tighter integration into the 5G Core’s Service-Based Architecture, including direct and indirect data subscription, improved UPF selection, event exposure and better connection management between UPF and other NFs e.g. full SBI adoption for N4 interface between UPF and Control Plane. This supports more dynamic and application-aware traffic handling, which is essential for diverse, low-latency 6G services.

- Support for diverse and stringent Service Level Agreements (SLAs)

6G will serve use cases ranging from IoT, smart cities, and industrial automation to immersive AR/VR and critical infrastructure. Each may require different guarantees for latency, throughput, reliability, and coverage. Enhanced UPF must interpret, enforce, and adapt to these varied SLAs in real time to meet application and user requirements.

- AI/ML integration

UPF enhancement is required for further integration with AI/ML-driven frameworks, enabling coordinated intelligence across user equipment (UE), RAN, and the 6G core. The enhanced UPF can serve as a critical data and control point for distributed intelligence in 6G networks.

- Security enhancements

The enhanced UPFs should support for continuous surveillance of traffic patterns for meeting enhanced security needs for future 6G network. This enables swift detection of anomalous behaviour, detecting emerging threats, improving the ability to contain and mitigate attacks before they impact users using various AI advancements.

- Support for new transport protocol

Most of the performance goals, reliability and scalability targets, SLA driven traffic management aspects, new security needs, flexible and programmable UPF which can adjust itself to the current traffic needs and AI/ML integration needs of future 6G UPF cannot be met with the current transport protocol. The study should focus on evaluating various options for new transport protocol for 6G network.

**** First Change (all text new)****

# Annex A.1.2.X WT#1.2 Scope for enhancing user plane architecture for 6G

Sub-Work Task 1.2 includes:

“1.2. Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.”

The Sub-Work Task for user plane architecture for UPF enhancement includes the following aspects:

- Study how to enhance the user plane architecture and its control to achieve performance, ultra reliability, and low latency goals defined in IMT-2030 [x1], enable 6G services and meet operator’s requirements specified in TR 22.780 [x2], regarding efficiency, latency, and flexibility.

  - Explore new transport protocol which can meet performance, scalability, security and other needs of the future 6G network.

  - Study user plane Interworking and Migration in coordination with WT#2.

  - Study how to fully integrate with the control of the user plane with the SBA architecture.

- Study how to enable efficient path programming and context aware forwarding in the user plane functions with the focus on performance optimization, diverse service/application and dynamic QoS requirements.

- Study how to leverage AI/ML to achieve autonomous user plane adaptation and improved resource management.

- Study how to improve security and resilience in the user plane.

**** Second Change (all text new) ****

# 5.1.2.x Key Issue #1.2.X: 6G User Plane Enhancement

This key issue addresses the need to enhance the user plane architecture in 6G to achieve greater efficiency and flexibility in order to support diverse services/applications and traffic patterns.

The KI includes the following aspects:

- Study how to enhance the user plane architecture and its control to achieve performance, ultra reliability, and low latency goals defined in IMT-2030 [x1], enable 6G services and meet operator’s requirements specified in TR 22.780 [x2], regarding efficiency, latency, and flexibility.

  - Explore new transport protocol which can meet performance, scalability, security and other needs of future 6G network.

  - Study user plane Interworking and Migration in coordination with WT#2.

  - Study how to fully integrate with the control of the user plane with the SBA architecture.

NOTE 1: 	 Flexible and programable UPF design aspects should be considered for better scalability and management.

NOTE 2:   Co-ordination with RAN and CT group is also required for evaluation of new transport protocol.

- Study how to enable efficient path programming and context aware forwarding in the user plane functions with the focus on performance optimization, diverse service/application and dynamic QoS requirements.

- Study how to leverage AI/ML to achieve autonomous user plane adaptation and improved resource management.

NOTE 3:   The common AI/ML framework for 6G core should be reused for UPF Architecture.

NOTE 4:   AI/ML design and services framework of UPF should be aligned with common 6G AI/ML Architectural principles.

- Study how to improve security and resilience in the user plane.

NOTE 5:   Some of the security aspects may need to be evaluated by SA3.

.

**** Third Change ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3] 	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4] 	3GPP TS 23.503: "Policies and Charging control architecture; Stage 2".

[5] 	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[6] 	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

…

[x1] 	Recommendation ITU-R M.2160-0: "Framework and overall objectives of the future development of IMT for 2030 and beyond". (https://www.itu.int/dms_pubrec/itu-r/rec/m/R-REC-M.2160-0-202311-I!!PDF-E.pdf).

[x2] 	3GPP TR 22.870: "IP Study on 6G Use Cases and Service Requirements".

**** End of Changes ****