SA WG2 Meeting #170	S2-2507023

25 – 29 Aug, 2025, Goteborg , SE

Source:	LG Electronics

Title:	[WT#1.2, User plane architecture] Scope and New Key Issue for User plane architecture

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This paper proposes scope refinement and new KI for WT#1.2 about user plane architecture.

1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

Regarding the following WT#1.2 included in the FS_6G_ARC SID, scope needs to be refined and new Key Issue should be added to TR 23.801-01.

WT#1.2 Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

Architectural Motivation:  The scalability for the distributed user plane can be achieved with the I-UPF controlled by the I-SMF in the 5G system, e.g. with Enhancing Topology of SMF and UPF in 5G Networks feature. Also, the I-UPF and I-SMF can be inserted to support Local Offloading Management feature for edge computing. Such design adds complexity and signalling overhead for the user plane and control in use cases (e.g. mobility) with I-SMF insertion / change / removal. Therefore, it is desired for the simplified procedure and the interaction among NFs in the 6G system to reduce complexity and signalling overhead.

Gap Analysis: Current design of the network for the use of optional I-UPF controlled by the I-SMF increases signaling overhead and complexity with insertion / change / removal of the I-SMF / I-UPF.

Technical Impact: This WT/KI would influence the following for the user plane architecture:

- Functions for 6G AMF, (I-)SMF and (I-)UPF.

- Interactions between the (I-)SMF and (I-)UPF, SMF and I-SMF, AMF and (I-)SMF.

# * * * * Start of 1st Change (All New Texts) * * * *

A.X 	Work Task 1.2.x

WT#1.2.x Study whether and how to enhance the user plane architecture to reduce signalling and simplify the procedure when I-SMF/I-UPF are involved.

# * * * * Start of 2nd Change (All New Texts) * * * *

5.X	Key Issue #X: Enhancement of user plane architecture

This key issue studies how to enhance the user plane architecture to reduce signalling and simplify the procedure when I-SMF/I-UPF are involved. The frequent change in the user plane with I-SMF/I-UPF insertion, change and removal adds complexity and signalling overhead.

This Key Issue aims to study whether and how to enhance the user plane architecture and simplify procedure for user plane management.

# * * * * End of Changes * * * *