3GPP TSG-WG SA2 Meeting #170	S2-2507025

# Goteborg, Sweden, 25 – 29 August 2025		(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0034 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the AIoT data in the UDR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, Lenovo? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The LS from CT4 on Using Nudr-dr service for accessing AIoT device profile data (C4-252411/S2-2506128) requests for feedback and update for the following agreement: In CT4#129 meeting, CT4 has discussed this topic and agreed the following: 1) Nudr_DR service is proposed to be used for accessing AIoT device profile data 2) a new TS will be created for the data modelling of AIoT device profile data. As per the conclusion in clause 8.2.2 of TR 23.700-13, the AIoT device profile data can be stored in the ADM and possibly together in the UDR as the following. When the AIoT device profile data is managed by the 5GC, it is stored in the ADM (Ambient IoT Data Management), possibly together with UDR, which exclusively supports AIoT devices. Also, the AF authorization data can be stored in the UDR and managed by the ADM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §4.5.8 - Add function of the UDR with the storage of AF authorization data. §5.5 - Add description in case the AIoT device profile data is stored in the UDR, it is managed by the ADM using Nudr_DR service. §5.6 - Add description that the AF authorization data can be stored in the UDR. §7.X (new) - Add new clause for the description related to the Nudr_DataManagement (DM) Service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification how the AIoT device profile data and the AF authorization data stored in the UDR is managed by the ADM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.8, 5.5, 5.6, 7.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS 23.502 CR 5556 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.5.8	UDR

In addition to the functions defined in TS 23.501 [3], the UDR may support the following functions:

-	Storage of AIoT device profile data.

-	Storage of AF authorization data.

* * * * Start of 2nd Change * * * *

5.5	AIoT Device Profile Management

The ADM may hold operator’s subscription data for the AIoT Device used in the network. If the AIoT Device is managed by the network, then the profile data for an AIoT Device is required in the network, otherwise the corresponding profile data (e.g. AIoT Device Permanent ID or credentials) is stored external to the network.

The AIoT Device Permanent ID is used by the AIOTF together with local configuration, 3rd party related context to locate the entity which stores the profile data of an AIoT Device.

In case the AIoT Device is managed by the network, the AIOTF checks whether the AIoT Device Permanent ID from AIoT Device has the profile data in the network and retrieves the profile data. The profile data for AIoT Device is different with UE subscription data as defined in clause 5.2.3 of TS 23.502 [4], it is stored in the ADM network entity that exclusively supports management of AIoT Device’s profile data which may also be stored in the UDR and managed by the ADM using Nudr_DataManagement service as specified in clause 7.X.

The table 5.5-1 below describes information storage structures for AIoT device profile data.

Table 5.5-1: AIoT Device Profile Data

| Field | Description |
| --- | --- |
| AIoT Device Permanent ID | Uniquely identifies the AIoT Device. |
| Last known AIOTF information | Indicate the last known AIOTF that serves the AIoT device, or unknown |

NOTE:	Security materials and security mechanism involving ADM are specified in TS 33.369 [9].

* * * * Start of 3rd Change * * * *

5.6	AF authorization for the AIoT Services

The information needed to support the authorization of the AF for performing the AIoT service can be stored as the authorization data for 3rd party AF in the ADM, UDR or locally configured in the AIOTF.

Table 5.6-1 below describes items stored as AF authorization data for the AIoT.

Table 5.6-1: AF Authorization Data for AIoT

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed area | Indicate the allowed area for the indicated AF to perform the AIoT services operations. |
| Allowed service operations | Indicate the allowed service operation (s) for the indicated AF, e.g. inventory, read, write, permanent disable. |
| Allowed target AIoT Devices | Indicate the allowed AIoT Device(s) for the indicated AF. The information indicating the allowed target AIoT Devices is a list of the permanent AIoT Device ID (see clause 5.7) or the filtering information (see clause 5.8). |

The authorization of the AF for the AIoT includes two parts:

-	NEF performs AIoT AF request authorization based on the service level agreement (SLA) between the 3rd party AF and the 5GS of the mobile network operator, the operator policy and local configuration as in TS 33.501 [8].

-	AIOTF may perform authorization of AIoT service requested by the AF, using the AF authorization data retrieved from the ADM or configured locally as described in above Table 5.6-1. When ADM is used, the AIOTF also subscribes to changes of AF authorization data in the ADM for synchronization.

* * * * Start of 4th Change * * * *

7.X	UDR Services

7.X.1	Nudr_DataManagement (DM) Service

7.X.1.1	General

The ADM makes use of the Nudr_DM service for AIoT Data, as described in clause 5.2.12 of TS 23.502 [4].

The AIoT Data includes AIoT Device Profile Data and AF Authorization Data. The AIoT Device Permanent ID is the Data Key of AIoT Device Profile Data, while the AF ID is the Data Key of AF Authorization Data, as illustrated in Table 7.X.1.1-1.

Table 7.X.1.1-1: Data keys

| Data Set | Data Subset | Data Key | Data Sub Key |
| --- | --- | --- | --- |
| AIoT Data | AIoT Device Profile Data | AIoT Device Permanent ID | - |
|  | AF Authorization Data | AF ID | - |

* * * * End of Changes * * * *