SA WG2 Meeting #170	S2-2507043

25 – 29 Aug, 2025, Goteborg, SE

Source:	vivo

Title:	KI#2: proposal for agreed principles

Document for:	Agreement

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes to agree the principles for KI#2 for FS_EnergySys_Ph2.

1.	Introduction

Key issue#2 studies the following aspects:

-	Whether and how to support taking the energy related information (e.g. energy related characteristics, renewable energy information) into account to determine traffic handling for the UE (e.g. QoS adjustment).

-	Whether and how to support policy control based on interaction with EIF.

-	Identify charging impacts caused by service adjustment (in cooperation with SA WG5).

Sol#2-7, 13-18 propose the solutions for this Key issue. Following table proposes the summary of the solutions for KI#2.

Table 1: Brief summary of the solutions for Key issue#2

| Sol# | Brief summary of the solutions | Highlights of the solutions |
| --- | --- | --- |
| 2 | In this solution, the PCF collects slice level EnC from OAM and compare the collected EnC to the thresholds in the policy data. And then the PCF could apply/revoke the AM policy, SM policy and/or BDT policy by monitoring the EnC according the EnC policy per slice. | - No EIF interaction; - PCF gets from OAM: slice level EnC; - PCF makes the policy control. |
| 3 | This solution aims to extend the PDTQ policy negotiation to consider energy. The AF provides its adaptability/flexibility in terms of QoS level and transfer time; and then, based on the energy-related information from OAM, the PCF identifies the enhanced PDTQ policy, e.g., the energy-optimized candidate time slots for data transfer, the recommended QoS profile in each slot. Then, the PCF negotiates the information with AF. | - No EIF interaction; - PCF gets from OAM: energy consumption or renewable energy ratio in the node level; - PCF extend the PDTQ policy negotiation with AF. |
| 4 | This solution proposes to adjust the QoS request based on the energy information. In this solution, the SMF is aware of the energy status of NG-RAN (e.g., whether the NG-RAN is using renewable energy or not) and then decide whether to update Alternative QoS profile or not. Then, the SMF updates the value of QoS rules, QoS profile and Alternative QoS profile by re-using the existing parameters. No new parameters are added based on the energy related information. | - EIF interaction; - PCF generates the PCC rule due to network energy saving; - SMF updates QoS rules, QoS profile and AQP due to energy status of NG-RAN. |
| 5 | In this solution, the AF may indicate to the network the preferred or expected network energy saving behaviours for its applications or its UEs. So the network energy saving behaviours, e.g. QoS adjustment, are negotiated between the application providers and operators. Then the PCF makes the policy decisions taking into consideration the AF request and the energy related information. | - May need EIF interaction; - AF indicates preferred or expected network energy saving behaviours; - The PCF makes the policy decisions considering the AF request and the energy related information. |
| 6 | This solution proposes that the PCF takes the energy related information in the notifications from the EIF into account for policy control, e.g., AM policy, SM policy. | - EIF interaction; - PCF makes the policy control, e.g., AM policy, SM policy. |
| 7 | This solution proposes to use the energy saving QoS profiles at RAN due to energy saving. The PCF generates the Energy Saving QoS profile(s) and the priority list for the QoS flow based on AF's request and operator policy. Then, the PCF sends the energy saving QoS profiles and the priority list to the NG-RAN. The NG-RAN selects the Energy Saving QoS profile only if NG-RAN decides to downgrade the QoS parameters due to energy saving. | - No EIF interaction; - RAN receives the Energy saving QoS profile(s) and selects the Energy saving QoS profile(s) due to energy saving. |
| 13 | This solution proposes that the PCF obtains the energy consumption information for the UE and the network slice from the EIF and aggregates the energy consumption information for the UE and the network slice for the UEs using the network slice. And then, the PCF updates the AM policy (e.g., network slice replacement) for the UEs to reduce energy consumption on the network slice. | - EIF interaction; - PCF aggregates the EC information for the network slice for the UEs. - PCF updates the AM policy. |
| 14 | In this solution, the subscription information in UDM includes Renewable Energy Indicator. The SMF retrieves this information and sends it to the PCF. The PCF subscribes the renewable energy related information from EIF, e.g., whether the renewable energy is used, or the ratio of the renewable energy for the required granularities. The PCF may update the policy information based on the energy related information from EIF. | - EIF interaction; - PCF gets Renewable Energy Indicator; - PCF updates the policy based on the Renewable Energy Indicator and EIF notification. |
| 15 | In this solution, the UE energy saving services can be deployed by the operators. The user can subscribe to one of the available UE energy saving services via the service provider AF or alternatively by the operator. The PCF has energy saving policy per each available UE energy saving service. Based on certain criteria within the AF, the AF may request activation for the UE subscribed UE energy saving service. | - No EIF interaction; - Energy saving services subscription in UDR; - PCF makes the policy control based on the subscription. - AF interaction for energy saving service provisioning. |
| 16 | This solution proposes that the PCF takes the energy related information into account for URSP generation, e.g., evaluate and set the RSD priority for the slice according to the slice level EnC/EnE. The energy related information in this solution can be from the OAM. | - No EIF interaction; - PCF gets from OAM: slice level EnC/EnE; - PCF update the URSP. |
| 17 | This solution proposes to differentiate charging for the service adjustment due to network energy saving. In this solution, the PCF adds "reason for a policy change" in charging info of the associated PCC rule. And then the SMF sends the reason for policy decisions to the CHF. The CHF applies differentiated (reduced) charging to the traffic of the UE. | - No EIF interaction; - Charging aspects: CHF receives the reason for policy change from SMF to differentiate/reduce the charging to the traffic under network energy saving action. |
| 18 | This solution proposes to add a per DNN/S-NSSAI level Energy Saving indication in the UE Session Management Subscription data in the UDM. And then the SMF retrieves the Energy Saving indication from UDM and send it to PCF in the SM policy association for the policy control. | - EIF interaction; - DNN/S-NSSAI level Energy Saving indication in UE Session Management Subscription data in the UDM; - PCF makes the SM policy control based on the indication and EIF notification. |

According to the current solutions for KI#2, the following two options and analysis are proposed:

Option 1: with EIF interaction.

- The PCF could get EIF notification for the required granularities, e.g., UE level, flow level;

- The PCF gets the Energy Saving indication from the UE Session Management subscription in UDM;

- The PCF makes or updates the policy control, e.g., SM policy, UE policy, BDT policy, PDTQ policy;

Option 2: without EIF interaction.

Option 2.1: with OAM interaction:

- The PCF could get the slice level energy consumption information from OAM;

- The PCF gets the Energy Saving indication from the UE Session Management subscription in UDM;

- The PCF makes the policy control, e.g., AM policy, SM policy, UE policy, BDT policy, PDTQ policy;

Option 2.2: without OAM interaction:

- QoS profile for energy saving at NG-RAN.

For both the above two options, the AF could interact with the network to negotiate the preferred energy saving behaviours for the service data flow or the UEs, e.g., the QoS requirements for network energy saving, PDTQ policy adaptability.

Regards to option 1, the PCF could make the policy control taking into account the UE level energy information, e.g., energy consumption information, renewable energy information. There is the argument that the UE level energy information is not accurate for the usage at PCF for policy control. However, considering the fact that the PCF takes the information into account for policy adjustment for the UE, this UE level energy related information could be the trigger or the reference for the PCF.

Regards to option 2, the PCF gets the slice level energy consumption information from OAM. However, this information still has the accuracy issue as the option 1. Besides, the PCF adjust the policy for the UE based on the slice level information may not be very appropriate, because it is possible that several UEs consumes high percentage of energy of the whole slice, but other UEs which may consume very little energy will also suffer from the SM policy adjustment for this slice. So it is more suitable for the PCF to make the policy control for the UE based on both the slice level energy consumption information and per-UE energy related information.

Regards to option 2.2, the selection of QoS profile for energy saving at NG-RAN is based on the energy information or energy status at RAN node. So the NG-RAN could apply the QoS profile for network energy saving due to the internal conditions.

So based on the above the evaluation and analysis, this contribution has the following proposals for KI#2.

2. Text Proposal

It is proposed to add the following changes for KI#1 in TR 23.700-67.

# * * * * First change (all new Text) * * * *

7.1.2	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

The following principles are agreed for KI#2:

- The subscription for network energy saving indictor in the UDM is needed. Extending the existing energy saving indicator or a new indicator in the SM subscription information may be possible;

- The PCF may adjust or update the policy for the UE, e.g., SM policy, UE policy, PDTQ policy, based on the following information:

  - slice level energy consumption information from OAM; and/or the UE level energy consumption information or renewable energy information from EIF;

  - subscription information for network energy saving indicator from the UDM;

  - AF provided information for the preferred network energy saving behaviours, e.g. QoS requirements for network energy saving, PDTQ policy adaptability.

- The SMF provides the QoS profile(s) for network energy saving to the NG-RAN. The NG-RAN applies and matches the QoS profile due to the specific energy status at NG-RAN and notifies the core network.

# * * * * End of change * * * *