SA WG2 Meeting #170	S2-2507044

# Goteborg, Sweden, August 25 – August 29, 2025	(revision of)

Source: 	vivo

Title: 	Interim principles for KI3

Document for: 	Approval

Agenda Item: 	20.4.1

Work Item / Release:	FS_EnergySys_Ph2/ Rel-20

Abstract of the contribution: This contribution tries to provide the interim principles for KI3.

- Discussion

This paper is trying to evaluate the solutions captured for KI#3(5GS enhancements for network energy saving and efficiency), provide the conclusions for the KI#3 based on the captured solutions.

From the current solution organized, following 3 part can be understood:

- NF profile extension including the energy related information (Sol#20)

- NF selection based energy related requirement and information (Sol#19, Sol#20)

- UP path adjustment based on energy related information of DNAI (Sol#21,22, 23)

For the NF profile extension including the energy related information, EnergySaving /Not EnergySaving State and Schedule of EnergySaving/not EnergySaving can effectively reflect the energy status of the NF, for the “Energy Priority Information” or “Schedule of Energy Priority Information”, it is not clear on the differences to the exist “priority” attribute in the NF profile, maybe based on implementation, the OAM can directly consider some energy related influences in the exist “priority” attribute in the NF profile, so the new introduced attributes seems not necessary.

For the NF selection based energy related requirement and information, the intention of extend NF profile to include the energy related information is not only applied to UP plane (i.e., UPF), CP plane function (e.g., AMF) is also needed to be considered.

For the UP path adjustment based on energy related information of DNAI: UP path adjustment can effectively improve the energy efficiency and decrease the energy consumption, the AF, SMF or PCF can obtain the energy related information per DNAI or UPF from OAM (e.g., energy consumption information or renewable energy information) and adjust the UP path (UPF reselection) accordingly.

2	Proposal

It is proposed to capture the following solution in TR 23.700-67.

Start of Changes

7.1	Agreed Principles

7.1.Y	Agreed Principles for KI#3

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

- NF profile is extended to include the following Energy-related NF profile attributes in the table.

Table 7.1.Y-1: Energy-related NF profile attributes

| Information | Description |
| --- | --- |
| EnergySaving /Not EnergySaving State | Value of the energy state as specified in TS 28.310 [9], e.g. notEnergySaving state, Energy Saving state. (NOTE 1) |
| Schedule of EnergySaving/not EnergySaving | The schedule of the EnergySaving/not EnergySaving states, includes the value of EnergySaving or not EnergySaving state which applies at each time interval in the schedule. If this is present, the EnergySaving /Not EnergySaving State attribute is not present. |
| NOTE 1: EnergySaving/not EnergySaving refers to states of a network function which are defined in TS 28.310 [9]. |  |

- NF discovery and (re-)selection, including control plane and user plane, is enhanced to consider the energy related information from the NF profiles and/or discovery request from the NF consumer.

- UP path of PDU session may be adjusted based on the energy related information of a list of DNAI(s) or UPFs from the OAM.

7.2	Topics for further consideration

7.2.Z	Topics for further consideration for KI#Z

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#Z. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.Z) and can either be then marked as not pursued or postponed to a future Release.

End of Changes