3GPP TSG-WG SA2 Meeting #170	S2-2507049

# Goteborg, Sweden, 25 – 29 August 2025		(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0038 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reference to the format of AIoT parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | An Editor’s note remains for the reference to the stage 3 specification as the following. Editor's note: The reference in NOTE 3 needs to be updated, when the appropriate stage 3 document is identified. As per the Reply LS on AIoT device identifier length (C4-252465/S2-2506129), CT4 has taken the requirements and attached the agreed CR on the format of the parameters used for Ambient IoT. This CR proposes to add references to the clauses in TS 23.003 for the format of the AIoT Device Permanent Identifier and the Filtering Information. Also, the general clause in clause 5.7 is deemed not needed, thereby suggest to be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §5.7.1 - Removed general clause and clause numbers corrected accordingly. §5.7.2 - Correct the clause number to §5.7.1. - Resolved the remaining Editor’s note by adding reference to clause in TS 23.003 for the format of the AIoT Device Permanent Identifier. §5.8 - Added reference to clause in TS 23.003 for the format and the length of the Filtering Information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification with remaining Editor’s note. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.1, 5.7.2, 5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.7	Identifiers

5.7.1	AIoT Device Permanent Identifier

In order to support the AIoT feature in 5G System, a globally unique AIoT Device Permanent Identifier shall be allocated to each AIoT Device. An AIoT Device Permanent Identifier is assigned either by an operator or by a third party. The AIoT Device Permanent Identifier is used to identify an AIoT Device and locate the entity where the AIoT Device related information is stored.

NOTE 1:	How to configure an AIoT Device with the AIoT Device Permanent Identifier is out of scope of this specification.

The AIoT Device Permanent Identifier includes the following components:

-	The ID Type, including:

-	Information indicating whether a PLMN ID is included.

-	Information indicating whether a NID is included.

-	Information indicating whether a third party identifier is included.

-	Identification Information Type, indicating whether the Identification Information contains an  EPC or unstructured information.

-	The Domain Information includes none, one or more of the following:

-	A PLMN Identifier (i.e., MCC and MNC) as specified in TS 23.003 [6] when the information in the ID type indicates it is included

-	A Network Identifier (NID) as specified in TS 23.003 [6] when the information in the ID type indicates it is included.

-	A third party identifier used to identify a third party when the information in the ID type indicates it is included.

-	The Identification Information is used to distinguish different AIoT Devices within the scope identified by Domain Information (if available) and can contain either:

-	An EPC, as defined in clause 14 of GS1 TDS Release 2.1 [7].

-	Unstructured information, where the contents is defined by the allocator.

Figure 5.7.1-1: AIoT Device Permanent Identifier Structure

An operator allocated AIoT Device Permanent Identifier should include the identifier of the network for the operator. The identifier of the network is present as either a PLMN Identifier, NID or both in the AIoT Device Permanent Identifier.

A third party allocated AIoT Device Permanent Identifier may include none of the following information or include any combination of at least one kind of the following information: a PLMN Identifier, NID or the third party identifier.

NOTE 3:	The length of ID Type, PLMN Identifier (if present), NID (if present) and the third party identifier (if present) components is fixed. The length of the Identification Information is variable. The details are specified in clause 31.2 of TS 23.003 [6].

NOTE 4:	When the Domain Information is empty, the AIOTF uses, e.g., Identification Information (i.e EPC) to discover and select the ADM instance or the external server for the AIoT Device Profile Data.

The following lengths are supported for the Identification Information in an AIoT Device Permanent Identifier: 96 bits, and 128 bits.

NOTE 5:	The encoding for the length of the Identification Information enables additional shorter or longer fixed lengths to be supported in the future.

* * * * Start of 2nd Change * * * *

5.8	Filtering Information

The filtering information is used to identify or filter multiple AIoT Devices and is constructed by one or multiple components(i.e. ID Type, PLMN Identifier, NID, third party identifier and Identification Information) of the AIoT Device Permanent Identifier as defined in clause 5.7.

The filtering information includes a list of filtering elements. A filtering element includes:

-	Information indicating a component of the AIoT Device Permanent Identifier that is used to match the bitstring.

-	A bitstring which is used to compare with the component.

-	An Offset from the beginning of the indicated component, which indicates the start location in the indicated component to be used to compare with the corresponding bitstring.

When the indicated component is PLMN ID, NID or Third Party Identifier, the Offset is not included in the filtering information, as the whole component is compared with the bitstring, otherwise the Offset is always included.

There may be multiple filtering element within the Filtering Information for the Identification Information.

Each bitstring is corresponding to one component of the AIoT Device Permanent Identifier.

NOTE 1:	It is assumed that the AIOTF includes at most one filtering element information within the Filtering Information for ID Type, PLMN ID, NID or Third Party Identifier. It is assumed that the AIoT Device does not validate the number of filtering element(s) for any indicated component within the Filtering Information.

NOTE 2:	The maximum number of the filtering element for Identification Information is left to stage3. The format and the length of the Filtering Information is specified in clause 31.3 of TS 23.003 [6].

Editor's note:	Whether and how to secure the filtering information is up to SA WG3.

Figure 5.8-1: The example of the comparing the bitstring to the component

To determine whether an AIoT Device Permanent Identifier matches the filtering information, it is compared with every filtering element information within filtering information by comparing the whole bitstring within a filtering element information with the indicated component of its AIoT Device Permanent Identifier. If all the compared bitstrings match the AIoT Device Permanent Identifier then an AIoT Device Permanent Identifier matches the filtering information. If an AIoT Device Permanent Identifier does not contain an indicated component then it does not match the filtering information.

* * * * End of Changes * * * *