SA WG2 Meeting #170	S2-2507055

25 – 29 Aug, 2025, Goteborg , SE

Source:	Nokia, AT&T, T-Mobile US

Title:	[WT#1.1, all topics] Work task #1.1 Support of 6G Control plane signaling

Document for:	Approval

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: Scope of Work task 1.1.

# 1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

Work task 1.1 scope as agreed in 6G System Study item S2-2506096:

.1.	Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

Architectural Motivation: Challenges with 5G System and 5G NAS:

- Orthogonality: Modularity principles supported but no strict separation yet between NAS functionalities (e.g. NAS MM and AMF still support session management functionality like reject/release PDU sessions).

- Upgrade: Upgrade of a new feature in any NAS layer may require an upgrade of basic MM functionality in the device / network.

- Devices: Monolithic NAS stack designed for eMBB devices as the baseline. Capability negotiation is introduced for UE(s) to indicate the capabilities at a fine granular level.

- Security: Single NAS security for all NAS functionalities, using a single NAS security mechanism (key, algorithm, ....)  for all NAS containers.

Proposal for 6G:

- Orthogonality: NAS in 6G should enable a strict separation of different functionalities (aka NAS modules), e.g. between NAS Mobility Management (MM) and Session Management (SM) layers. This avoids the need for overlapping features in AMF and SMF as in 5G.

- Upgrade: Ability to upgrade, including remote upgrade, of the device with changed or new NAS functionality/modules, orthogonal NAS modules should be independent of each other – both in the UE and in the network. From the network perspective, when a new NAS module is introduced, only routing in the AMF is impacted to forward the NAS message to the correct NF terminating the respective NAS module.

- Devices: Different devices - Smartphones, LPWA, Public Safety, Ambient IoT, Sensing, FWA - can support different combinations of NAS modules, allowing easier introduction of new features and device testing. Device types could be defined to support a minimum set of mandatory NAS modules.

- Security: Enables differentiated security levels for different NAS associations that are terminated at different NFs, while still supporting a unique authentication for a given UE. Different modules can use independent security keys, which strengthens the concept of orthogonality.

**** First Change ****

# Annex A.X. WT#1.1

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#1.1 Study the support for control plane signalling for 6G for connectivity services and/or beyond connectivity services, including the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of independent non-access stratum functionalities depending on the type of typical user equipment (e.g. IoT vs eMBB).

c.	Whether and how to develop generic mechanisms (e.g. authorization, discovery) for UE-Core Network interaction to support operator services.

**** Second Change ****

# 5.X. Key Issue #X: Support of 6G Control plane signaling

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

1.1 Study the support for control plane signalling for 6G for connectivity services and/or beyond connectivity services, including the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of independent non-access stratum functionalities depending on the type of typical user equipment (e.g. IoT vs eMBB).

c.	Whether and how to develop generic mechanisms (e.g. authorization, discovery) for UE-Core Network interaction to support operator services.

**** End of Changes ****