SA WG2 Meeting #170	S2-2507076

Goteborg, Sweden, 25 – 29 August 2025	(revision of S2-25xxxxx)

Source:	CATT

Title:	New solution for KI#2: Call setup without precondition and SCC AS acting as B2BUA

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: The contribution proposes a new solution to KI#2: IMS enhancement for GEO NB-IoT NTN access.

Discussion

None.

Proposal

It is proposed to add this solution for KI#2 in the TR 23.700-19.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[7]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.101: "Service aspects; Service principles".

[10]	3GPP TS 29.211: "Rx Interface and Rx/Gx signalling flows".

[11]	3GPP TS 29.213: "Policy and charging control signalling flows and Quality of Service (QoS) parameter mapping".

[12]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[13]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[14]	3GPP TS 24.294: "IP Multimedia Subsystem (IMS) Centralized Services (ICS) protocol via I1 interface".

[15]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services; Stage 2".

[16]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[17]	3GPP TS 23.203: " Policy and charging control architecture".

[18]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[19]	IETF RFC 4566: "SDP: Session Description Protocol".

[20]	IETF RFC 5049: "Applying Signaling Compression (SigComp) to the Session Initiation Protocol (SIP)".

[x]	IETF RFC 3312: " Integration of Resource Management and Session Initiation Protocol (SIP)".

* * * Next Change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| #1 | X | X |  |  |
| #2 | X |  |  |  |
| #3 | X |  |  |  |
| #4 | X | X | X | X |
| #5 | X |  |  |  |
| #6 | X |  |  |  |
| #7 | X | X |  |  |
| #8 | X |  |  |  |
| #9 | X |  |  |  |
| #10 | X |  |  |  |
| #11 | X |  |  |  |
| #12 |  | X |  |  |
| #X |  | X |  |  |

* * * Next Change * * * *

6.X	Solution #X: Call setup without precondition and SCC AS acting as B2BUA

6.X.0	High-level solution Principles

The solution addresses KI#2: IMS enhancement for GEO NB-IoT NTN access.

It proposes to skip the precondition negotiation to reduce the call setup time, and SCC AS acts as B2BUA for precondition negotiation if MO UE and MT UE have different precondition requirements.

6.X.1	Description

Precondition mechanism specified in RFC 3312 [x] is used to guarantee dedicated resources are successfully allocated for both sides of the call, which is especially important in the mobile network, for example, to avoid the "ghost ring".

Figure 6.x.1-1 shows the call setup procedure with precondition. If the procedure is used for GEO satellite access scenario, the call setup time could reach about 100s as per Table A.2-1 in Annex A, and such delay is unacceptable for the end users.

Figure 6.x.1-1: Call setup with precondition

Then a method to reduce call setup time is not to use precondition negotiation, as shown in Figure 6.x.1-2. This procedure is normally taken in fixed IMS network, where resource allocation is not needed.

Figure 6.x.1-2: Call setup without precondition

This contribution proposes to not use precondition for call setup when GEO satellite access is involved, while the IMS network still needs to acquire the status of resource allocation during the call setup to avoid the ghost ring.

On the other hand, the IMS network needs to be capable of acting as "proxy" to handle precondition negotiation as the MO UE and MT UE may have different requirements for precondition.

The SCC AS serving MT UE is proposed to act as this kind of proxy (i.e., B2BUA) for the following reasons:

-	SCC AS is mandatory node in the mobile networks and at least for terminating domain selection.

-	SCC AS can acquire MT UE's access type, which can be used to determine whether precondition can be skipped for the MT UE.

-	Compared with some solutions where P-CSCF or IMS AS serving MO UE acts as B2BUA, this solution can avoid unnecessary signalling interaction. For example, when P-CSCF serving the GEO UE acts as B2BUA, the P-CSCF may have to always add or remove the precondition indication because it has no knowledge of the access type of the other side UE (and it is unnecessary to exchange the access types of the MO UE and MT UE between the originating and terminating network).

-	The SCC AS solution is more easy to be implemented compared with P-CSCF solution, since as a centralized node, the number of SCC AS is much fewer than that of the P-CSCF.

6.X.2	Procedures

This clause describes the detailed procedures for the following cases:

-	Both MO UE and MT UE are accessing network via GEO satellite.

-	Only MO UE is accessing network via GEO satellite.

-	Only MT UE is accessing network via GEO satellite.

The MO UE and MT UE may be served by one IMS system or different IMS systems. For the latter case, there may be gateway functions (e.g., IBCF) to handle the proxy agent first.

6.x.2.1	Both MO UE and MT UE are accessing network via GEO satellite

The Figure 6.x.2.1-1 shows the call setup procedures when both MO UE and MT UE are accessing network via GEO satellite. The P-CSCF1/S-CSCF1/SCC AS1 are the serving nodes of MO UE, and P-CSCF2/S-CSCF2/SCC AS2 are the serving nodes of MT UE.

Figure 6.x.2.1-1: Both MO and MT UE access the network via GEO satellite

The UE1 and UE2 have registered in the IMS network. The UEs support the voice over GEO satellite and will skip the precondition negotiation in the call setup.

1. UE1 initiates the call without the requirement of precondition. The codec for GEO satellite is included in the SDP.

2. The serving P-CSCF1 of UE1 triggers the dedicated resource allocation.

3. The P-CSCF1 forwards the SIP Invite to terminating side via S-CSCF1, and P-CSCF1 updates the codecs in the SDP by adding the codecs supported by the network.

4. The S-CSCF2 sends the SIP Invite to the SCC AS2 based on the IFC trigger, and SCC AS2 determines whether the precondition is required, for example, by the GEO access type of UE2.

5. Since the UE2 is accessing the network via GEO satellite, SCC AS2 can determine precondition is not required by UE2, then SCC AS2 doesn't change the received precondition information and sends the request to P-CSCF2 serving the UE2 via S-CSCF2.

6. The P-CSCF2 may update the codec list according to its own configuration, while the codec for GEO satellite is always included.

7. UE2 replies with 180 Ringing.

8. The P-CSCF2 triggers the dedicated resource allocation.

9. After receiving the 180 Ringing, SCC AS2 holds the message and waits for the completion of resource allocation of UEs.

10. UE1 sends the SIP INFO to the network indicating that its dedicated resource has been allocated. SDP is not needed in the SIP INFO message.The SCC AS2 terminates such message.

11. Similarly, UE2 sends the SIP INFO to SCC AS2 when its dedicated resource has been allocated.

12. SCC AS2 confirms that resource allocation has been successful and forwards the SIP 180 Ringing. Then Call setup continues.

6.x.2.2	Only MO UE is accessing network via GEO satellite

The Figure 6.x.2.2-1 shows the call setup procedures when only MO UE is accessing network via GEO satellite.

Figure 6.x.2.2-1: Only MO accesses the network via GEO satellite

The UE1 and UE2 have registered in the IMS network. The UE1 support the voice over GEO satellite and will skip the precondition negotiation in the call setup. While the UE2 is not accessing via GEO satellite and it requires precondition. SCC AS2 acts as B2BUA for precondition negotiation.

1-4. Same as the steps 1-4 in clause 6.x.2.1.

5. Since the UE2 is not accessing the network via GEO satellite, SCC AS2 can determine that precondition is required by UE2, then SCC AS2 acts as B2BUA and updates the SIP header indicating precondition is required, then sends the request to P-CSCF2.

6. The P-CSCF2 may update the codec list according to its own configuration, and the codec for GEO satellite may be included.

7. UE2 replies with 183 Session Progress.

8. The P-CSCF2 triggers the dedicated resource allocation.

9-10. SCC AS2 does the precondition negotiation with UE2 and waits for the resource allocation status from MO side.

11. UE1 sends the SIP INFO to the network indicating that its dedicated resource has been allocated.

12-14. SCC AS2 confirms that resource allocation for UE1 has been successful and continues the call setup procedure.

6.x.2.3	Only MT UE is accessing network via GEO satellite

The Figure 6.x.2.3-1 shows the call setup procedures when only MT UE is accessing network via GEO satellite.

Figure 6.x.2.3-1: Only MT UE accesses the network via GEO satellite

The UE1 and UE2 have registered in the IMS network. The UE2 support the voice over GEO satellite and will skip the precondition negotiation in the call setup. While the UE1 is not accessing via GEO satellite and it requires precondition. SCC AS2 acts as B2BUA for precondition negotiation.

1. UE1 initiates the call with the requirement of precondition, and the SIP Invite is transferred to terminating side.

2. The S-CSCF2 serving the UE2 sends the SIP Invite to the SCC AS2 based on the IFC trigger, and SCC AS2 determines whether the precondition is required, for example, by the GEO access type of UE2.

3. Since the UE2 is accessing the network via GEO satellite, SCC AS2 can determine precondition is not required by UE2, then SCC AS2 acts as B2BUA and updates the SIP header indicating precondition is not required, then sends the request to P-CSCF2.

4. The P-CSCF2 updates the codec list according to its own configuration, e.g., adding the codec for GEO satellite access.

5. UE2 replies with 180 Ringing.

6. The P-CSCF2 triggers the dedicated resource allocation.

7-10. After receiving the 180 Ringing, SCC AS2 does the precondition negotiation with UE1 and waits for the resource allocation status update from MT side.

11. UE2 sends the SIP INFO to the network indicating that its dedicated resource has been allocated.

12-13. SCC AS2 confirms that resource allocation for UE2 has been successful and continues the call setup procedure.

6.X.3	Impacts on Services, Entities and Interfaces

SCC AS:

-	Determine whether precondition can be skipped for MT UE.

-	Support B2BUA for precondition negotiation.

UE:

-	Support skipping precondition negotiation and reporting resource allocation successful status to network.

* * * End of Changes * * * *