SA WG2 Meeting #170	S2-2507079

25 – 29 Aug, 2025, Goteborg, SE

Source:	CATT

Title:	23.700-19: Discussion and Interim Agreement for KI#1

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This paper discusses the CP/UP and IP/Non-IP of KI#1, and proposes Interim Agreements for KI#1.

1 Background

  - SA1 Requirement of IMS voice using GEO satellite access

The requirements of IMS voice using GEO satellite access defined in SA1 TS22.261 as follows:

-	The 5G system with GEO satellite access shall be able to support IMS voice communication as defined in TS 22.228 [55].

-	The 5G system with GEO satellite access shall be able to provide mechanisms to optimize IMS voice (e.g., call setup, transmission overhead) and support a codec for the transfer of the voice considering the transmission data rate, latency and packet size.

The performance requirements of IMS voice using GEO satellite access defined in SA1 TS22.261 as follows:

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity(DL) | Area traffic capacity(UL) | Overall user density | Activity factor | UE speed | UE type | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IMS voice call using GEO (note 7) | [1-3] kbit/s | [1-3] kbit/s | N/A | N/A | N/A | N/A | TBD | Handheld | Call set up time (note 8) ≤30 s (note 9) |
| Note 7: All the values defined for IMS voice call using GEO satellite access are generic for both regular and emergency calls. Note 8: call set up time refers to [56]; Note 9: 30s is the upper bound that is derived based on the user’s patience suggestions (30s) in [57]; |  |  |  |  |  |  |  |  |  |

1.2 Summary of the solutions on KI#1

| Sol# | CP/UP | Bearer (single/two bearers) & related optimizations | IMS voice data over IP/Non-IP | IMS signalling over IP/Non-IP |
| --- | --- | --- | --- | --- |
| #1 | UP | - default EPS bearer for both IMS signalling and voice data; - non-GBR QoS update / resource reservation is skipped / pre-initiated | IP | IP |
| #2 | UP | - default EPS bearer for IMS signaling; - a dedicated EPS bearer for IMS voice; - established together / pre-establish; | IP - PDCP(ROHC) | IP |
| #3 | UP | - default EPS bearer for IMS signaling; - a dedicated EPS bearer for IMS voice; - new Packet filter component type identifier, e.g. "(IMS) voice traffic type" | Non-IP; - PGW translate between non-IP and IP/UDP/RTP | IP |
| #4 | UP | - two default DRBs, one for signalling and one for voice media; - assign an existing or newly defined QCI / configure separate APNs; | Non-IP; - PtP tunnel based on UDP/IP are pre-configured on the P-GW | Non-IP; - security association is FFS |
| #5 | CP | - new SRBx / dedicated EPS bearer for IMS voice; - a separate SRB for the IMS signalling is FFS; - dedicated EPS bearer is non-GBR and possibly needs a new QCI; - associated EPS bearer identities for the corresponding SRB(s); - UE can locally derive the UL TFT based on the SDP information; | Option 1:IP Option 2:Non-IP Option 3:Non-IP In case of non-IP: - UE support TFTs in Session Management request - how the PGW performs the IP and non-IP conversion is FFS | Option 1:IP Option 2:Non-IP Option 3: IP |
| #6 | CP | - SRBx / default EPS bearer for IMS signalling; - SRBy / a dedicated EPS bearer for IMS voice; - Whether and how to provide QoS to the IMS voice service via SRB is FFS - pre-establish; new QCI is FFS - the UL TFT can be updated by the UE locally based on the IMS SDP negotiation result | IP - NAS(ROHC) | IP |
| #7 | CP | - default EPS bearer for both IMS signalling and voice data - an SRB is used for IMS signalling and can be switched to data delivery via DRB and S1-U bearer for voice data transmission - non-GBR QoS update / resource reservation is skipped / pre-initiated - It is FFS whether or how to differentiate QoS/priority further between IMS signalling and voice data. | IP | IP |
| #8 | CP+UP | - CP for IMS signalling / default bearer - UP for voice data / dedicated bearer - SGW needs to simultaneously maintain corresponding S11-U path and S1-U path | IP | IP |
| #9 | CP+UP | - UP for IMS signalling and voice when the UE is in ECM-CONNECTED - dedicated GBR bearer for voice; - IMS SIP INVITE data transport in CP | IP | IP |
| #10 | CP+UP | - a pair of PDN connections - two PDN connections, one without DRB for SIP, the other with DRB for voice media. - coexistence of S11-U and S1-U | Non-IP | Non-IP |
| #11 | CP+UP | - used together with sol#10 - P-GW is assumed to know the IP address of P-CSCF by pre-configuration. - UE: Extracting an IMS AGW IP address from SDP and informing it to MME - The current description mentions IP header only. FFS whether to consider UDP/TCP header addition/removal as well. | Non-IP； - It is FFS how the UE generates TCP/UDP payload without IP header - PGW performs the IP and non-IP conversion | Non-IP； - PGW performs the IP and non-IP conversion |

2 Discussion

2.1 CP vs UP Bearer for IMS Voice Data

| Analysis Aspects |  | CP Solution | UP Solution | Comparison |
| --- | --- | --- | --- | --- |
| Protocol Stack |  | 4 layers: MAC+RLC+RRC+NAS | 3 layers: MAC+RLC+PDCP | CP has one additional layers (NAS) |
| Protocol Overhead | Total | 13B (NOTE 1) | 4B | CP take much more protocol overhead than UP |
|  | PDCP | - NA | 1B |  |
|  | NAS | 11B->7B(CT WG1 work in Rel-19 reduced NAS 7B)->1B | - NA |  |
|  | RRC | 2B->1B->0B | - NA |  |
|  | RLC | 2B(AM)->1B(UM) or 0B(TM) | 1B(UM) |  |
|  | MAC | 2B->1B(Single MAC PDU per TBS) | 2B |  |
| PHY rate (ptime= 80ms, and with condition of ROHC(3B) account in) (NOTE 2) | Codec Rate = 0.6kbps Voice Data = 6B | 2.2 kbps | 1.3 kbps | UP requires lesser physical rate to transfer voice data for the same codec rate. |
|  | Codec Rate = 1.2 kbps Voice Data = 12B | 2.8 kbps | 1.9 kbps |  |
|  | Codec Rate = 2.4kbps Voice Data = 24B | 4.0 kbps | 3.1 kbps |  |
| QoS |  | CP is typically handled with a single, high-priority scheduling class intended for NAS PDUs/small data. Supporting QoS-differentiated, CP voice requires new SRB types and potential new QoS mechanism; | UP is used for QCI-based traffic handling, and suitable for continuous media streams. | Additional CP function needs to be defined to support QoS differentiation |
| NOTE 1: FFS is how to reduce total CP protocol overhead to 2B (without considering protocol layer of IP/UDP/RTP) for the best case, referencing solution #5 in TR 23700-19. NOTE 2: The protocol overhead will impact the requirements for target codec rate also required physical rate. Reference to discussion paper S2-2505534 (Protocol Overhead for IMS voice transmission). |  |  |  |  |

UP solutions have lower protocol overhead compared to CP solutions. Potential CP optimizations are challenging to implement - NAS reduction efforts yielded limited results in Rel-19, and optimizing CP requires extensive multi-WG coordination while UP is already well-supported.

UP inherently supports QoS differentiation through standard EPS mechanisms including default and dedicated bearers with QCI-based traffic handling. CP faces fundamental QoS limitations as it relies on SRBs designed for small NAS messages rather than continuous media streams.

As analyzed above for voice packets, the better choice is UP. The following principle is proposed:

Principle 1: Voice packets are transferred over GEO via the User Plane.

2.2 IP vs non-IP Comparison for Voice Media Transfer

| Analysis Aspects |  | IP with RoHC | Non-IP(RTP only) | Comparison |
| --- | --- | --- | --- | --- |
| Protocol Stack |  | IP/UDP/RTP + RoHC | RTP (Non-IP transport) |  |
| Protocol Overhead |  | SO (Steady State): 3 B FO (Recovery State): 8 B IR/IR-DYN (Init State): 15-40 B (NOTE 1) | 1 B (SN only) ~12 B (typical two-way voice call) (NOTE 2) | RoHC steady state competitive with Non-IP realistic scenarios. Non-IP advantage exists but is limited to basic functions. |
| RTP Functionality Support | Multi-call Support | Support, ROHC header no change | Requires SSRC (+4 B) | Supporting RTP functionality may introduce additional protocol header overhead. |
|  | Forward Error Correction | Support, ROHC header no change | Requires SSRC (+4 B) |  |
|  | Jitter Buffer Management | Support, ROHC header no change | Requires TS (+4 B) |  |
| Performance Efficiency | 20ms P-time (6B payload) | 3B → 3B/(6B+3B) = 33.3% | 1B → 1B/(6B+1B) = 14.3% to 5B → 5B/(6B+5B) = 45.5% | RoHC becomes highly efficient at operational p-time, while non-IP advantage reduced. |
|  | 80ms P-time (24B payload) | 3B → 3B/(24B+3B) = 11.1% | 1B → 1B/(24B+1B) = 4.0% to 5B → 5B/(24B+5B) = 17.2% |  |
| IMS Architecture Compatibility |  | Native IP PDN | Requires Non-IP to IP conversion | IP seamlessly integrates with existing IMS core. |
| NOTE 1: ROHC issues may be resolved by some optimization methods, reference to S2-2507021 (discussion paper) NOTE 2: RTP protocol is used to transmit real-time voice defined in RFC 3550, and the first twelve octets are present in every RTP packet as mentioned. Any RTP reduction relies on SA4 work. |  |  |  |  |

At GEO-appropriate 80ms p-time, IP with RoHC achieves 11.1% overhead while maintaining full IMS functionality, compared to Non-IP's 4.0-17.2% range with significant feature limitations.

Non-IP's theoretical minimum (1B) may only apply to basic voice without essential IMS features. According to SA1, the UE type is handheld, and there are no special service limitations such as supporting only one call per UE. To support multiple calls for a UE (e.g., call waiting/call hold and conference calls), it may require the SSRC field of the RTP header, which significantly increases the non-IP protocol overhead to 5B.

Since IMS PDN is inherently an IP-type PDN Connection, non-IP solutions require PGW enhancements for Non-IP to IP conversion with address/port management, which can be implemented by reusing the P2P tunnel or designed similar to solution #3 in TR 23700-19.

IP with RoHC can operate in steady state with 3B overhead, and any remaining issues can be resolved through further optimizations, providing more benefits compared to non-IP solutions that rely on RTP reduction with functional clipping.

Based on the above analysis, IP with RoHC is the preferred choice. The following principle is proposed:

Principle 2: IP-based voice media is transferred with RoHC over GEO.

NOTE 1:	RoHC enhancement and other optimizations can be used for IP/UDP/RTP header.

2.3 IP vs non-IP Comparison for Voice Signalling Transfer

| Analysis Aspects |  | IP with RoHC | Non-IP (SIP only) | Comparison |
| --- | --- | --- | --- | --- |
| Protocol Stack |  | IP/(UDP or TCP) + RoHC + SIP | SIP (Non-IP transport) |  |
| Protocol Overhead |  | SO (Steady State): 3 B FO (Recovery State): 8 B IR/IR-DYN (Init State): 15-40 B | 0 B |  |
| Voice SIP Message Analysis (NOTE 1) | SIP INVITE (simplified voice) | 3B + 600B payload = 603B | 600B payload = 600B | Marginal Non-IP advantage diminished by large SIP payload |
|  | SIP OK (200, voice only) | 3B + 400B payload = 403B | 400B payload = 400B |  |
|  | SIP ACK (confirmation) | 3B + 180B payload = 183B | 180B payload = 180B |  |
| Security |  | IPsec ESP support | No standardized security | TS 33.203 mandates IPsec ESP for SIP signalling integrity protection between UE and P-CSCF. Non-IP solutions lack this mandatory security framework. |
| IMS Architecture Compatibility |  | Native IP PDN | Requires Non-IP to IP conversion | IP seamlessly integrates with existing IMS core. |
| NOTE 1: For GEO voice call, assume the SIP messages are simplified, therefore, propose 600B, 400B, 180B for example. |  |  |  |  |

Even with simplified GEO voice SIP messages, the efficiency gain from non-IP solutions is likely negligible.

According to TS 33.203 for IMS security, IPsec ESP shall provide integrity protection of SIP signalling between the UE and the P-CSCF, protecting all SIP signalling messages at the IP level. In contrast, Non-IP solutions lack any standardized security mechanisms between the UE and the P-CSCF, and require further work in SA3.

NOTE 1:	I1 protocol is a transport independent protocol, so both IP and non-IP can support it, and the security association between UE and SCC AS may be established with IP more conveniently.

Based on the above analysis, IP is the better choice. The following principle is proposed:

Principle 3: IP-based signalling is transferred over GEO, with RoHC applied if needed.

NOTE 2:	RoHC enhancement and other optimizations can be used for IP/UDP/TCP header.

3 Proposal

Based on the above observations, it is proposed to agree the interim agreement as follows:

* First change (all New)*

7.Y	Agreed principles for KI#1 of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC

To supports KI#1 of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC, the following principles are proposed:

Principle 1: Voice packets are transferred over GEO via the User Plane.

Principle 2: IP-based voice media is transferred with RoHC over GEO.

NOTE 1:	RoHC enhancement and other optimizations can be used for IP/UDP/RTP header.

Principle 3: IP-based signalling is transferred over GEO, with RoHC applied if needed.

NOTE 2:	RoHC enhancement and other optimizations can be used for IP/UDP/TCP header.

* End of change (all New)*