3GPP TSG-WG SA2 Meeting #170	S2-2507081

Goteborg, Sweden, Aug 25th – 29th, 2025

Source:	NTT DOCOMO, Nokia, Huawei, SK Telecom, Deutsche Telekom, China Telecom

Title:	Discussion on NR to LTE mobility restriction

Document for:	Discussion

Agenda Item:	30.1

Work Item / Release:	TEI20_NEMO / Rel-20

Abstract: Discussion on Enhancement of NR to LTE mobility restriction.

# 1. Introduction

TS 23.501 specifies the high-level principle of Mobility Restriction in clause 5.3.4.1.1. As per this clause, the Mobility Restriction Function refers to the forbiddenAreas defined in AccessAndMobilitySubscriptionData in TS 29.503. However, this refers to only List of forbidden areas in 5GS, which does not allow forbidden areas in EPS to be set.

This discussion paper explains the need for also including forbidden areas in EPS to restrict mobility from NR to specific EPS TACs in RRC_CONNECTED state based on user subscription information. Furthermore, the paper discusses the possible enhancements foreseen to achieve this which allows configuration of forbidden EPS TAC while imposing no UE impact.

# 2. Discussion

## 2.1.	Problem

A high-level principle for Mobility Restriction function is specified in TS 23.501. In RRC_IDLE/INACTIVE state, this function is executed by UE, while in the RRC_CONNECTED, it is executed by both the RAN and CN. From operator's perspective, this function is useful in some cases.

The figure shows an example scenario. In some deployments, there is an area in which limited users can access the RAN, based on their subscription information. This area is configured as forbidden area to which UE is restricted to access. For instance, such areas include military zones or enterprise specific area.

In Case A in the figure, the forbidden area is neighbouring to the non-forbidden area, and both are covered by LTE cells. This situation may occur in early phase of migration.

In Case B in the figure, the layout of the forbidden area and the non-forbidden area is the same as in Case A, but the NR SA area has been deployed furthermore, and there is a place where NR SA area and LTE area are adjacent to each other.

In Case C in the figure, the forbidden area situation is the same, and NR SA has been fully deployed, and all areas are covered by NR SA cells.

The handover from the non-forbidden area to the forbidden area is avoided by mobility restriction function provided by EPC and eNB in Case A and 5GC and gNB in Case C, respectively. However, in Case B, the handover to the forbidden area cannot be avoided because 5GC cannot configure gNB for the mobility restriction to LTE cells based on existing specification.

Case D in the figure is another example of a scenario, where most of the area is covered by NR SA cells for non-forbidden area. Some new LTE cells have been deployed (for example in coastal areas for NTN including HAPS). Some of these LTE cells are forbidden areas and they neighbour the NR cells. Even in this scenario, the handover to the forbidden area cannot be avoided because 5GC cannot configure gNB for the mobility restriction to LTE cells based on existing specification.

Figure: Example scenario for forbidden areas.

From the operator's point of view, there is a strong demand for the function which allows to restrict mobility from NR to specific EPS TACs in RRC_CONNECTED state based on user subscription information, in order to avoid UEs from accessing restricted areas.

There is currently no issue in terms of the UE behaviour. The UE behaviour in 5GS regarding mobility restriction is specified in TS 24.501, where the forbidden area can include only 3 octet NR TAC. On the other hand, since the UE can simultaneously register with the EPS and the information related to mobility restriction to EPS TAC can be received directly from EPC, mobility toward specific EPS TAC can be restricted even if 5GC cannot configure 2 octet EPS TAC as a forbidden area.

However, the gNB cannot behave appropriately. The gNB behaviour related to the mobility restriction is based on the information exchanged in the Mobility Restriction List IE in the NGAP, but gNB cannot receive mobility restriction for EPS TAC from the Core Network.

Currently, the AMF cannot obtain subscription information for a particular UE from UDM and obtain mobility restrictions with EPS TACs. In TS 29.503 clause. 6.1.6.2.4, which specifies the forbiddenAreas exchanged via SBI in 5GC, describes this IE as "List of forbidden areas in 5GS" only and there is no mention of EPS.

Also, in the specifications in RAN, In TS 38.413, Mobility Restriction List IE is specified, and this IE can be included in some messages (e.g. HANDOVER REQUEST) for the UE from AMF to gNB to indicate restriction information. This IE includes Forbidden TACs within Forbidden Area Information to which the UE is prohibited to access. However, this IE refers TAC for NR defined as 3 octets, hence current NGAP specification does not support to configure 2 octets TAC for E-UTRAN as a forbidden TAC.

## 2.2	Required enhancements

A possible way forward to resolve this issue would be via the following minor enhancements:

Stage 2:

- In TS 23.501, include forbidden EPS areas in the restrictions provided from the UDM to the AMF and from the AMF to the gNB. See the details for the accompanying CR.

Stage 3:

- In TS 29.503, clause 6.1.6.2.4, in addition to forbiddenAreas include forbiddenEPSAreas. So, that the AMF receives both the forbiddenAreas as well as the forbiddenEPSAreas.

- In TS 38.413 and 38.423, Add new Forbidden Area Information EPS IE within Mobility Restriction List IE to indicate 2 octets forbidden EPS TAC(s).

# 3. Conclusion and proposal

Based on the above observations, we have the following proposal:

Proposal: To initiate a TEI-20 WI to resolve this NR to LTE mobility restriction issue.