3GPP TSG-WG SA2 Meeting #170 	S2-2507085

Gothenburg, Sweden, August 25th – 29th, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	Key issues on Enhancement of IMS Network Capability Exposure

Document for:	Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3 / Rel-20

Abstract: This paper proposes to add the key issue of Key issues on enhancement of IMS network capability exposure into FS_NG_RTC_Ph3 TR.

1. Introduction

The FS_NG_RTC_Ph3 SID includes the following objective:

-	WT#2: Study on IMS and DC capability exposure enhancements

-	WT#2.1: How to enhance IMS capability exposure framework to expose IMS capability for eMMTel services (voice/video) and related events besides Data Channel, supporting IMS session establishment, modification and release to enable eMMTel services that may require enhancing audio/video services with DC capabilities;

In support of IMS capability exposure for eMMTel services (voice/video) as depicted above, study how to enhance Rel-19 IMS event exposure framework and event definition to support IMS DC events supported by DCSF (e.g. application downloading event) and non-DC events, including subscriber specific and non-subscriber specific supported by IMS AS, (e.g. subscriber registration event, or event of all UEs calling a particular customer support number and that have DC capabilities for customization of customer support to UE capabilities);

NOTE: 	This WT will analyze relevant use cases, the gap between OMA and 3GPP APIs and specify exposure functionality missing from OMA APIs. This WT does not intent to transfer the responsibility of OMA API(s) to 3GPP.

2. Text Proposal

It is proposed to capture the following changes into TR 23.700-56.

# * * * * First change * * * *

5.X	Key issue #X: the impact of IMS architecture, interfaces and procedures to enhance IMS capability exposure framework

5.X.1	Description

The solution of this KI will address the following aspects:

-	Identification of new events and/or capabilities to be exposed, in addition to those already listed in Table AD.2.5.3-1 of TS 23.228 V19.3.0 [2], including:

-	application download event;

-	Establishing IMS session with audio/video and IMS data channel.

-	Specify enhancements to IMS architecture, interfaces and procedures to enhance the IMS network capability exposure framework to support the identified new events and/or capabilities.

NOTE 1:	The solutions proposed for this KI need to be backward compatible with the existing OMA API and existing IMS exposure framework defined in TS 23.228 [2].

# * * * * End of changes * * * *