3GPP TSG-SA WG2 Meeting #170	S2- 2507090

25 - 29 August, 2025, Goteborg, Sweden

Title:	23.700-14: New Solution for Key Issue#5: Sensing Result Exposure

Source:	ETRI

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract:	This contribution proposes a solution to support Key Issue #5: Sensing Result Exposure.

Proposal

The following solution is proposed for inclusion in TR 23.700-14 to address Key Issue #5, which concerns the exposure of sensing results to support sensing services. This solution specifies mechanisms by which the Core Network can expose sensing results, with or without contextual information as defined in TS 22.137 [2], to authorized service consumers, including both trusted and untrusted Application Functions (AFs).

Key aspects to be studied for this solution include:

-	Types of sensing results: The nature and format of sensing results, optionally enriched with contextual information (e.g., timestamp, location, velocity, object type, confidence score).

-	Exposure mechanisms: Interfaces and procedures for delivering sensing results directly to trusted AFs or indirectly via the NEF for untrusted AFs, with applicable access control and policy enforcement.

-	Reporting modes: Support for one-time delivery, periodic reporting, and event-triggered notifications to meet different service requirements.

* * * First Change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 | X |  | X | X | X |  |
| #2 |  | X |  |  | X |  |
| #3 | X | X | X | X | X | X |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  | X |  |  |
| #6 | X | X | X |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 |  | X |  |  |  |  |
| #10 |  | X |  |  |  |  |
| #11 |  | X |  |  |  |  |
| #12 |  | X |  |  |  |  |
| #13 |  |  | X |  |  |  |
| #14 |  |  | X |  |  |  |
| #15 |  |  | X |  |  | X |
| #16 |  |  | X |  |  |  |
| #17 |  |  | X |  |  |  |
| #18 |  |  |  | X |  |  |
| #19 | X | X |  |  | X |  |
| #20 |  |  |  |  |  | X |
| #X |  |  |  |  | X |  |

6.X	Solution #X: Sensing Result Exposure for (un)trusted Application Functions

6.X.0	High-level Solution Principles

Editor's note:	Where possible similar/overlapping solution proposals should be documented as a single solution in the TR.

Documentation of more than one solution from the same company for the same KI is allowed but it shouldn't be expected that all of them will be documented in the TR.

This solution defines a mechanism for exposing sensing results, along with associated service parameters and procedures, to both trusted and untrusted Application Functions (AFs). The focus is on specifying a clear service interface and parameter set that supports one-time requests, continuous monitoring, updates, and notifications for sensing services. Flexible and secure delivery modes are supported, including one-time, periodic, and event-triggered reporting, with interfaces reflecting the consumer’s trust relationship with the network:

- Trusted AFs obtain sensing results directly from the Sensing Function via interfaces such as Nsf_SensingService, supporting request, response, update, and notify operations.

- Untrusted AFs access sensing services via the NEF using Nnef_EventExposure, with policy enforcement and authorization applied.

The Sensing Function aggregates data from distributed sensing entities, processes the information, and delivers results enriched with structured metadata such as timestamp, object type, location, velocity, and detection confidence. Optional sensing assistance information (e.g., area of interest, motion thresholds, target object profile) may be provided to improve sensing accuracy and relevance.

6.X.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Clause(s) may be added to capture details.

This solution addresses Key Issue #5: Sensing Result Exposure by specifying how sensing results, optionally enriched with contextual information, are exposed to authorized sensing service consumers. These consumers may include trusted or untrusted AFs requesting sensing data for use cases such as object detection, environmental monitoring, or motion tracking.

A Sensing Service Consumer can request sensing results:

- Via NEF, for untrusted AFs, ensuring policy control and authorization.

- Directly to the sensing function, for trusted AFs, using a dedicated interface.

The Sensing Function is responsible for:

- Collecting and aggregating sensing data from distributed sensing entities (e.g., NG-RAN nodes).

- Processing sensing data to generate sensing results.

- Exposing sensing results to authorized consumers, potentially enriched with contextual information (e.g., timestamp, service area, velocity, confidence level).

NOTE:	The Sensing Function may correspond to the network function responsible for sensing result exposure, as defined in the outcome of the study on Key Issue #1: System Architecture to Support Sensing.

The solution supports the following reporting modes:

- One-time delivery (on-demand)

- Periodic reporting (e.g., every 10s, 1 min)

- Event-triggered reporting (e.g., object movement, area entry/exit)

The sensing function exposes these results using:

- Direct interface (Nsf_SensingService) for trusted AFs.

- Indirect interface (Nnef_EventExposure) via NEF for untrusted AFs.

Optionally, sensing assistance information, as described in TS 22.137 [2], may be provided by authorized third parties (e.g., area of interest, target object parameters, velocity threshold) to improve sensing performance. The returned sensing results include structured metadata, such as object ID, result ID, timestamp, confidence level, and termination causes, if applicable.

6.X.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

This clause details how trusted or untrusted AFs interact with the sensing function to request, subscribe to, and receive sensing results. The process encompasses sensing service request initiation, authorization, sensing operation execution, and result notification.

The Sensing Function exposes sensing services as described in Table 6.X.2-1. The Nsf_SensingService interface supports request/response-based interactions for sensing service configuration and subscription-based delivery of sensing results, accessible directly by trusted AFs or indirectly via the NEF for untrusted AFs.

Table 6.X.2-1: Sensing Function Exposure Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) | Description |
| --- | --- | --- | --- | --- |
| Nsf_SensingService | Request | Request/Response | NEF, AF | One-time request or implicit subscribe for monitoring |
|  | Response | Request/Response | NEF, AF | One-time sensing result response or ACK for subscription |
|  | Update | Request/Response | NEF, AF | Modify or cancel existing subscription |
|  | Notify | Subscribe/Notify | NEF, AF | Sensing result for monitoring |

6.X.2.1	Sensing Service Request from AFs

A trusted AF subscribes to sensing services using the Nsf_SensingService_Subscribe directly to the sensing function. In contrast, an untrusted AF initiates subscriptions via the NEF using Nnef_EventExposure_Subscribe, which forwards the request to the sensing function. Each request includes:

- Sensing type: object detection, tracking, environment monitoring, etc

- Target: specific object(s) or area

- QoS: accuracy, resolution, latency

- Reporting mode: one-time, periodic, or event-triggered

- Optional sensing assistance information: object information (e.g., ID, expected type), area information (e.g., coordinates, boundaries), thresholds (e.g., velocity, direction)

6.X.2.2	Sensing Result Exposure to AFs

Upon receiving a valid subscription, the sensing function configures sensing operations among the relevant sensing entities. When results become available, the sensing function transmits them either directly to the trusted AF using the Nsf_ SensingService_Notify or indirectly through the NEF using Nnef_EventExposure_Notify for untrusted AFs. Each notification includes:

- Sensing result payload: object-specific information such as type, location, speed, direction, and geometric representation

- Contextual information: timestamp, accuracy estimation, confidence score, and serving area

Figure 6.X.2-1: Sensing Service exposure procedure for sensing result

The sensing result exposure process generally follows these sequential steps:

- Sensing Service Request: AF issues request (direct or via NEF if untrusted).

- Data Collection: Sensing function gathers data from sensing entities as configured.

- Sensing Service Response: Sensing function response with sensing result.

- (Un)Subscribe Request: AF subscribes to updates or event triggers.

- (Un)Subscribe Response: Acknowledged by sensing function.

- Event Notification: Sensing result exposed to AF as per configuration, including:

- Result parameters (e.g., object type, location, velocity, direction)

- Contextual metadata (e.g., timestamp, accuracy, confidence level)

6.X.3	Impacts on Services, Entities and Interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

Sensing Function:

-	Introduced as a new functional entity responsible for processing sensing data and exposing sensing results to authorized consumers.

Sensing Service Consumer (e.g., AF):

-	External application that consumes sensing results to enable vertical use cases such as motion tracking, object detection, or environmental sensing.

NEF:

-	Optionally mediates sensing result exposure for untrusted AFs, providing access control, authorization, and policy enforcement.

* * * End of Changes * * * *