SA WG2 Meeting #170	S2-2507100

25 – 29 Aug, 2025, Goteborg , SE

Source:	CATT

Title:	[General aspects] 6G System Architectural Requirements and Assumptions

Document for:	Approval

Agenda Item:	20.6.0

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This paper discusses how 6G NFs to be designed for supporting connectivity services and/or beyond connectivity services, and proposes 6G System Architectural Requirements and Assumptions.

1. Justifications

Architectural Motivation:

Per the approved FS_6G_ARC SID (SP-250806), the support of connectivity services and/or beyond connectivity services is to be studied for 6G system architecture. This paper provides an overview on how 6G NFs are to be designed for supporting these services.

Gap Analysis:

a)	6G NFs to be added for supporting new features

In order to support the beyond connectivity services (e.g. AI, Sensing, and Computing), new 6G NFs need to be added. For example:

-	For support of AI, new 6G NFs like AI Management Function need to be added

-	For support of Sensing, new 6G NFs like Sensing Function need to be added

-	For support of Computing, new 6G NFs like Computing Coordination Function need to be added

For support of data framework, new 6G NFs like Data Management Function, Data Processing Function and Data Storage Function need to be added.

b)	6G NFs that need further study and that may be redesigned

In order to overcome the obstacles identified for 5G system, e.g. strong dependency between AM and SM in the current 5G NAS design, some 6G NFs need to be redesigned along with the new 6G NAS design. These 6G NFs may include, e.g. AMF, SMF, LMF, etc.

c)	6G NFs that use 5GC NFs as basis and any enhancements

Some CN NFs do not need to be redesigned during the ‘G’ revolution, so some 5GC NFs can be used as basis for 6G NFs design. These 6G NFs may include, e.g. UDM, NRF, NEF, etc.

The following Figure-1 provides an illustrative overview on new 6G NFs, redesigned 6G NFs and reused 6G NFs. It can be foreseen that a 6G new core is a must for the support of beyond connectivity services as well as new 6G NAS design.

Figure-1 Overview of 6G Architecture

2. Proposal

It is proposed to capture the following changes into TR 23.801-01.

**** First Change (all text new) ****

4	Architectural Requirements, Assumptions and Principles

4.1	High level Architectural Requirements

The architecture of the 6G System shall:

1	Support and enable use of AI in 6G (e.g. AI agent, framework).

2.	Support integration of Sensing and Communication over 3GPP access.

3.	Support computing for UE, core network and application server.

4.	Support data framework for all aspects related to efficient and scalable data handling.

**** Second Change (all text new) ****

4.2	Architectural Assumptions

1.	6G New Core shall support new 6G NFs, redesigned 6G NFs and reused/enhanced 5G CN NFs.

**** End of Changes ****