SA WG2 Meeting #170	S2-2507101

25 – 29 Aug, 2025, Goteborg , SE

Source:	CATT

Title:	[WT#8] Key Issue for WT#8

Document for:	Approval

Agenda Item:	20.6.8

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This paper proposes Key Issue for WT#8.

1. Justifications

Architectural Motivation:

Per the approved FS_6G_ARC SID (SP-250806), support of cellular IoT is to be studied for 6G system architecture as formulated in WT#8.

WT#8: Study whether and how to support cellular IoT enablers in 6G, based on RAN decision for 6G IoT.

NOTE 9:	The detailed scope for WT#8 will be coordinated and aligned with RAN. Ambient IoT is not in the scope of the study.

Gap Analysis:

The cellular IoT device refers to LPWA (Low Power Wide Area) device, and in 3GPP terminology cellular IoT refers to Cellular Internet-of-Things (CIoT), and CIoT is also referred to MTC in earlier 3GPP releases. Many CIoT features, e.g. CIoT CP optimization/CIoT UP optimization, power saving, coverage enhancement, etc., have been supported for 5GS as defined in clause 5.31 of TS 23.501, and these features can be used as basis and need to be reviewed for the study of support of cellular IoT for 6G system.

Technical Impact:

This WT has dependency on WT#1.

2. Proposals

It is proposed to capture the following changes into TR 23.801-01.

**** First Change (all text new) ****

5.X. Key Issue #X: Support of cellular IoT

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

The cellular IoT device refers to LPWA (Low Power Wide Area) device, and in 3GPP terminology cellular IoT refers to Cellular Internet-of-Things (CIoT), and CIoT is also referred to MTC in earlier 3GPP releases. Many CIoT features, e.g. CIoT CP optimization/CIoT UP optimization, power saving, coverage enhancement, etc., have been supported for 5GS as defined in clause 5.31 of TS 23.501 [xx], and these features can be used as basis and need to be reviewed for the study of support of cellular IoT for 6G system.

Service requirements for 6G Massive Communication as defined by SA1 need to be considered for this Key Issue.

The following aspects will be studied:

-	Whether and how to support the CIoT features as defined in clause 5.31 of TS 23.501 [xx] in 6G system.

NOTE 1:	The detailed scope for this Key Issue will be coordinated and aligned with RAN.

NOTE 2:	Ambient IoT is not in the scope of this Key Issue.

NOTE 3:	6G Core does not support prior-G CIoT devices (e.g. NR RedCap UE, LTE NB-IoT UE) access.

**** End of Changes ****