3GPP TSG-SA WG2 Meeting #170	S2-2507103

Stor-Göteborg, Sweden, Aug 25 – 29, 2025

Source: 	ZTE

Title: 	Interim conclusion for KI#1 Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes an interim conclusion for KI#1 of FS_5GSAT_Ph4_ARC regarding the evaluation evaluation of IP-based and non-IP-based voice packet transmission approaches.

1	Discussion

In TR 23.700-19, a total of eleven candidate solutions are identified to support IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC, covering both IP-based and non-IP-based voice packet transmission mechanisms.

The discussion paper S2-2507102(ZTE) clarifies that several assumptions made during SA2#169 were inaccurate. Specifically:

For the IP-based solution:

- In GEO voice scenario, RoHC header can achieve a 1B (instead of 3B), with only 1B (instead of 7-9B) during talk-to-silence transitions.

- RoHC related issues are either minor or can be mitigated by implementation (e.g., sending silence frames in advance, applying UO-0/UO-1 format).

For non-IP based solution:

- DTMF support is essential for GEO voice, which increases the Reduced RTP header to 2 bytes—comparable to RoHC's 1–2 bytes.

- The solution has strong dependencies on SA1, SA3, and SA4, making it unlikely to complete within the Rel-20 timeline.

- Its feasibility and impact require further evaluation due to higher complexity compared to the IP-based approach.

In summary, compared to the non-IP (Reduced RTP) solution, the IP-based (with RoHC) approach provides lower or comparable header overhead (1–2 bytes), better future extensibility (e.g., DTMF, multi-call, FEC, JBM), fewer cross-WG dependencies (e.g., SA1, SA3, SA4), minimal system impact, and RoHC-related issues can be effectively mitigated by implementation.

2 Proposal

It is proposed to agree and update TR 23.700-19 on FS_5GSAT_Ph4_ARC as follows.

# * * * * First Change* * *

8	conclusions

Editor's note: This clause will list conclusions that have been agreed during the course of the study item activities.

8. X	Interim Conclusions for KI#1

To support IMS voice call over NB-IoT NTN, the following principles are concluded:

- The IP-based solution, utilizing Robust Header Compression (RoHC), is identified as the preferred option for transmitting voice packets.

Editor's note: Other principles are FFS.

# * * * * End of change * * * *