SA WG2 Meeting #170 	S2-2507104

25th – 29th August, 2025, Goteborg, Sweden

Source:	Samsung

Title:	New WID: NSSRG enhancement when UE have multiple PDN connections

Document for:	Approval

Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: NSSRG in EPS

Acronym: TEI20_NSSRG_EPS

Unique identifier:

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

In TS 23.501 Clause 5.15.12, Network Slice Simultaneous Registration Group was introduced to ensure that only the compatible slices can be allowed to use simultaneously in the network. With the help of this NSSRG information, AMF in 5GS ensures that UE is not able to use slices simultaneously which does not share the common NSSRG information. But when UE gets attached to EPS and triggers multiple PDN connections, it may happen that the slices used in those PDN connections does not share common NSSRG information. There are two scenarios/use cases which will occur when UE trigger multiple PDN connections in the EPS and this NSSRG feature does not work.

Also, in Rel 19, if the UE can select the S-NSSAI based on the URSP while being in EPS and send the same S-NSSAI during PDN connection establishemnt. At the same time, the S-NSSAI used in EPS may not share common NSSRG information with S-NSSAIs present in Allowed NSSAI in 5GS.

1st use case (when PDN connections are handled by the same SMF+PGW-C, network assigns S-NSSAI)

At the moment when SMF+PGW-C supports more than one S-NSSAI and APN is valid for more than one S-NSSAI then during PDN connection establishment SMF+PGW-C provides one S-NSSAI which is part of the UE’s subscribed S-NSSAI and the S-NSSAI is not subjected to NSSAA. For the subsequent PDN connections from the same UE, if SMF+PGW-C does not consider the NSSRG information then there is probability that SMF+PGW-C will provide one slice which may not share the common NSSRG information with the S-NSSAI of the already established PDN connections and as a consequence the NSSRG feature won’t work as expected.

2nd use case (when PDN connections are handled by different SMF+PGW-Cs, network assigns S-NSSAI)

If the subsequent PDN connection land at a different SMF+PGW-C, then most probably it will provide one S-NSSAI which may not share a common NSSRG information with the S-NSSAI of the already established PDN connections as it is not aware of any established PDN connection that is handled by a different SMF+PGW-C and as a consequence the NSSRG feature won’t work as expected.

3rd use case (UE has Allowed NSSAI in 5GS and sends one S-NSSAI during PDN connections based on the URSP received for EPS)

UE successfully registers in 5GS and received Allowed NSSAI (e.g. in non-3GPP access) having S-NSSAIs sharing common NSSRG information. If the same UE has received URSP applicable for EPS and send one S-NSSAI during PDN connection based on the URSP, it may happen that the used S-NSSAI in PDN connection does not share common NSSRG information with S-NSSAIs present in Allowed NSSAI of 5GS.

4th use case (UE sends one S-NSSAI during PDN connections based on the URSP received for EPS and then triggering registrations with incompatible slices in 5GS)

UE has received URSP applicable for EPS and send one S-NSSAI during PDN connection based on the URSP, PDN connection is successfully established. Now it may happen that UE triggers registration in 5GS (e.g. in non-3GPP access) requesting S-NSSAIs not sharing common NSSRG information with the used S-NSSAI for PDN connection and then AMF adds the requested S-NSSAIs into Allowed NSSAI.

# 4	Objective

The objective is to address the NSSRG in EPS by:

-	Assuming that SMF+PGW-C consider the received NSSRG information from UDM while providing the S-NSSAI for subsequent PDN connections.

-	Also, SMF+PGW-C queries the UDM for any PDU Session ID(s) and S-NSSAI(s) that are already registered in the UDM for the UE.

-	UE considers the common NSSRG information while sending any S-NSSAI during PDN connection establishment based on the received URSP applicable in EPS with any available Allowed NSSAI in 5GS and vice-versa.

Note: This work will require 1 TU.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.502 | Add description that SMF+PGW-C queries the UDM to know about any existing PDU sessions and corresponding S-NSSAIs and then considering NSSRG information while providing S-NSSAI to UE in PCO. | SA#110 (Dec 2025)?? |  |
| 23.501 | Add description about the NSSRG handle when UE is in EPS and send S-NSSAI during PDN connections only if share common NSSRG information with Allowed NSSAI of 5GS and vice-versa. | SA#110 (Dec 2025)?? |  |

# 6	Work item Rapporteur(s)

Ashok Kumar Nayak, Samsung (ashok.nayak@samsung.com)

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

Supporting IM name

Samsung