3GPP TSG-WG SA2 Meeting #170	S2-2507107

Goteborg, Sweden, Aug 25th – 29th, 2025

Source:	NTT DOCOMO

Title:	Discussion on impacts of using Control Plane-based solution for voice media

Document for:	Discussion

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract: Discussion on impacts of using Control Plane-based solution for voice media and potential way forward.

# 1. Introduction

TR 23.700‑19 explores the integration of satellite components, particularly the support for IMS voice over NB-IoT NTN connected to EPC.  Under Key Issue #1, one key scenario involves supporting IMS voice services over NB-IoT via GEO satellite connectivity to the EPC. Several solutions proposed in the study are control-plane-based, wherein SIP signaling and/or media are conveyed using Signaling Radio Bearers (SRBs).

This paper discusses the potential impacts of using Control Plane-based solution for voice media in this specific context of IMS voice over NB-IoT NTN connected to EPC and proposes that SA2 consult with the appropriate RAN working groups (if needed based on agreed proposal) to assess its feasibility.

# 2. Discussion

Observation 1 – Control-plane and hybrid solutions proposed: Control-plane (and hybrid) solutions using SRBs for IMS signaling or media are gaining attention in Key Issue #1 (solutions #5, #6, #7, #8, #10, #11, etc in TR 23.700-19), due to their suitability for NB-IoT devices. These solutions propose using SRBs for SIP signaling and/or voice media.

Observation 2 – RLC AM vs UM: GEO satellite communication imposes a high round-trip time (RTT) — typically exceeding 500 ms — which significantly impacts protocols that rely on retransmission-based reliability such as RLC AM. RLC AM involves retransmissions, reordering, and status reporting mechanisms, all of which introduce latency that can be detrimental to the setup time and perceived quality of IMS voice services, particularly for NB-IoT devices with limited processing capability.

Whereas RLC UM provides lower-latency transmission by avoiding retransmissions and reordering, which may result in occasional packet loss but benefits applications such as voice over control plane.

Furthermore, for voice media transmission, occasional loss is typically tolerable, especially compared to delay-induced service failures. Also, NB-IoT UEs are typically resource-constrained, and RLC UM is operationally simpler than RLC AM, providing benefits in terms of memory and processing.

Observation 3 – Existing Specifications:

Under the current specifications (TS 36.331), RLC AM is the only applicable RLC mode for SRBs. RLC UM is not permitted for these SRBs in existing system configurations.

Moreover, as per GSMA IR.92’s specification for “IMS Profile for Voice and SMS”, in the current VoLTE, RLC UM is used for voice media, albeit over UP.

However, in the case of control plane only (CP + CP) proposed solutions, voice media is proposed to be over SRB, which currently uses only RLC AM. Hence, maybe one consideration for CP only solution would be to consider RLC UM also over SRBs.

# 3. Conclusion and proposal(s)

Based on the above observations, there are 3 possible proposals as the way forward. They are as follows:

Proposal 1: Voice media shall be transported over UP and not CP.

Proposal 2: If CP solution is pursued also for voice media, accept the existing RLC AM for voice media over SRB. Send a corresponding Liaison Statement to RAN2 WG enquiring about the possible implications of accepting RLC AM for voice media in terms of propagation delay etc.

Proposal 3: Send a Liaison Statement to RAN2 WG asking them to develop SRB over RLC UM in the context of IMS voice over NB-IoT NTN connected to EPC.