SA WG2 Meeting #S2-170	S2-2507110

25 - 29 August, 2025, Goteborg, Sweden	(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6396 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the Energy exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the clasue 5.51.2.2.1, the general clause for the Energy Consumption information collection, the Node ID (gNB ID, (I-)UPFs ID) collection part is missing. It proposes to add. In the clause 5.51.2, there are three term about report time condition, i.e reporting period, time window, reporting time period. It cause the confusion in the CT3 discussion. What does the reporting time period mean? It refers to reporting period, time window, or both. In the context, it was combined with threshold. In general, the periodic reporting is not combined with threshold in the CT3 design. Therefore, this CR proposes to replace the reporting time period with “time window” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying Node description and reporting time condition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Node ID is missing in information collected by EIF. What does the reporting time period mean is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.2.2.1, 5.51.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.51.2.2	Energy Consumption information collection

##### 5.51.2.2.1	General

The Energy Information Function (EIF) collects the UE related Energy Consumption information including Node-level energy consumption information, Node-level data volume from OAM and data volume of the required granularities (i.e. S-NSSAI, PDU Session and/or Service Data Flow) from UPF (via SMF) and Nodes ID (i.e. gNB, (I-) UPF) related to the data volume. The EIF can expose the energy consumption at the requested granularity over a certain time window (i.e. the total energy consumed from a start time to an end time) or periodically (i.e. every reporting period of e.g. 10 minutes).

Node-level energy consumption is collected network-wide at a configurable starting time with interval T such that the Energy Consumption information reported to the EIF is time-aligned.

The reporting period for periodic reporting by the EIF to its consumers can only be a multiple of this time interval T and is time-aligned to the node-level energy consumption reporting occurring at every time interval T configured in the network.

NOTE:	If the reported Energy Consumption information is not time-aligned, it cannot be aggregated.

Next CHANGE

#### 5.51.2.4	Energy Consumption information exposure

AF/NEF subscribes the EIF for Energy consumption information of required granularities (UE, S-NSSAI, PDU session and/or Service Data Flow).

-	For UE level energy exposure, the consumer NF provides UE ID (SUPI/GPSI).

-	For S-NSSAI of the UE level exposure, the consumer NF provides UE ID (SUPI/GPSI), S-NSSAI.

-	For PDU session level exposure, the consumer NF provides UE ID (SUPI/GPSI), DNN/S-NSSAI.

-	For Service Data Flow level exposure (e.g. per UE per application), the consumer NF provides UE ID (SUPI/GPSI), DNN/S-NSSAI and application information, e.g. Application Identifier, or Flow description(s).

The consumer NF may also subscribe the above information exposure with providing reporting period, or time window. AF/NEF may also subscribe the Energy consumption information exposure of required granularity and may provide reporting threshold for the required granularity and the time window.

NOTE 1:	For the threshold-based reporting, the EIF sends the Energy consumption information of the required granularity to the NEF/AF once when the Energy consumption information of the required granularity matches or exceeds the threshold during the time window.

NOTE 2:	In this Release of the specification, requesting energy-related information by the consumer is supported only for reporting time periods in the future.

END OF CHANGES