SA WG2 Meeting #170	S2-2507119

25 – 29 Aug, 2025, Goteborg, SE

Source:	MediaTek Inc.

Title:	[WT#1.2, Slicing] Discussion on Network Slicing in 6G

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This document discusses the scope of the work on network slicing for 6G pertaining to WT1.2 in the approved SI.

# 1. Justifications

A new Study Item on Study on Architecture for 6G System was approved at SA#108 (SP-250806).

Work Task 1.2 reads as:

Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

This contribution intends to clarify the scope of the work related to network slicing.

Architectural Motivation:

A first point for discussion per WT#1.2 is "whether" to support network slicing in 6G.

Network slicing, though complex and arguably over-engineered in 5G, is an important feature for 5G SA MNOs. It is not expected that this feature be challenged, however, its definition ought to be re-examined as part of the 6G Study in view of lessons learnt from 5G and new beyond-communications services.

Proposal 1: Introduce network slicing as a basic architecture assumption at least for communication services.

Gap Analysis:

The next point for discussion is basically how to define network slicing in 6G. This is the expected outcome from the study. In our view this can be structured into:

a)	Identifying the lessons learnt from network slicing in 5G SA, what to port into 6G (as is or modified) and what not to port into 6G, and why. This activity focuses on communication services.

b)	Identifying whether and how to support network slicing with new "beyond-communication" services including Compute, AI, Data, Sensing in view of network slicing with communication services.

NOTE:	Stage 1 requirements (none at the moment) may relate to a) and b)

It is strongly recommended to first proceed with a) before diving into b) i.e., to first understand how slicing could look like in 6G before adapting it to beyond-communication. Indeed, while a lot of complexity has been introduced for network slicing across several releases in 5G SA, implementations in networks and devices have simply not followed suit. One important observation, which is not isolated to slicing (similar observations have been made in RAN on some other network features), is the undue burden put on networks to depend on device implementations to introduce new network functionality.

It is also very important to stress the following key principle (SP-250806): "create lean and streamlined standards for 6G, e.g., by dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, avoiding excessive configurations, etc. Any exception to the above shall be well justified."

As a result, two Key Issues are proposed to be introduced, related to points a) and b) above.

Key Issue 1: Based on 5G SA network slicing introduced since Rel-15, to identify:

-	what network slicing functionality should be introduced into 6G, whether as is or modified and how; and

-	if modified, to characterise the issue being addressed and the related solutions

-	what network slicing functionality should not be introduced into 6G

-	interworking between network slicing in 5G SA and network slicing in 6G will be considered.

Key Issue 2: Depending on Key Issue 1, to identify whether and how to support network slicing with new "beyond-communication" services including Compute, AI, Data, Sensing in view of network slicing with communication services.

Proposal 2: to introduce Key Issues 1 & 2 above into the TR.

**** First Change ****

# Annex A.X. WT#1.2 Scope - Slicing

Extracted below from SP-250806:

1.2. Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

Note the below update is just intended as information in the TR and no change is expected in the WT wording in the SID.

This sub-WT focuses on studying whether and how to support and/or enhance network slicing. Below aspects are studied:

X.1	Based on 5G SA network slicing introduced since Rel-15, to identify:

-	what network slicing functionality should be introduced into 6G, whether as is or modified and how; and

-	if modified, to characterise the issue being addressed and the related solutions

-	what network slicing functionality should not be introduced into 6G

-	interworking between network slicing in 5G SA and network slicing in 6G will be considered.

X.2	Depending on X.1, to identify whether and how to support network slicing with new "beyond-communication" services including Compute, AI, Data, Sensing in view of network slicing with communication services.

**** Second Change ****

# 5.X. Key Issue #X: Network Slicing in 6G

This Key Issue focuses on studying whether and how to support and/or enhance network slicing. Below aspects are studied:

X.1	Based on 5G SA network slicing introduced since Rel-15, to identify:

-	what network slicing functionality should be introduced into 6G, whether as is or modified and how; and

-	if modified, to characterise the issue being addressed and the related solutions

-	what network slicing functionality should not be introduced into 6G

-	interworking between network slicing in 5G SA and network slicing in 6G will be considered.

X.2	Depending on X.1, to identify whether and how to support network slicing with new "beyond-communication" services including Compute, AI, Data, Sensing in view of network slicing with communication services.

**** End of Changes ****