3GPP TSG-WG SA2 Meeting #170	S2-250xxxx

# Goteborg, SE, 25th Aug – 29th Aug, 2025		(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | - | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement of Non-3GPP Device Identifiers and Framed Routing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20_IPARA |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-19, UE initiates a PDU session modification procedure to associate the Non-3GPP Device Identifier behind UE with user plane address information. In case of IPv4, multiple ports or port ranges have to be allocated to identify the Device, considering that each application running in the Device needs at least one dedicated port. Always relying on multiple ports or port ranges to identify a Device is not flexible and introduces additional overhead in the signalling. In addition, Framed Route information is provided to SMF by a DN-AAA server or UDM, containing range(s) of IPv4 addresses or IPv6 prefixes. So far the network does not send Framed Route information to the UE, and how the devices behind the UE gets the IP addresses is not defined by 3GPP. In terms of realization, the static configuration is not flexible, while requesting the Framed Routes via RADIUS consumes extra resource over the air. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Specify that the SMF can allocate and send the range of IPv4 addresses to the UE during the PDU Session Establishment for QoS differentiation of traffic for Non-3GPP Device Identifier behind the UE. - Specify that the SMF can send the Framed Route information to the UE via NAS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Difficult to identify the devices in order to implement QoS differentiation of traffic for Non-3GPP Device Identifier, in case of IPv4; Difficult for the devices behind the UE to get IP addresses of Framed Routes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4b, 5.6.14, 5.52.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR xxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.4.4b	UE 5GSM Core Network Capability handling

The UE 5GSM Core Network Capability is included in PDU Session Establishment/Modification Request.

The UE shall indicate in the UE 5GSM Core Network Capability whether the UE supports:

-	"Ethernet" PDU Session Type supported in EPC as PDN Type "Ethernet";

-	Reflective QoS;

-	Multi-homed IPv6 PDU Session (only if the Requested PDU Type was set to "IPv6" or "IPv4v6");

-	ATSSS capability (as referred to clause 5.32.2);

-	Transfer of Port Management Information containers;

-	Support for secondary DN authentication and authorization over EPC (as referred to clause 5.17.2.5);

-	(S)RTP Multiplexed Media Identification Information in IP Packet Filters;

-	Range of IPv4 address for non-3GPP device;

-	Framed Routing.

The 5GSM Core Network Capability is transferred, if needed, from V-SMF to H-SMF during PDU Session Establishment/Modification procedure.

After the first inter-system change from EPS to 5GS for a PDU session established in EPS, the 5GSM Core Network Capability is also included in the PDU Session Modification if the Reflective QoS and/or Multi-homed IPv6 PDU Session is present.

# * * * * Second change * * * *

5.6.14	Support of Framed Routing

Framed Routing is only defined for PDU Sessions of the IP type (IPv4, IPv6, IPv4v6) and allows to support an IP network behind a UE, such that a range of IPv4 addresses or IPv6 prefixes is reachable over a single PDU Session, e.g. for enterprise connectivity. Framed Routes are IP routes behind the UE.

A PDU Session may be associated with multiple Framed Routes. Each Framed Route refers to a range of IPv4 addresses (i.e. an IPv4 address and an IPv4 address mask) or a range of IPv6 Prefixes (i.e. an IPv6 Prefix and an IPv6 Prefix length). The set of one or more Framed Routes associated to a PDU Session is contained in the Framed Route information. If the UE supports to receive Framed Route information from the network via NAS, the SMF sends the Framed Route information to the UE in the PDU Session Establishment procedure, as specified in clause 4.3.2.2.1 of TS 23.502 [3]. In other cases, the network does not send Framed Route information to the UE: devices in the network(s) behind the UE get their IP address by mechanisms out of the scope of 3GPP specifications. See RFC 2865 [73], RFC 3162 [74].

NOTE 1:	How the UE allocates the IPv4 address / IPv6 Prefix belonging to one of the Framed Routes to the non-3GPP device behind the UE is implementation specific.

Framed Route information is provided by the SMF to the UPF (acting as PSA) as part of Packet Detection Rule (PDR, see clause 5.8.5.3) related with the network side (N6) of the UPF.

NOTE 2:	SMF can take the UPF capabilities into account when selecting PSA UPF, to ensure that the SMF chooses PSA UPF(s) that support Framed Routing for PDU Sessions to DNN and/or slices deemed to support Framed Routing e.g. DNN and/or slices intended to support RG or if Framed Route information has been received as part of Session Management Subscription data.

The Framed Route information may be provided to the SMF by:

-	the DN-AAA server as part of PDU Session Establishment authentication/authorization by a DN-AAA server (as defined in clause 5.6.6); or by

-	Session Management Subscription data associated with DNN and S-NSSAI sent by UDM (as defined in clause 5.2.3.3.1 of TS 23.502 [3]).

If the SMF receives Framed Route information both from DN-AAA and from UDM, the information received from DN-AAA takes precedence and supersedes the information received from UDM.

The IPv4 address / IPv6 Prefix allocated to the UE as part of the PDU Session establishment (e.g. delivered in NAS PDU Session Establishment Accept) may belong to one of the Framed Routes associated with the PDU Session or may be dynamically allocated outside of such Framed Routes.

If PCC applies to the PDU Session, at PDU Session establishment the SMF reports to the PCF the Framed Route information corresponding to the PDU Session (as described in clause 6.1.3.5 of TS 23.503 [45]). In this case, in order to support session binding, the PCF may further report to the BSF the Framed Route information corresponding to the PDU Session (as described in clause 6.1.2.2 of TS 23.503 [45]).

If the UDM or DN-AAA updates the Framed Route information during the lifetime of the PDU Session, the SMF releases the PDU Session and may include in the release request an indication for the UE to re-establish the PDU Session.

# * * * * Third change * * * *

5.52.3	Session management enhancement

For the traffic of non-3GPP devices requiring differentiated QoS, the Non-3GPP Device Connection Information may be signalled by the UE as defined in TS 24.501 [47]. When a non-3GPP device is connected to the UE, the UE may include the Non-3GPP Device Connection Information in PDU Session Modification Request to SMF. The SMF forwards the Non-3GPP Device Connection Information to the PCF for policy control.

NOTE 1:	It is up to UE implementation to determine when to initiate PDU Session Modification procedure for updating the Non-3GPP Device Connection Information.

For an Ethernet PDU Session, the Non-3GPP Device Connection Information includes:

-	Non-3GPP Device Identifier;

-	MAC address used in PDU session;

-	Optionally, VLAN tag ID that is associated with the non-3GPP device used in PDU session.

For an IPv4 or IPv4v6 PDU Session, the Non-3GPP Device Connection Information includes:

-	Non-3GPP Device Identifier;

-	IPv4 Address associated with the non-3GPP device used in PDU session;

-	Optionally, port ranges associated with the non-3GPP device used in PDU session.

For an IPv6 or IPv4v6 PDU Session, the Non-3GPP Device Connection Information includes:

-	Non-3GPP Device Identifier;

-	IPv6 Address/prefix(sub) associated with the non-3GPP device used in PDU session;

-	Optionally, port ranges associated with the non-3GPP device used in PDU session.

NOTE 2:	If IPv4v6 PDU Session is applied, it is up to UE implementation to determine to use IPv4 or IPv6 or both Address/prefix(sub) based on the associated traffic of the non-3GPP device and the IP allocation by the network to the PDU Session.

Editor's note:	The detailed encoding of Non-3GPP Device Connection Information will be defined in stage-3.

For an IPv4 PDU Session, the SMF may decide to allocate a range of IPv4 address for the non-3GPP device(s) behind the UE based on DNN configuration, subscription data and operator policies, and send it to the UE during the PDU Session establishment procedure, as specified in clause 4.3.2.2.1 of TS 23.502 [3]. The UE then allocates the IPv4 Address(es) belonging to the range of IPv4 address to the non-3GPP device(s) and includes it in the Non-3GPP Device Connection Information in the PDU Session Modification Request to the SMF.

If the PCF indicates to the SMF that the corresponding Non-3GPP Device Identifier is not available for the UE as specified in clause 6.1.3.31 of TS 23.503 [45], the SMF rejects the PDU Session Modification with a cause code to notify the UE that the Non-3GPP Device Identifier is not available for the UE.

# * * * * End of changes * * * *