3GPP TSG-WG SA2 Meeting #170	S2-2507129

Goteborg, SE, 25th Aug – 29th Aug, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	23.700-65 KIs for Work Task 1: Identification of Emergency Response Centre for SMS and discovery of support

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract: Proposed key issue to address work task 1

# 1. Introduction and Discussion

The FS_SMS2EC_ARC (SP-250837) includes the following WTs:

The study item aims at investigating how to specify architectural and procedural enhancements to support Short Message Service to Emergency Response Centre in both EPS and 5GS, based on the requirements specified in TS 22.101. The study includes the following work tasks: - WT-1: How to identify a Short Message to Emergency Response Center based on the use of emergency numbers. Address aspects when serving PLMN does not support this feature, but UE supports it. - WT-2: Whether and how to identify the type of emergency service (police, ambulance, fire brigade, etc.) using the same identification mechanism as for emergency calls. - WT-3: How to route a Short Message to Emergency Response Center (i.e. PSAP) serving the UE’s location, according to the local regulations (in roaming case, the regulations applicable to the UE’s location). NOTE 1: Depending on the type of emergency service, Short Messages can be routed to different emergency response centres. NOTE 2: This study assumes that Short Messages initiated by the Emergency Response Center in response to the Short Message to the Emergency Response Center are treated as regular Short Messages. For EPS and 5GS, 3GPP access (including TN and NTN) is in scope. UE in limited service state will not be in scope of this study item. The architectures for SMS over NAS (see TS 23.272, TS 23.401, TS 23.501, TS 23.502) and SMS over IP (see TS 23.204), and the architectural principles of Emergency Services (see TS 23.167) will be used as a basis. SMS over IP can be applied for both 3GPP access and non-3GPP access (either over EPS or 5GS).

To address work task 1 it is proposed to study two aspects:

1. 	How the network identifies an SMS as SMS to Emergency Centre based on the destination number;

2.	What is the expected outcome in case the UE sends an SMS that is to be routed to an Emergency Centre, but the serving PLMN does not support this functionality.

The following Key Issue is proposed based on the above aspects of Work Task 1.

# 2. Text proposal

It is proposed to capture the following changes vs. TR 23.700-56.

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[a]	3GPP TS 22.101: "Service aspects; Service principles"

# * * * * Second change (all new) * * * *

## 5.a	Key Issue #x: Identification of SMS to Emergency Response Centre

### 5.a.1	Description

Based on the requirements in TS 22.101 [a], the EPS and 5GS shall have the capability to route a Short Message where the destination is an emergency number to a local response centre (i.e. a PSAP) according to the local regulation.

This key issue aims to study solutions to identify, based on the destination number, whether an SMS should be routed to a PSAP or instead routed as a normal SMS.

The solutions should also consider the scenario where the serving PLMN does not support routing an SMS to an emergency response centre (e.g. whether the support is signalled by the network).

# * * * * End of changes * * * *