3GPP TSG-WG SA2 Meeting #170	S2-2507130

Goteborg, SE, 25th Aug – 29th Aug, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	23.700-65 KIs for Work Task 2: Identification of type of Emergency

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract: Proposed key issue to address work task 2, identification of type of emergency

# 1. Introduction and Discussion

The FS_SMS2EC_ARC (SP-250837) includes the following WTs:

The study item aims at investigating how to specify architectural and procedural enhancements to support Short Message Service to Emergency Response Centre in both EPS and 5GS, based on the requirements specified in TS 22.101. The study includes the following work tasks: - WT-1: How to identify a Short Message to Emergency Response Center based on the use of emergency numbers. Address aspects when serving PLMN does not support this feature, but UE supports it. - WT-2: Whether and how to identify the type of emergency service (police, ambulance, fire brigade, etc.) using the same identification mechanism as for emergency calls. - WT-3: How to route a Short Message to Emergency Response Center (i.e. PSAP) serving the UE’s location, according to the local regulations (in roaming case, the regulations applicable to the UE’s location). NOTE 1: Depending on the type of emergency service, Short Messages can be routed to different emergency response centres. NOTE 2: This study assumes that Short Messages initiated by the Emergency Response Center in response to the Short Message to the Emergency Response Center are treated as regular Short Messages. For EPS and 5GS, 3GPP access (including TN and NTN) is in scope. UE in limited service state will not be in scope of this study item. The architectures for SMS over NAS (see TS 23.272, TS 23.401, TS 23.501, TS 23.502) and SMS over IP (see TS 23.204), and the architectural principles of Emergency Services (see TS 23.167) will be used as a basis. SMS over IP can be applied for both 3GPP access and non-3GPP access (either over EPS or 5GS).

When setting up an emergency call, the UE may indicate the specific emergency service (or emergency services) it requires. In case of emergency calls in the CS domain this indication is provided by setting the relevant bit (or bits) of the emergency call type. As per TS 24.008 the possible emergency services that can be invoked are as follows:

Bit 1	Police

Bit 2	Ambulance

Bit 3	Fire Brigade

Bit 4	Marine Guard

Bit 5	Mountain Rescue

Bit 6	manually initiated eCall

Bit 7	automatically initiated eCall

Bit 8	is spare and set to "0"

In case of PS domain, the indication of the emergency service requested is encoded in the SOS.URN as described in RFC 5031.

It is proposed to study solutions that enable the user to convey the type of emergency service the SMS should be sent to.

# 2. Text proposal

It is proposed to capture the following changes vs. TR 23.700-56.

# * * * * First change (all new) * * * *

## 5.a	Key Issue #x: Identification of Emergency Service Category

### 5.a.1	Description

When setting up an emergency call, the UE may indicate the emergency service category and this information is used in the network to route the call to the most suitable PSAP. In the CS domain the emergency service category is encoded in a byte that is included in the emergency call setup signalling. In the PS domain the emergency service category is encoded in the URN.

An equivalent functionality should be supported also for SMS to Emergency Centre.

This key issue aims to study whether and how the Emergency Service Category can be conveyed so that the network can route the SMS to the desired a Emergency Centre.

# * * * * End of changes * * * *