3GPP TSG-WG SA2 Meeting #170	S2-2507131

Goteborg, SE, 25th Aug – 29th Aug, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	23.700-65 KIs for Work Task 3: SMS2EC routing

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract: Proposal of key issue to address work task 3 – routing of SMS to Emergency Centre

# 1. Introduction and Discussion

The FS_SMS2EC_ARC (SP-250837) includes the following WTs:

The study item aims at investigating how to specify architectural and procedural enhancements to support Short Message Service to Emergency Response Centre in both EPS and 5GS, based on the requirements specified in TS 22.101. The study includes the following work tasks: - WT-1: How to identify a Short Message to Emergency Response Center based on the use of emergency numbers. Address aspects when serving PLMN does not support this feature, but UE supports it. - WT-2: Whether and how to identify the type of emergency service (police, ambulance, fire brigade, etc.) using the same identification mechanism as for emergency calls. - WT-3: How to route a Short Message to Emergency Response Center (i.e. PSAP) serving the UE’s location, according to the local regulations (in roaming case, the regulations applicable to the UE’s location). NOTE 1: Depending on the type of emergency service, Short Messages can be routed to different emergency response centres. NOTE 2: This study assumes that Short Messages initiated by the Emergency Response Center in response to the Short Message to the Emergency Response Center are treated as regular Short Messages. For EPS and 5GS, 3GPP access (including TN and NTN) is in scope. UE in limited service state will not be in scope of this study item. The architectures for SMS over NAS (see TS 23.272, TS 23.401, TS 23.501, TS 23.502) and SMS over IP (see TS 23.204), and the architectural principles of Emergency Services (see TS 23.167) will be used as a basis. SMS over IP can be applied for both 3GPP access and non-3GPP access (either over EPS or 5GS).

As for emergency calls, it is critical that the SMS to Emergency Centre is routed to a PSAP taking into account the location of the user and the emergency category (if provided)

The following Key Issue is proposed to study how to meet the requirements of work task 3

# 2. Text proposal

It is proposed to capture the following changes vs. TR 23.700-56.

# * * * * First change (all new) * * * *

## 5.a	Key Issue #x: Routing of SMS2EC

### 5.a.1	Description

The routing of an SMS to Emergency centre should take into account the local regulations, i.e. the regulations applicable to the country where the UE is located at the time when the SMS is sent, the location of the UE and (if applicable) the Emergency Service Category. This will ensure that the SMS will reach the most suitable PSAP.

It is proposed to study solutions to route an SMS to Emergency Centre to the most suitable PSAP. The solutions should ensure compliance with local regulations.

# * * * * End of changes * * * *