3GPP TSG-WG SA2 Meeting #170 	S2-2507134

Goteborg, Sweden, Aug 25th – 29th, 2025

Source:	NTT DOCOMO

Title:	Solution#5 Update: Addressing ENs.

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract: This contribution proposes to update Solution #5 by addressing ENs.

# 1. Discussion

In TR 23.700‑19 v0.2.0, Solution #5 (Support of IMS voice NB-IoT NTN connected to EPC using control plane bearers and new SRBs) currently has open ENs. The purpose of this paper is to resolve these ENs, particularly the EN for: Bearer mapping when using non-IP type PDN connection. Below is a short description of how and which EN is resolved:

Editor's note:	Details regarding how the PGW performs the IP and NIDD conversion is FFS.

IP and NIDD conversion is covered by existing mechanism as explained in clause 4.5.14 in TS 23.682, where SGi PtP tunnelling based on UDP/IP is used between PGW and AS, and adds (for UL) / removes (for DL) the UDP/IP and forwards the packets from/to UE and AS. Furthermore, details on this are provided in clause 4.3.17.8.3.3.2 in TS 23.401 under SGi PtP tunnelling based on UDP/IP. Therefore, this EN can be resolved using existing specifications.

Furthermore, based on offline comments, there were concerns regarding how the PGW routes the IMS traffic to P-CSCF and the voice to the AGW. This can also be resolved using existing mechanisms in TS 29.244 i.e., the Outer Header Creation/Removal IE (clauses 8.2.56 and 8.2.64). The Outer Header Creation IE in FAR can be used to add UDP/IP encapsulation header which in this context would be the IP/port of the P-CSCF (IMS signalling) or AGW (voice media). The PGW-C indicates the UDP/IP address of the I1 endpoint in P-CSCF in the FAR to PGW-U. In similar manner, if voice media from/to UE is non-IP based RTP, the PGW-C indicates the UDP/IP address of the AGW in the FAR to PGW-U. The PGW-C can configure the PGW-U with a PDR that contains traffic filters, either for I1 or RTP, and associate this PDR with the FAR, so that only I1 traffic goes to the UDP/IP tunnel towards P-CSCF or AGW, respectively.

# 2. Proposal

It is proposed to update the Solution #5 of TR 23.700‑19 v0.2.0.

# * * * * First change* * * *

##### 6.5.1.4.3	Bearer mapping when using non-IP type PDN connection

An additional challenge in supporting voice traffic over NB-IoT NTN via GEO satellite lies in fulfilling the data rate and call setup time requirements, taking into account the increased round-trip delay inherent to GEO satellite links. To address this, one alternative approach would be the use of a non-IP PDN type, rather than an IP-based PDN. This would for instance reduce the protocol overhead (see clause 6.5.1.5). The use of a non-IP PDN type could apply regardless of whether Control Plane CIoT EPS Optimization is used or not.

However, in case of non-IP PDN type, the TFT filters can no longer use IP 5-tuple (i.e. source/destination IP addresses port numbers and transport protocol) to map traffic to separate EPS bearers. Hence, the following new functionalities are proposed:

-	The separation of IMS signalling and voice media shall be based on the traffic type (SIP or I1 vs. RTP/RTCP) in the UL direction. The UE shall map the UL traffic based on the traffic type to the corresponding EPS bearer identities and shall use the corresponding radio bearer(e.g. SRB as per this proposal) for the EPS bearer as configured in the RRC.

-	In DL direction, the PGW shall map the DL traffic using DL TFT as a PDN type PGW as per existing mechanism for IP PDN type i.e. using IP 5-tuple (i.e. source/destination IP addresses port numbers and transport protocol).

The IP and NIDD conversion is performed by using SGi PtP tunnelling based on UDP/IP between PGW and P-CSCF as detailed in clause 4.3.17.8.3.3.2 in TS 23.401 [5]. Furthermore, PGW routes the appropriate traffic to P-CSCF and IMS AGW using the existing Outer Header Creation/Removal as detailed in TS 29.244.

# * * End of changes * * * *