3GPP TSG-SA WG2 Meeting #170	S2-2507136

Gothenburg, Sweden, August 25h - 29th 2025

Source:	CSCN

Title:	Solution for key issue 2 by reducing SIP messages size through JSON-based transmission between UE and P-CSCF

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: The contribution proposes solutions for KI#2.

1. Discussion

In the context of NB-IoT (GEO) voice communication scenarios, the typical IMS signaling sequence typically involves ‌8 message exchanges‌ (INVITE, 100 Trying, 183 Progressing, PRACK, 200 OK, UPDATE, 200 OK, 180 Ringing), with the ‌aggregate SIP message payload‌ generally approximating ‌10 KB‌ in total size. This cumulative data transfer requirement significantly radio resource over bandwidth-constrained NB-IoT networks.

To improve IMS signaling efficiency and reduce SIP messages size in NB-IoT environments, the following optimizations are proposed:

‌1. pre-configuration procedure for invite and SDP parameters‌

- During the registration phase, the UE transmits a pre-configured invite message and SDP parameters to the P-CSCF, where the invite message does not contain information about the peer.

‌2. Minimized INVITE message and SDP parameters during call setup‌

- The invite message only contains its own identifier and the peer's identifier. If any other information needs to be modified, the invite or SDP will only carry the portion that requires changes. P-CSCF reconstructs the complete INVITE and SDP.

3. JSON over TCP/UDP transmission of Minimized SIP message between the UE and P-CSCF

- UEs sending SIP messages must adhere to RFC requirements, and certain Information Elements (IEs) in SIP messages are mandatory. Using JSON to encode SIP messages can omit some mandatory fields and effectively prevent the issue of over-simplified SIP messages violating RFCs.

- Omit mandatory fields already present in pre-configured messages for invite and 200 OK for invite. Optionally, all SIP messages between UE and P-CSCF are transmitted through JSON. The traditional SIP protocol is still used between P-CSCF and MS CN.

- Furthermore, when using JSON to encapsulate transcoded SIP messages, it is advisable to consider employing efficient compression algorithms to compress the transcoded SIP messages.

- Enable a SIP simplification policy between UE and P-CSCF, which can define SIP keyword abbreviations, e.g., P-Access-Network-Information: NR(GEO) = "p-geo".‌

- The UE can use a dedicated port to transmit JSON, or the P-CSCF can determine whether the UE is sending SIP or JSON based on the content of the message.

4. Optionlly, JSON over HTTP/2.0 transmission of Minimized SIP message between the UE and P-CSCF

- Optionally, Minimized SIP messages can be transmitted over TCP using HTTP/2.0 with JSON to omitting mandatory fields. Then an invitation and an 180ringing message will act as a pair of HTTP request and response messages, with 200 OK and ACK messages forming another pair of HTTP request and response messages.

- The UE and P-CSCF can exchange their respective URLs as HTTP servers in the message body of registration requests and 200 OK messages.

5. UE is prohibited from using multiple concurrent calls and forking

- A single caller URI cannot initiate multiple concurrent INVITE requests to the same callee URI, and a single INVITE must not be replicated by a proxy to multiple endpoints, to omit mandatory fields, such as the call id, tag.

6. P-CSCF as enhanced B2BUA

- The core responsibility of a B2BUA is to act as a full-featured SIP User Agent (UA), interacting with the IMS network on behalf of the originating UE. It must fully implement both the UAC and UAS state machines.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-19.

* * * Start of Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| X |  | X |  |  |

* * * Second of Change * * * *

6.X	Solution #X: Reduce SIP message size during INVITE procedure through JSON-based transmission between UE and P-CSCF

### 6.X.1	High level principles

To improve IMS signaling efficiency and reduce SIP messages size in NB-IoT environments, the following optimizations are proposed:

‌1. pre-configuration procedure for invite and SDP parameters‌

- During the registration phase, the UE transmits a pre-configured invite message and SDP parameters to the P-CSCF, where the invite message does not contain information about the peer.

‌2. Minimized INVITE message and SDP parameters during call setup‌

- The invite message only contains its own identifier and the peer's identifier. If any other information needs to be modified, the invite or SDP will only carry the portion that requires changes. P-CSCF reconstructs the complete INVITE and SDP.

3. JSON over TCP/UDP transmission of Minimized SIP message between the UE and P-CSCF

- UEs sending SIP messages must adhere to RFC requirements, and certain Information Elements (IEs) in SIP messages are mandatory. Using JSON to encode SIP messages can omit some mandatory fields and effectively prevent the issue of over-simplified SIP messages violating RFCs.

- Omit mandatory fields already present in pre-configured messages for invite and 200 OK for invite. Optionally, all SIP messages between UE and P-CSCF are transmitted through JSON. The traditional SIP protocol is still used between P-CSCF and MS CN.

- Furthermore, when using JSON to encapsulate transcoded SIP messages, it is advisable to consider employing efficient compression algorithms to compress the transcoded SIP messages.

- Enable a SIP simplification policy between UE and P-CSCF, which can define SIP keyword abbreviations, e.g., P-Access-Network-Information: NR(GEO) = "p-geo".‌

- The UE can use a dedicated port to transmit JSON, or the P-CSCF can determine whether the UE is sending SIP or JSON based on the content of the message.

4. Optionlly, JSON over HTTP/2.0 transmission of Minimized SIP message between the UE and P-CSCF

- Optionally, Minimized SIP messages can be transmitted over TCP using HTTP/2.0 with JSON to omitting mandatory fields. Then an invitation and an 180ringing message will act as a pair of HTTP request and response messages, with 200 OK and ACK messages forming another pair of HTTP request and response messages.

- The UE and P-CSCF can exchange their respective URLs as HTTP servers in the message body of registration requests and 200 OK messages.

5. UE is prohibited from using multiple concurrent calls and forking

- A single caller URI cannot initiate multiple concurrent INVITE requests to the same callee URI, and a single INVITE must not be replicated by a proxy to multiple endpoints, to omit mandatory fields, such as the call id, tag.

6. P-CSCF as enhanced B2BUA

- The core responsibility of a B2BUA is to act as a full-featured SIP User Agent (UA), interacting with the IMS network on behalf of the originating UE. It must fully implement both the UAC and UAS state machines.

### 6.X.2	Description

This solution address Key Issue #2: "IMS Enhancements for GEO NB-IoT NTN Access".

This solution aims to pre-configure invite and SDP parameters for subsequent SIP calls by introducing invite and SDP pre-configuration procedures (e.g., during the SIP registration procedure). During call establishment, only a minimized invite and SDP are transmitted, omitting mandatory fields already present in pre-configured messages. JSON can be used instead of SIP for transmission between UE and P-CSCF, while SIP is still used for transmission between the P-CSCF and other IMS network elements.

### 6.X.3	Procedures

6.X.3.1	Registration

The following figure depicts the IMS registration procedure in the NB-IoT (GEO) scenario.

Figure 6.X.3.1: IMS registration procedure

1.	The UE sends a SIP REGISTER message, which includes a “pre-config” feature tag to indicate that the UE support “pre-config” to the IMS CN.

2.	The P-CSCF supports “pre-config” and determines to use the "pre-config" mechanism and responds with a 401 Unauthorized message that also includes the"pre-config" feature tag.

3.	The UE sends a REGISTER message, which includes a JSON body containing the pre-configured invite and SDP of UE side to the P-CSCF.The invite message does not contain information about the peer.

{"headers":{"Call-ID":"60f1f041e91ed7abd218c8b9b7a64067@10.1.1.199","CSeq":"4 INVITE","From":"<sip:8610000000000@open-ims.test>;tag=1004","Via":"SIP/2.0/UDP 10.1.1.199:5061;branch=z9hG4bK5cde41c01990a8b66684b2967b107fd1","Max-Forwards":"20","Route":"<sip:orig@scscf.open-ims.test:6060;lr>","Content-Type":"application/sdp","Contact":"<sip:10.1.1.199:5061>","P-Preferred-Identity":"\"alice\" <sip:alice@open-ims.test>","Privacy":"none","User-Agent":"Fraunhofer FOKUS/NGNI Java IMS UserEndpoint FoJIE 0.1 (jdk1.3)","P-Access-Network-Info":"3GPP-UTRAN-TDD; utran-cell-id-3gpp=00000000","Allow":"INVITE,ACK,CANCEL,BYE,MESSAGE,NOTIFY","Content-Length":"115"},"body":"v=0\r\no=user 0 0 IN IP4 10.1.1.199\r\ns=The funky IMS stream\r\nc=IN IP4 10.1.1.199\r\nt=0 0\r\nm=audio 8000 RTP/AVP 0 3 8"}

If JSON over HTTP/2.0 is used to transmit minimized SIP messages over TCP, JSON payload will contain the UE's URL as the server.

4.	The P-CSCF save invite and SDP parameters in the P-CSCF and responds with a 200 OK message. If JSON over HTTP/2.0 is used to transmit minimized SIP messages over TCP, the200 OK messages carries a JSON payload containing the P-CSCF's URL as the server.

5.	The pre-configured pre-configured invite and SDP parameters determined during during SIP Registration Procedure will be used in all the following IMS calls during the IMS registration lifecycle.

6.X.3.2	Call Set Up Procedure option1: JSON over TCP/UDP transmission between UE and P-CSCF after pre-configuration procedure

Figure 6.X.3.2: MO Call Setup Procedure with JSON over TCP/UDP transmission

Figure 6.X.3.2 is a signaling diagram of a UE using GEO satellite access makes an MO IMS call, which includes the following steps:

0.	The UE successfully finished the IMS registration procedure and invite and SDP parameters are pre-configured.

1.	The UE using GEO access initiates an IMS call and send an Minimized INVITE message and SDP parameters, if SDP parameter is not changed, no SDP parameter is included. JSON over TCP/UDP is used to transmit Minimized INVITE message and SDP parameters instead of SIP.In the most simplified scenario, the invite message only contains its own identifier and the peer's identifier.

{"Request-Line":"INVITE tel:8620000000000@open-ims.test SIP/2.0","From":"<sip:8610000000000@open-ims.test>"}

2.	The P-CSCF restores the pre-configured parameters to compose a complete invite message with SDP offer .

Request-Line: INVITE sip: 8620000000000@open-ims.test SIP/2.0

Call-ID: 60f1f041e91ed7abd218c8b9b7a64067@10.1.1.199

CSeq: 4 INVITE

From: <sip:8610000000000@open-ims.test>;tag=1004

To: <sip:8620000000000@open-ims.test>

Via: SIP/2.0/UDP 10.1.1.199:5061;branch=z9hG4bK5cde41c01990a8b66684b2967b107fd1

Max-Forwards: 20

Route: <sip:orig@scscf.open-ims.test: 6060;1r>

Content-Type: application/sdp

Contact: <sip:10.1.1.199:5061>

P-Preferred-Identity: "8610000000000" <sip:8610000000000@open-ims.test>

Privacy: none

User-Agent: Fraunhofer FOKUS/NGNI Java IMS UserEndpoint FoJIE 0.1 (jdk1.3)

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=00000000

Allow: INVITE, ACK, CANCEL, BYE, MESSAGE, NOTIFY

Content-Length: 115

Session Description Protocol Version (v): 0

Owner/Creator, Session Id (o): user 00 IN IP4 10.1.1.199

Session Name (s): The funky IMS stream

Connection Information (c): IN IP4 10.1.1.199

Time Description, active time (t): 00

Media Description, name and address (m): audio 8000 RTP/AVP 0 38

3.	The P-CSCF forward the INVITE with SDP offer to IMS CN through SIP.

4.	IMS CN sends 180 ringing to the P-CSCF.

5.	P-CSCFsends Minimized 180 Ring to the UE trough JSON.

{

"Status-Line": "SIP/2.0 180 Ringing",

"From": "<sip:8610000000000@open-ims.test>",

"To": "<sip:8620000000000@open-ims.test>",

"CSeq": "4 INVITE"

}

6.	Peer picks up the call.

7.	Peer sends SIP 200 OK for INVITE with SDP answer, IMS CN sends SIP 200 OK for INVITE to the P-CSCF.

8.	The P-CSCF sends SIP 200 OK with SDP answer to the UE. JSON is used to transmit Minimized 200 OK messages and SDP parameters. Minimized field is carried.

{"Status-Line":"SIP/2.0 200 OK","From":"<sip:8610000000000@open-ims.test>","To":"<sip:8620000000000@open-ims.test>","CSeq":"4 INVITE","Session Description":{"o":"user 0 0 IN IP4 10.1.1.200","m":"audio 8000"}}

9.	UE sends a Minimized ACK message to P-CSCF through JSON

{

"Request-Line": "ACK sip:10.1.1.200:5060 SIP/2.0",

"From": "<sip:8610000000000@open-ims.test>",

"To": "<sip:8620000000000@open-ims.test>",

"CSeq": "4 ACK"

}

10.	The P-CSCF receives ACK from the UE and sends it to the IMS CN.

11.	The media paths between UE and IMS CN have been established.

6.X.3.3	Call Set Up Procedure option2: JSON over HTTP/2.0 transmission between UE and P-CSCF after pre-configuration procedure

Figure 6.X.3.3: MO Call Setup Procedure with JSON over HTTP/2.0 transmission

Figure 6.X.3.3 is a signaling diagram of a UE using GEO satellite access makes an MO IMS call, which includes the following steps:

JSON over HTTP/2.0 is used to transmit minimized SIP messages, then the invite message will be an HTTP request message, and the 180 message will be a response message, then the 200 OK message will serve as the second HTTP request message, and the ACK message will serve as the response message.

0.	The UE successfully finished the IMS registration procedure and invite and SDP parameters are pre-configured.

1.	The UE using GEO access initiates an IMS call and send a Minimized INVITE message and SDP parameters, if SDP parameter is not changed, no SDP parameter is included. JSON is used to transmit Minimized INVITE message and SDP parameters instead of SIP.In the most simplified scenario, the invite message only contains its own identifier and the peer's identifier.

{"Request-Line":"INVITE tel:8620000000000@open-ims.test SIP/2.0","From":"<sip:8610000000000@open-ims.test>"}

2.	The P-CSCF restores the pre-configured parameters to compose a complete invite message with SDP offer.

Request-Line: INVITE sip: 8620000000000@open-ims.test SIP/2.0

Call-ID: 60f1f041e91ed7abd218c8b9b7a64067@10.1.1.199

CSeq: 4 INVITE

From: <sip:8610000000000@open-ims.test>;tag=1004

To: <sip:8620000000000@open-ims.test>

Via: SIP/2.0/UDP 10.1.1.199:5061;branch=z9hG4bK5cde41c01990a8b66684b2967b107fd1

Max-Forwards: 20

Route: <sip:orig@scscf.open-ims.test: 6060;1r>

Content-Type: application/sdp

Contact: <sip:10.1.1.199:5061>

P-Preferred-Identity: "8610000000000" <sip:8610000000000@open-ims.test>

Privacy: none

User-Agent: Fraunhofer FOKUS/NGNI Java IMS UserEndpoint FoJIE 0.1 (jdk1.3)

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=00000000

Allow: INVITE, ACK, CANCEL, BYE, MESSAGE, NOTIFY

Content-Length: 115

Session Description Protocol Version (v): 0

Owner/Creator, Session Id (o): user 00 IN IP4 10.1.1.199

Session Name (s): The funky IMS stream

Connection Information (c): IN IP4 10.1.1.199

Time Description, active time (t): 00

Media Description, name and address (m): audio 8000 RTP/AVP 0 38

3.	The P-CSCF forward the INVITE with SDP offer to IMS CN through SIP.

4.	IMS CN sends 180 ringing to the P-CSCF.

5.	P-CSCFsends Minimized 180 ring to the UE trough JSON.

{

"Status-Line": "SIP/2.0 180 Ringing",

"From": "<sip:8610000000000@open-ims.test>",

"To": "<sip:8620000000000@open-ims.test>",

"CSeq": "4 INVITE"

}

6.	Peer picks up the call.

7.	Peer sends SIP 200 OK for INVITE with SDP answer, IMS CN sends SIP 200 OK for INVITE to the P-CSCF.

8.	The P-CSCF sends SIP 200 OK with SDP answer to the UE. JSON is used to transmit Minimized 200 OK messages and SDP parameters. Minimized field is carried.

{"Status-Line":"SIP/2.0 200 OK","From":"<sip:8610000000000@open-ims.test>","To":"<sip:8620000000000@open-ims.test>","CSeq":"4 INVITE","Session Description":{"o":"user 0 0 IN IP4 10.1.1.200","m":"audio 8000"}}

9.	UE sends a Minimized ACK message to P-CSCF through JSON

{

"Request-Line": "ACK sip:10.1.1.200:5060 SIP/2.0",

"From": "<sip:8610000000000@open-ims.test>",

"To": "<sip:8620000000000@open-ims.test>",

"CSeq": "4 ACK"

}

10.	The P-CSCF receives ACK from the UE and sends it to the IMS CN.

11.	The media paths between UE and IMS CN have been established.

6.X.4	Message size estimation for Pre-configuration flow

| Message | Existing Flow (byte)+UDP header(8byte) | Existing Flow (byte)+TCP header(20byte) | Minimized with JSON (byte)+UDP header (8) | Minimized with JSON (byte)+TCP header (20) | Minimized with JSON (byte)+TCP header (20) +HTTP header (60) |
| --- | --- | --- | --- | --- | --- |
| Invite with SDP | 1028+8 | 1028+20 | 108+8 | 108+20 | 108+20+60 |
| 180Ringing | 357+8 | 357+20 | 270+8 | 270+20 | 270+20+60 |
| 200 OK(invite) | 676+8 | 676+20 | 208+8 | 208+20 | 208+20+60 |
| ACK | 309+8 | 309+20 | 166+8 | 166+20 | 166+20+60 |
| total | 2402 | 2450 | 784 | 832 | 1072 |

Example Existing Invite for Byte Statistics:

Request-Line: INVITE sip: 8620000000000@open-ims.test SIP/2.0

Call-ID: 60f1f041e91ed7abd218c8b9b7a64067@10.1.1.199

CSeq: 4 INVITE

From: <sip:8610000000000@open-ims.test>;tag=1004

To: <sip:8620000000000@open-ims.test>

Via: SIP/2.0/UDP 10.1.1.199:5061;branch=z9hG4bK5cde41c01990a8b66684b2967b107fd1

Max-Forwards: 20

Route: <sip:orig@scscf.open-ims.test: 6060;1r>

Content-Type: application/sdp

Contact: <sip:10.1.1.199:5061>

P-Preferred-Identity: "8610000000000" <sip:8610000000000@open-ims.test>

Privacy: none

User-Agent: Fraunhofer FOKUS/NGNI Java IMS UserEndpoint FoJIE 0.1 (jdk1.3)

P-Access-Network-Info: 3GPP-UTRAN-TDD; utran-cell-id-3gpp=00000000

Allow: INVITE, ACK, CANCEL, BYE, MESSAGE, NOTIFY

Content-Length: 115

Session Description Protocol Version (v): 0

Owner/Creator, Session Id (o): user 00 IN IP4 10.1.1.199

Session Name (s): The funky IMS stream

Connection Information (c): IN IP4 10.1.1.199

Time Description, active time (t): 00

Media Description, name and address (m): audio 8000 RTP/AVP 0 38

Example Existing 180Ringing for Byte Statistics:

Status-Line: SIP/2.0 180 Ringing

Call-ID: 60f1f041e91ed7abd218c8b9b7a64067@10.1.1.199

[Generated Call-ID: 60f1f041e91ed7abd218c8b9b7a64067@10.1.1.199]

CSeq: 4 INVITE

From:<sip:8610000000000@open-ims.test>;tag=1004

To: <sip:8620000000000@open-ims.test>;tag=1004

Via: SIP/2.0/UDP 10.1.1.199:5061;rport=5061;branch=z9hG4bK5cde41c01990a8b66684b2967b107fd1

Example Existing 200 OK for Byte Statistics:

Status-Line: SIP/2.0 200 OK

Call-ID: 60f1f041e91ed7abd218c8b9b7a64067@10.1.1.199

CSeq: 4 INVITE

From: <sip:8610000000000@open-ims.test>; tag=1004

To: <sip:8620000000000@open-ims.test>; tag=1004

Via: SIP/2.0/UDP 10.1.1.199:5061; rport=5061;branch=z9hG4bK5cde41c01990a8b66684b2967b107fd1

Contact: < sip: 10.1.1.200:5060>

Content-Type: application/sdp

Content-Length: 115

Session Description Protocol Version (v): 0

Owner/Creator, Session Id (o): user 0 0 IN IP4 10.1.1.200

Session Name (s): The funky IMS stream

Connection Information (c): IN IP4 10.1.1.200

Time Description, active time (t): 0 0

Media Description, name and address (m): audio 8000 RTP/AVP 0 3 8

Example Existing ACK for Byte Statistics:

Request-Line: ACK sip:10.1.1.200:5060 SIP/2.0

Via: SIP/2.0/UDP 10.1.1.199:5061;branch=z9hG4bK78f3dcbdf3108b1c05b894a60e7e3b94

CSeq: 4 ACK

Call-ID: 60f1f041e91ed7abd218c8b9b7a64067@10.1.1.199

From: <sip:8610000000000@open-ims.test>;tag=1004

To: <sip:8620000000000@open-ims.test>;tag=1004

Max-Forwards: 70

### 6.X.5	Impacts to Services, Entities and Interfaces

UE:

-	pre-configuration procedure for invite and SDP parameters‌

-	Support JSON transmission of Minimized SIP message to P-CSCF

-	UE is prohibited from using multiple concurrent calls and forking.

P-CSCF:

-	pre-configuration procedure for invite and SDP parameters‌

-	Support restore information for a SIP message if stored during registration procedure

-	Support JSON transmission of Minimized SIP message to UE

-	B2BUA is to act as a full-featured SIP User Agent (UA)

* * * End of Change * * * *