3GPP TSG-WG SA2 Meeting #170	S2-2507146

August 25 – 29, 2025, Goteburg, Sweden

Title:	[Draft] Reply LS on the RAN simulation assumptions for ULBC

Response to:	LS on the RAN simulation assumptions for ULBC (S2-2506169/S4-251584)

Release:	Release 20

Work Item:	FS_5GSAT_Ph4-ARC

Source:	SA WG2

To:	SA WG4

Cc:	RAN WG1, RAN WG2, RAN WG4, CT WG1, SA WG1

Contact Person:

#### Name:	Lin Shu

E-mail Address:	shulin AT vivo DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 thanks SA4 for the LS on the RAN simulation assumptions for ULBC. Regarding the actions to SA2:

# To SA2 and RAN2: ACTION: SA4 kindly asks SA2 and RAN2 to comment on - the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option, and - specifically, whether a packet overhead of 1 byte of MAC header is realistic.

SA2 has analyzed the different options and associated protocol overheads, excluding the AS layer, as summarized below (detailed analysis is available in S2-2507021):

|  | RTP | UDP | IP | NAS | Total |
| --- | --- | --- | --- | --- | --- |
| UP IP | ROHC (RTP/UDP/IP): 1-3 bytes |  |  | N/A | 1-3 bytes |
| UP non-IP | - 6 Bytes - 2 Bytes - 1 Byte | N/A | N/A | N/A | - 6 Bytes - 2 Bytes - 1 Byte |
| CP IP | ROHC (RTP/UDP/IP): 1-3 bytes |  |  | 7 bytes | 8-10 bytes |
| CP non-IP | - 6 Bytes - 2 Bytes - 1 Byte | N/A | N/A | 7 bytes | - 13 Bytes - 9 Bytes - 8 Bytes |

Regarding the AS layer protocol overhead (PDCP/RLC/MAC) and whether 1 byte of MAC header is realistic, SA2 believes this is fall in scope of RAN2 and RAN2 will provide feedback accordingly.

Regarding the preferred option, SA2 has concluded that the voice packets are transported over the NB-IoT (GEO) user plane, i.e. via DRB and S1-U in the context of FS_5GSAT_Ph4_ARC. SA2 understands that the reduction of protocol overhead for transmitting voice packets is essential in the case of GEO access. To achieve this, two options remain under consideration for further down-selection: UP IP and UP non-IP:

- In the UP IP option, Robust Header Compression (RoHC) is expected to be applied, e.g. RoHC Profile 1 could be used for compressing the RTP/UDP/IP headers.

- In the UP non-IP option, a new RTP header reduction mechanism is expected.

2. Actions:

To SA4:

ACTION: 	SA2 kindly asks SA4 to take the above protocol overhead analysis and interim conclusion — that the voice packets are carried via the NB-IoT (GEO) user plane — into account for their ULBC work. SA2 will further inform SA4 the progress of the down-selection between UP IP and UP non-IP.

3. Date of Next TSG-SA WG2 Meetings:

TSG-SA2 Meeting #171		13-17 October 2025	Wuhan, China

TSG-SA2 Meeting #172		17-21 November 2025	Dallas, USA