SA WG2 Meeting #170	S2-2507151

25 – 29 Aug, 2025, Goteborg, SE

Source:	ETRI

Title:	[WT#1.2] Technical Scope of WT#1.2 – Architectural Enhancement of 6GS

Document for:	Approval

Agenda Item:	20.6.6

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the technical scope of WT#1.2, focusing on architectural enhancements of the 5G Service-Based Architecture (SBA), the user plane, policy and exposure frameworks, and deployment support for specialized access environments. The aim is to evolve the architecture to meet the needs of 6G services including AI-native applications, deterministic communication, and programmable multi-access integration. The scope also covers the evaluation of alternatives to GTP-U, enhancements to the QoS and policy frameworks, and architectural provisions for deployment scenarios such as fixed wireless access (FWA) and localized services.

# 1. Justifications

The transition to 6G requires significant architectural evolution beyond the control plane. To support AI-native, deterministic, and context-aware services at scale, the 6G system architecture must provide enhanced programmability, data plane flexibility, and intelligent service exposure. This includes evolution of the Service-Based Architecture (SBA), user plane, policy and QoS frameworks, and deployment models tailored for emerging access types and specialized service environments.

Gap Analysis:

- The SBA in 5G lacks support for real-time, AI-assisted, and context-aware service invocation, as well as cross-domain composition and localized orchestration.

- The current user plane architecture based on GTP-U is unsuitable for deterministic, distributed, and programmable data delivery, and lacks telemetry exposure and AI-native control capabilities required for 6G.

- Exposure and policy frameworks in 5G are not designed for intent-driven control or integration with external orchestration entities.

- Support for emerging access types, such as non-terrestrial networks (NTN) or sensing-assisted links, is absent or insufficient in existing multi-access frameworks.

- Integration of non-3GPP access remains static and policy-based, and does not support AI-driven or programmable coordination.

- 5G architecture provides limited support for specialized deployment environments, such as Fixed Wireless Access (FWA) or localized service access.

Technical Impact:

- Architectural evolution of the SBA to support real-time, AI-native, and cross-domain service orchestration, including flexible service composition and registration mechanisms.

- Introduction of a next-generation user plane framework, potentially replacing or augmenting GTP-U, to support distributed deployment, programmable data paths, and deterministic performance (e.g., for XR, industrial automation).

- Expansion of policy and QoS frameworks to support intent-based and service-driven models, enabling dynamic and AI-assisted control.

- Enhancement of the network exposure framework to allow secure and programmable interface with third-party services, external agents, and verticals.

- Architectural support for emerging access types (e.g., NTN, sensing-based access) through open and programmable integration mechanisms.

- Definition of architectural models for specialized deployment scenarios, such as Fixed Wireless Access (FWA), private networks, or localized service environments, supporting modular and customizable service delivery.

**** First Change ****

## Annex A.X. WT#1.2 - Technical Scope

This Work Task investigates the enhancement of architectural components in 6G beyond control signaling. It focuses on functional evolution of the SBA, user plane, QoS and policy control frameworks, network exposure capabilities, and support for deployment-specific scenarios.

This Work Task considers the following aspects:

- Evolution and enhancement of the Service-Based Architecture (SBA) for 6G.

- Support for real-time, AI-assisted, and context-aware service orchestration.

- Investigation of alternatives to the current user plane architecture (e.g., GTP-U), to enable deterministic, distributed, and programmable data transport.

- Enhancement of QoS and policy control frameworks to support new 6G services.

- Expansion of the network exposure framework to support programmable and intent-driven service control.

- Architectural considerations for specialized deployments, including fixed wireless access and localized service access.

- Evolution of network slicing and network sharing models in 6G.

**** Second Change ****

## 1.X. Key Issue #X: SBA Enhancement and Evolution

#### 1.x.x.x	Description

This key issue explores how to enhance the SBA to support emerging 6G functions, scalability requirements, and new service classes such as AI-native services.

This key issue will investigate:

- How to extend SBA to support real-time and context-aware service invocation.

- Enhancements to SBA service registration, exposure, and routing across domains.

- How to accommodate stateful and stateless service exposure.

- Inter-SBA-domain service interaction (e.g., cross-slice, cross-network SBA).

- Integration of data-driven or AI-assisted service elements.

- Integration with updated network slicing and sharing models.

## 1.X. Key Issue #X: Integration with Non-3GPP Access and Multi-Access Connectivity

#### 1.x.x.x	Description

This key issue investigates architectural mechanisms for intelligently integrating a wide variety of access technologies—including 3GPP, non-3GPP (e.g., Wi-Fi, wireline), and emerging access types such as satellite (NTN), sensing-assisted links.

Unlike 5G, where support for non-3GPP access was relatively static and pre-defined, 6G is expected to enable dynamic, programmable, and AI-assisted orchestration of access selection, steering, and policy enforcement across diverse connectivity environments.

This key issue will investigate:

- How to support AI-native, context-aware access selection and switching across heterogeneous access types.

- Architectural models for open programmability of access functions (e.g., via network exposure APIs or intent interfaces).

- Integration of emerging access modes (e.g., NTN) and their interaction with existing multi-access frameworks.

- Mechanisms to ensure QoS continuity and mobility across dynamically changing access configurations.

- How to expose access-related capabilities, analytics, and policies to UE, third-party services, or AI agents for autonomous decision-making.

## 1.X. Key Issue #X: User Plane Architecture Evolution

#### 1.x.x.x	Description

This key issue focuses on evolving the user plane architecture to meet the stringent performance, flexibility, and intelligence requirements of 6G. This includes enabling distributed data paths, AI-native forwarding, and deterministic delivery mechanisms.

This key issue will investigate:

- How to support dynamic and distributed user plane function (UPF) placement.

- How to introduce AI-assisted traffic steering and in-network inference.

- Requirements for deterministic or ultra-reliable data delivery.

- Exposure and programmability of user plane capabilities.

- Feasibility of alternatives to GTP-U, including SRv6 or other AI-integrated data plane technologies that enable enhanced flexibility, observability, and control.

**** End of Changes ****