3GPP TSG-WG SA2 Meeting #170	S2-2507152

# Goteborg, Sweden, August 25 - 29, 2025

Source: 	China Mobile

Title: 	Solution #20 update: Energy aware NF and path selection

Document for: 	Approval

Agenda Item: 	20.4

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: Propose a update for solution #20: Energy aware NF and path selection.

1	Discussion

This paper is to propose update on solution #20.

Solution #20 has proposed an extension on NF profile to take energy related information into consideration for NF selection. This solution introduces additional information to NF profile, including energy state, energy priority information and schedule of such information. According to existing solutions (for example, solution #8 and solution #14) in TR 23.700-67, renewable energy consumption and the ratio of renewable energy can be collected, calculated and exposed. Therefore, this update proposes to include Renewable Energy Information in the NF profile.

2	Proposal

It is proposed to include the below changes into TR 23.700-67 v0.2.0.

FIRST CHANGE

6.20	Solution #20: Energy aware NF and path selection

6.20.0	High-level solution principles

This solution proposes the following principles to be agreed:

1.	The NF profile is augmented with Energy Priority Information.

2.	The NF profile is augmented with a Schedule of Energy Priority Information to enable providing a schedule of Energy Priority information, if one exists, instead of requiring additional updates at a time when the Energy Priority information is known to change.

3.	The NF profile is augmented with Schedule of EnergySaving/not EnergySaving State (See TS 28.310 [9]) and the EnergySaving/NotEnergySaving state attributes to enable the NF service consumer and NRF to know when the NF may be taken out of service according to a schedule or to enable the NRF to know a NF is temporarily out of service to save energy.

4.	The NF profile is augmented with Renewable Energy Information to enable providing the use of renewable energy by the NF.

5.	The UPFs in the UP path are selected by the SMF taking into account the Attributes above in the NF profile, if any are present, based on a UP Path Selection policy.

6.20.1	Description

This solution assumes that the operator ensures that the NF service consumers of the NFs that can be discovered and selected based on energy criteria are upgraded.

This solution proposes to enhance the NF selection related functionalities for energy aware NF selection.

The following solution components are envisaged:

-	New optional Energy-related NF profile attributes are defined as in Table 6.20.1-1 and are stored at NRF.

-	OAM provisions (and updates as needed) or the service producer NF registers (and updates as needed) Energy-related NF profile attributes in its NF profile at the NRF.

-	The NF Discovery procedure, based on operator policy, takes into account any Energy-related NF profile attributes query parameters. Both the NRF and the NF service consumer can take into account the Energy-related NF profile attributes if available, as part of the NF Discovery procedure. If a NRF provides a scheduling information about an Energy-related NF profile attribute, it shall update the NF service consumer which subscribe to NF profile updates only about unscheduled changes of Energy-related NF profile attribute.

-	A NF service consumer takes into account any Energy-related NF profile attributes from the NF profiles discovered from NRF using the NF Discovery procedure (and subsequent updates if the NF subscribed to NF profile updates), for the selection of a target NF. The NF consumer can take Selection decisions based on the Energy-related NF profile attributes scheduling information it receives if any, without the need to contact the NRF further.

Table 6.20.1-1: Energy-related NF profile attributes

| Information | Description |
| --- | --- |
| EnergySaving /Not EnergySaving State | Value of the energy state as specified in TS 28.310 [9], e.g. notEnergySaving state, Energy Saving state. (NOTE 1) |
| Schedule of EnergySaving/not EnergySaving | The schedule of the EnergySaving/not EnergySaving states, includes the value of EnergySaving or not EnergySaving state which applies at each time interval in the schedule. If this is present, the EnergySaving /Not EnergySaving State attribute is not present. |
| Energy Priority Information | Value of Energy priority relative to other NFs of the same type considering the energy characteristics of the NF. |
| Schedule of Energy Priority Information | The Schedule of the Energy Priority includes the value of Energy Priority information which applies at each time interval in the schedule. If this is present, the Energy Priority information attribute is not present |
| Renewable Energy Information | The use of renewable energy by the NF, including either 1) whether the UPF uses renewable energy or non-renewable energy, or 2) percentage of Energy Consumption using renewable energy in the total Energy Consumption. |
| NOTE 1: EnergySaving/not EnergySaving refers to states of a network function which are defined in TS 28.310 [9]. |  |

NOTE 1:	The EnergySaving /Not EnergySaving OAM States attribute can be implemented by setting in NRF the UNDISCOVERABLE and REGISTERED existing attributes in TS 29.510 [10] is FFS. The need of a new attribute is FFS.

NOTE 2:	For non-supporting NF service consumers, the SHUTDOWN TIME existing attributes in TS 29.510 [10] can be used when a future event of transition to EnergySaving State is known in advance as part of the Schedule of EnergySaving/not EnergySaving attribute.

The Energy Priority Information indicates the priority to select a NF relative to other NFs of the same type considering the energy characteristics of the NF. If Energy is a criterion for selection of a NF, the NF Consumer can use Energy Priority Information parameter in conjunction with the other attributes to select those matching the requested criteria (e.g. supported capability, slice, etc.) with higher Energy Priority. The NF Service Consumer or the NRF discovery service may select the NF or NF instance considering the Energy Priority information in the discovered NF profile. The Network operator configures the (supporting) NF service consumers with the behaviour in handling the Energy Priority Information attribute network-wide in one of two ways:

-	Consider the Energy Priority Information (or Energy Priority Information Schedule) and other attributes to discover the list of the candidate NFs with highest Energy Priority and then among the highest Energy Priority NFs select the NF based on "NF Priority" and "NF Capacity attributes" if present.

-	Consider the other attributes first (including the "NF Priority" and "NF Capacity attributes" if present), then among the selected NFs (if more than one exists) consider the Energy priority information (or its schedule) by selecting the NF(s) with highest Energy Priority Information attribute.

When the NRF indicates (e.g., via schedule or via attribute update) the NF is in EnergySaving State, the NF is not usable by the NF service consumer (i.e. all other attributes are considered for discovery and selection only for the time when NFs that are in notEnergySaving state).

The value of the Energy Priority Information is determined by the Operator based on their policies (e.g. energy source, nominal energy efficiency) outside the scope of 3GPP.

NOTE 3:	The information within NF profile specified in Table 6.20.2-1 is expected not to change frequently, e.g. constantly reflecting changes in energy consumption.

The difference between the Scheduled EnergySaving/not EnergySaving information and the EnergySaving/not EnergySaving state specified in TS 28.310 [9] is that EnergySaving/not EnergySaving state indicates the current state of the NF, e.g. the EnergySaving state indicates that the NF is in the state where Energy Saving is progress, while the Scheduled EnergySaving/not EnergySaving indicates the time when the NF will be moved to from Not EnergySaving state (i.e. powered on) to EnergySaving state (i.e. powered-off) and vice-versa. By knowing the Schedule of Energy Saving/not EnergySaving a NF service consumer can implement NF selection and NF reselection strategies taking this information into account. This may help to migrate Sessions from an SMF or UPF about to go out of service. Also, PDU sessions with SSC mode 1 should not be caused to use UPFs/SMFs that are known to enter EnergySaving state.

The Renewable Energy Information describes the use of renewable energy by the NF. It may include Energy Type or Renewable Energy Ratio. The Energy Type indicates the type of energy used by the NF, i.e. renewable energy or non-renewable energy. The Renewable Energy Ratio indicates the percentage of the renewable energy used by the NF, and is defined as the proportion of renewable energy consumption in the total energy consumption of the NF.

It is also proposed that the UPF to be used to serve a UE PDU session is selected as any other NF taking energy criteria also into account as described above.

6.20.2	Procedures

The procedures are not impacted but the NF profile information is impacted.

6.20.3	Impacts on existing services, entities and interfaces

The impacts are:

NRF:

-	support the Energy-related NF profile attributes as per Table 6.20.1-1.

-	enhancement of NRF services and NRF logic to handle the new Energy-related NF profile attributes.

OAM:

-	support the enhancements to handle the new Energy-related NF profile attributes.

Other NFs:

-	enhancement of NF service producers to register new Energy-related NF profile attributes.

-	enhancement of NF service consumers to discover and select NFs by also considering Energy-related NF profile attributes.

End of CHANGE