SA WG2 Meeting #170	S2-2507163

25 – 29 Aug, 2025, Goteborg, SE

Source:	Robert Bosch GmbH

Title:	[WT#1.2, Local Networks] Introduction and justification of required studies for local networks

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This TDoc proposes sub-work tasks for architecture enhancements to enable local networks in WT#1.2 of the SA2 6G SID.

# 1. Justifications

Many vertical use cases require highly localized wireless communication within systems for e.g., within a vehicle to complement existing communication technologies e.g., Ethernet. Such wireless local networks may bring many benefits, such as lower complexity, higher flexibility, and lower costs for e.g., by simplifying the wiring harness in a car. However, the existing 3GPP communication modes and topologies may not be optimal for such localized communication.  The integration and operation of all system components/devices through the 3GPP infrastructure can be cumbersome or might not be feasible at all if only short-range direct communication within a system is required. Besides, loss of the connection to the 3GPP network may lead to an outage of the system, which is not tolerable for such systems e.g., in a car. The routing of local traffic and management functions through public mobile networks may also lead to unnecessarily higher latency and network load, as well as inefficient usage of available spectrum.

3GPP local networks were approved in SA1 as a 6G use case and the requirements have been already captured in TR 22.870. Local networks are use-case-centric and attached to a mobile network (e.g., PLMN or NPN or NTN), and having partial autonomy to survive during loss of connectivity to the mobile network. The mobile network is envisioned to provide external connectivity for local networks and provide bulk communication resources for local communication between local network devices. Local networks can be enabled with limited core network functionalities to achieve partial autonomy from the mobile network, e.g. to counteract loss of connectivity to the mobile network. The internal traffic of local networks may be realized by intra-local network coordination, i.e. resource management within a local network using the bulk resources granted by the mobile network.

Figure 1 illustrates 3 different examples of local networks which are connected to a mobile network and need the local connectivity between their components.

Figure 1: 3GPP local networks such as machine cell networks, personal networks, and in-vehicle local networks.

As mentioned earlier, existing 3GPP technologies such as Personal IoT Networks (PIN), Sidelink and Non-Public Networks (NPN) cannot fulfil the requirements of such local networks and there is a need for specifications of local networks in 6G.

### NPN has all necessary core network functionalities and may not need any configuration or management by the public network. On the other hand, a local network does not need all core network functionalities except  as subset of functionalities for e.g., local authorization of devices when the local network is out of coverage of mobile network.

### Sidelink devices must be authorized by public network and no sidelink device can take over controlling other devices without having these extended capabilities. This means we cannot have any local network control using existing sidelink capabilities.

PIN allows the utilization of licensed spectrum only for public safety PIN when PIN element is out of the coverage of the mobile network, otherwise unlicensed spectrum should be used. Therefore, there is no local management and authorization in PIN, if there is a loss of the connection to the mobile network.

**** First Change ****

# Annex A.1. WT#1.2 Scope

It is proposed to capture in the scope of WT1.2 in SP-250806

“1.2.	Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.”

the following aspects of local networks:

- Study limited core network functionalities of local networks to achieve partial autonomy from the mobile network

- Study intra-local network coordination, i.e. resource management within the local network

- Further studies on architecture enhancements to enable local networks in 6G

**** End of Changes ****