SA WG2 Meeting #170	S2-2507165

25 – 29 Aug, 2025, Goteborg , SE

Source:	Tencent, Tencent Cloud

Title:	[WT#6, Computing in 6G] Work Task Scope and Key Issues for 6G

Document for:	Approval

Agenda Item:	20.6.6

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This pCR proposes the work task scope and key issues for computing in 6G  including the rationale, expected scope, and key issues for this work task.

1. Justifications

Architectural Motivation: Computing is a new dimension for 6G system to consider because there are more and more services to support which require computing requirements to be met.  For example, to support immersive communication, there needs to be computing power in UE, core network as well as application server in cloud for encoding, decoding, rendering etc.  Due to the fact there are different hardware configuration of the UEs, the computing power may not be sufficient and then offload the computing load to core network or application server becomes a good choice.  There are also other use cases capture in 3GPP TR 22.870 which elaborates the need of considering computing power for 6G services.

Gap Analysis: In 5G and 5G Advanced (Rel-15-Rel-20), there is no standardized functionality to support computing power awareness among core network, UE and application server.  Also, there is no support to consider NF’s computing power therefore there is no standardized approach to maintain service continuity during UE mobility.  The UE may be handed over to an network entity (or this network entity is selected to serve the UE’s session) which doesn’t have sufficient computing resources which is not desirable.

Technical Impact:

- Introduce new functionality to support computing power exposure among UE, core network and application server which need to be supported by corresponding NFs

- Introduce support of service continuity including UE mobility by considering computing power

Based on the above justifications, it is proposed to update the annex of TR 23.801-01 to reflect the scope of WT#6 and also capture the key issue for this work task in 5.x.

**** First Change ****

Annex A.X. WT#6 Scope

WT#6: Study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination between UE, core network and application server, exposure of computing service in the core network, etc.).

NOTE 7: Application layer mechanism and exposure framework may require coordination with SA6.

**** Second Change ****

5.X. Key Issue #X: Support of Computing in 6G

This key issue will address the system architecture support for computing in 6G:

-	How does the 6G network expose its computing power to UE and authorized third party in cloud to enable the computing power sharing and coordination among UE, core network and application server?

-    How to support service continuity including UE mobility taking into account of computing power of the NFs?

**** End of Changes ****