SA WG2 Meeting #S2-170	S2-2507166

25 - 29 August, 2025, Goteborg, Sweden

Source: 	China Telecom

Title: 	New solution on emergency SMS to PSAP over NAS

Document for: 	Approval (P-CR)

Agenda Item: 	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: This paper proposes a solution on emergency SMS to PSAP over NAS for FS_SMS2EC_ARC.

# Proposal

It is proposed to approve following content for solution on emergency SMS to PSAP over NAS of TR 23.700-65.

* * * * Start of 1st change (all new text) * * * *

## 6.X	Solution #X: Emergency SMS to PSAP over NAS

### 6.X.1	High level principles

The high level principles of the proposed solution are as follows:

-	The proposed solution focuses on Emergency SMS to emergency response centre or PSAP over NAS.

-	UE sends emergency SMS to the network via UL NAS TRANSPORT.

-	There should be an emergency identifier included in the UL NAS TRANSPORT message content.

-	The emergency identifier reuses the existing mechanism as emergency call.

### 6.X.2	Description

Currently the addresses of emergency response centres/PSAPs vary in different areas even they belong to the same country. When UE triggers emergency SMS service, it would be more flexible if the UE only needs to report certain emergency type identifier which is similar with the URN applied in emergency call. It is up to the network to reflect the received emergency type identifier to certain destination address of emergency response centre/PSAP based on the local policy and deployment.

To assist the emergency response centre/PSAP locate the UE more accurately, both the UE and the AMF may provide more UE location related information in the emergency message.

Editor's Note:	KI related description to be updated.

### 6.X.3	Procedures

Figure 6.X.3-1: Emergency SMS to PSAP over NAS

The steps of figure 6.X.3-1 are described as follows:

- The UE reports its capability of SMS over NAS by SMS indication during registration procedure.

- The UE triggers the request for emergency SMS over NAS with dedicated emergency type (e.g. sos.police, sos.ambulance, sos.fire, etc.). The UE encapsulates the emergency NAS message with emergency type and UE location information included.

- The UE sends the emergency NAS message to the AMF via UL NAS TRANSPORT.

- The AMF identifies the UL NAS message as emergency message with the emergency type information.

- The AMF forwards the emergency SMS to the SMSF by invoking Nsmsf_SMService_UplinkSMS service operation, including SUPI, SMS payload, emergency type identifier, UE location information, and optionally UE location trends etc.

As it is an emergency SMS, the AMF may include the message priority indication in the request to SMSF.

6a.	The SMSF determines the target address of the emergency response centre/PSAP based on the emergency type identifier received from AMF, and includes it in the SMS destination address header.

6b-6c.	The SMSF perform the reconstruction of emergency SMS. The SMSF may include the UE location information both from the UE and the AMF in the SMS payload to assist the emergency response centre/PSAP locate the UE more accurately.

7.	The SMSF sends the reconstructed emergency SMS to the emergency response centre/PSAP through SMSC.

### 6.X.4	Impacts on services, entities and interfaces

UE:

-	Encapsulates the emergency NAS message with emergency type and UE location information included.

AMF:

-	Identifies the emergency message by emergency type identifier.

-	Determines the message priority of emergency message.

-	Enhances the UE location information.

-	Forwards the emergency message to SMSF with emergency type information.

SMSF:

-	Stores the association between emergency type identifier and address of emergency response centre/PSAP.

-	Identifies the emergency message by emergency type information.

-	Includes the received UE location information in the emergency message body.

- 	Determines the address of emergency response centre/PSAP based on the emergency type information.

-	Sends the emergency message to the related emergency response centre/PSAP.

* * * * End of change * * * *