3GPP TSG-SA WG2 Meeting #170	S2-2507167

Gothenburg, Sweden, August 25 – 29, 2025

Source:	Samsung

Title:	KI#2: New Solution on access control based on EIF notification

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes a new solution on access control for a UE that the PCF and AMF may utilize the notification including Energy Consumption information from the EIF and update the access control parameters for a UE.

# 1.	Discussion

One of the key issues for Rel-20 EnergySys Phase2 includes the study on solutions that can adjust UE’s data transmissions for the network energy saving based on the energy information of a UE/Network Slice per UE/PDU Session per UE/QoS flow per UE. A new solution is proposed to support the PCF and AMF to perform access and mobility control for a UE based on the energy consumption information received from the EIF.

# 2. Proposal

It is proposed to adopt the following changes in TR 23.700-67.

* * * Start of Change (all new text) * * * *

## 6.X	Solution #X: Access and mobility control based on energy information from EIF

### 6.X.0	High-level solution principles

This solution is to address access and mobility control that the PCF updates a policy for the network energy saving based on energy related information received from the EIF and the AMF set parameters for access control of the selected UE.

- The PCF may subscribe to the EIF for the desired granularity of the energy consumption information for the UE.

- The EIF collects the energy consumption information and notifies the energy consumption information in the requested granularity to the PCF.

- The PCF may decide the UE that should be under energy usage control, based on the Energy Saving indication in the UE Subscription Data, energy consumption information received from the EIF, operator policy, etc.

- The PCF may inform the AMF of the selected UE that should be under energy usage control.

- The AMF updates the access and mobility control parameters based on the indication for network energy saving that is received from the PCF

### 6.X.1	Description

The solution assumes the Energy Saving indication for the UE is stored in the Subscription Data of a UE. The PCF and/or the AMF may retrieve the information during the AM Policy Association or Registration procedure. In case of the dynamic policy control, the PCF retrieves from the UDR and stores the Energy Saving indication information for UE and provides a policy to control the energy use for a UE during the Policy Association Establishment or Modification with the AMF. The PCF may subscribe to the EIF and receive the Energy Consumption information in a required granularity to control the energy consumed in the network to serve the UE. The PCF may decide, based on the energy related information received from the EIF, operator policy, Energy Saving indication, and local configuration, if network energy saving is required. The PCF may provide the AMF with an indication for the network energy saving in the policy information for UE, and request to apply a proper access and mobility control parameters to the UE for the sake of network energy saving.

### 6.X.2	Procedures

Figure 6.x.2-1 illustrates Registration procedure considering energy related information from the EIF.

Figure 6.X.2-1: Registration procedure considering energy related information

1.	UE sends a Registration Request message to the AMF to attach to the network.

2.	The AMF performs Subscription Retrieval procedure with UDM and receives Subscription Data for the UE. The Subscription Data may include Energy Saving indication information for the UE. The AMF may use this information together with operator policy and local configurations to decide control parameters for the UE, e. g. RFSP, service area restriction, etc.

3.	The AMF performs Policy Association Establishment procedure with PCF if dynamic policy is supported. The PCF may provide the energy related policy in the AM policy information for the UE based on the Energy Saving indication and energy related information that is recieved from the EIF (or OAM). The AMF may use the policy  information received from the PCF to set the control parameters for the UE, e.g. RFSP, service area restriction, etc.

4.	The remaining steps for Registration procedure as defined in steps 17 to 24 in Figure 4.2.2.2.2-1 of TS 23.502 [3] proceeds.

5.  The PCF may subscribe to the EIF for notification of the energy-related information related to the UE.

6. The EIF may send the updated energy related information as requested by the PCF in the step 5.

7.	The PCF may decide to change the AM policy of the selected UE for energy saving, e.g., to trigger the aggressive saving mode when the energy consumption in the network increases beyond the defined threshold happens, based on the energy related information included in the notification from the EIF, operator policy, local configuration, etc.

8.	The PCF performs AM Policy Modification procedure with AMF and provides the indication for network energy saving in the updated policy information for the UE.

9.	The AMF stores the updated AM policy and updates the parameters for access and mobility control of the UE based on the indication for network energy saving from the EIF, Energy Saving indication, operator policy, local configurations, etc. The AMF provides the updated parameters for the UE when the Registration procedure is initiated by the UE.

### 6.X.3	Impacts on existing services, entities and interfaces

PCF:

-	Subscribes to the EIF for energy related information and decide if the network energy saving is required for a UE.

-	Provides the AMF with indication for network energy saving for the selected UE in the policy information to control the energy use by a UE.

AMF:

-	Sets parameters for access and mobility control of a UE based on the Subscription Data from the UDM or indication for network energy saving received from the PCF.

* * * End of Change * * * *